SECTION VI

WORKS REQUIREMENT, TECHNICAL SPECIFICATIONS, DRAWINGS/LAYOUTS, SUPPLEMENTARY INFORMATION AND BILL OF QUANTITIES

# Section IV. Works Requirement, Technical Provisions, Drawings/layouts, Supplementary Information & Bill of Quantities

## Scope of Works

(Describe Scope of Work)

The scope of work includes:

- Removal of existing structures as required.

- Installation of base and structural supports.

- Construction and modifications to support the new design.

- Construction of new lavatory block.

- Provision and installation of new flooring tiles.

- Supply and installation of ceiling tiles.

- New plastering and patchwork for walls.

- Fabrication and installation of doors and wooden structures.

- Application of enamel and emulsion paint on interior and exterior surfaces.

- Replacement of bathroom tiles, removal of rusted Gl pipelines, and necessary plumbing work.

- Repair and maintenance of electrical points, provision of ceiling lights, tube lights.

- Replacement of broken glass panes.

- Repair and maintenance of expansion joint sheets.

- Repair and maintenance of kitchen tiles, cabinets, and other fixtures.

- Installation of tuff tiles, cladding, and lighting enhancements.

- Repair and maintenance of aluminum doors and windows.

- Roof treatment with terrazzo flooring.

## Technical Specifications

Technical Specifications are attached in Annex A at the end of this document.

## Drawings

(Insert reference to the drawing of the Construction works to be carried out under this contract)

Old Layout

New expected layout of Ground Floor Front Entrance

Bill of Quantities

SUMMARY

| Bill # | Description |
| --- | --- |
| 1 | Civil Works |
| 2 | Electricity |
| 3 | LAN & Surveillance |

SUMMARY

Bill No. 1 Title: Civil Works

| Bill # | Description |
| --- | --- |
| 1.1 | Excavator |
| 1.2 | Concrete & Pavior |
| 1.3 | Brick Layer |
| 1.4 | Terrazzo, Stone and Tile Worker. |
| 1.5 | Plasterer and Pointer |
| 1.6 | Carpenter and joiner |
| 1.7 | Metal Worker and Glazier |
| 1.8 | Painter and Decorator. |
| 1.9 | Water Supply, Sanitary, Plumbing, Fixture & Sewerage. |
| 1.10 | Roofer and water proofer |
| 1.11 | Dismantling and Demolishing |
| 1.12 | Miscellaneous |

Bill No. 1.1 Title: Excavator

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 101 | Structural excavation for foundations and basement in any type of soil including stone or gravel, up to required depths including back filling in trenches and under floors/pavements with approved excavated material, in layers including watering, ramming compacting, leveling & dressing etc., as per specification and disposal of surplus excavated material within or outside the project limits, including all lift and lead etc., complete as directed. | 1025 | 100 Cft |
| 102 | Selected/approved earth fill under floors in layers, earth brought from outside including transportation, ramming, watering, compacting, leveling and necessary lead and lift, complete. | 392 | 100 Cft |
| 103 | Providing and laying out Termite proofing in foundations, under floors and woodwork surface with a solution using of confront TC with water and spraying the solution of the area to be treated as completed as per direction of Engineer Incharge. | 6466 | P.Sft |

Bill No. 1.2 Title: Concrete & Pavior

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 201 | Provide, mix, lay compact, level, finish and cure etc., lean concrete 1:4:8 using 1" down crush including formwork and its removal, complete in all respects. | 223 | 100 Cft |
| 202 | Provide, mix, lay, compact, vibrate, level, finish, cure etc. cast - in site structural concrete of 3000 PSI 28 days cylindrical strength having the nominal mix proportions not leaner than 1:2:4, including form work and its removal, complete in all respects, excluding only the steel reinforcement, in: - |  |  |
|  | i): Footing, bases | 109 | 100 Cft |
|  | ii): Column | 40 | 100 Cft |
|  | iii): Plinth & Band Beams | 60 | 100 Cft |
|  | iv): Suspended beams, lintels etc. | 57 | 100 Cft |
|  | v): Suspended slabs, projections, shelves and the like | 129 | 100 Cft |
|  | vi): Parapets, hanging stair purdies, decorative shades/arches, cantilevers. | 13 | 100 Cft |
| 203 | Provide, clean, cut, bend, place in position, bind etc., steel reinforcement including cost of binding wire, spacer block, wastage, steel chairs/pins and the overlaps not shown in drawings, complete in all respects:- |  |  |
|  | 1): Round deformed steel bars of 40,000 PSI yield strength (Intermediate grade). | 2507 | Kg |
| 204 | Provide, lay, compact, finish, cure etc. 2" thick PCC 1:2:4 topping/flooring in panels using marble dividers in joints. | 100 | 100 Sft |
| 205 | Providing and laying floors of 1:2:4 cement concrete using graded screened bajri 3/4 inch (19 mm) and down gauge in raise floor laid in panels including form work, consolidation, finishing and curing etc. complete. | 1978 | 100 Cft |
| 206 | Providing and laying floor 1-1/4 inches (32 mm) thick of cement tiles (Concrete Concepts/Banu Mukhtar/Izhar Concrete) 16" X 16" 1-1/4" (1/2 inch topping in grey cement with flower or pattern in required colour of dark shade and 3/4 inch base in grey cement) or 406 mm X 406 mm 32 mm (13 mm topping in grey cement with flower or pattern in required colour of dark shade and 19 mm base in grey cement) in ground floor over 1 inch (25 mm) lime mortar 1:2 (one lime two sand) including setting the tiles with grey cement slurry over lime mortar jointing and washing the tiles with neat cement slurry of matching colour including grinding, rubbing, polishing and curing etc. complete, including the cost of mortar. | 2526 | 100 Cft |
| 207 | Providing and laying road kerb of precast cement concrete 1:2:4 (Concrete Concepts/Banu Mukhtar/Izhar Concrete) (24" long 6" wide and 12" deep) using screened graded bajri, from work and its removal compacting and curing laid in cement mortar 1:4 and cement plaster 1:4 neatly finished including cost of excavation and back filling of excavated stuff and its disposal within one chain lead as desired. | 144 | 100 Rft |

Bill No. 1.3 Title: Brick Layer

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 301 | Provide, lay, cure etc. first class burnt brickwork in foundation and plinth, 9” and over thick, set in 1:5 cement sand mortar, complete. | 510 | 100 Cft |
| 302 | Provide, lay, cure etc. first class burnt brickwork in walls in superstructure, including form work etc. complete:- |  |  |
|  | 1): 9" and over thick, set in 1:6 cement sand mortar |  |  |
|  | a) L.G. Floor / Ground Floor | 1673 | 100 Cft |
|  | b) First Floor | 15 | 100 Cft |
|  | 2): 4-1/2" thick, set in 1:4 cement sand mortar: |  |  |
|  | a) L.G. Floor / Ground Floor | 280 | 100 Cft |
|  | b) First Floor | 20 | 100 Cft |

Bill No. 1.4 Title: Terrazzo, Stone, and Tile Worker

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 401 | Providing and laying light colour, glazed/non skid vitrified porcelain tiles (Polished) exceeding (24" x 24") 3600 Sqcm each, (Master/Shabbir/Stile) (Supreme Quality) on walls and floors, in any floor, laid with dry bond (stile bond) over existing floors, including jointing to tiles with joint filler of approved quality as per direction of the Engineer incharge. | 5471 | 100 Sft |
| 402 | Providing and laying flooring (glazed) comprising 16" x 16" x 3/8" (1600 cm) (available thickness) Granitto tiles (Master/Shabbir/Stile) fixed in neat cement buttering laid over 1¼" thick PCC 1:2:4 using 3/8" down coarse aggregates to final level and stop i/c filling joints with ready made approved filler of suitable colour (at any height). | 604 | 100 Sft |
| 403 | Provide, lay, joint, cure, etc. 1” total thick glazed (glossy) ceramic tiles dado using (Master/Shabbir/Stile) (Supreme Quality) tiles of approved colour and shade in toilets, including ¼” thick neat cement buttering on back of tiles, remaining thickness made up with 1:2 cement sand rough base, complete Size: 1.5 sft to 3 sft. |  |  |
|  | a) L.G. Floor / Ground Floor | 1970 | 100 Sft |
|  | b) First Floor | 2203 | 100 Sft |
| 404 | P/F flooring comprising 12” x 12” (Master/Shabbir/Stile) supreme quality non glazed skid free tiles of approved shade set in neat cement paste not exceeding ¼” thick and filling the joint with neat while cement slurry using matching colouring i/c ¾” thick base in 1:3 cement sand mortar laid over a bed of 1 ½” thick P.C.C 1:2:4 i/c cleaning & curing etc, complete. |  |  |
|  | a) L.G. Floor / Ground Floor | 401 | 100 Sft |
|  | b) First Floor | 542 | 100 Sft |
| 405 | Providing and laying marble flooring comprising 24″ x 12″ x 7/8″ tiles resized fixed in neat cement buttering laid over 1¼″ thick P.C.C. 1:2:4 using ¼″ down coarse aggregates to final level/slopes, including filling joints with white cement, curing, grinding, sand papering and polishing in chemical polish complete as per specification. Marble quality to be “Badel-light/Ziarat Light Gray” and subject to prior approval by the Engineer, at any floor. | 25 | 100 Sft |
| 406 | Providing and fixing marble skirting cladding etc.in 24"x4"x 3/8" pre-polished (in chemical polish) tiles with neat cement buttering over 1/2" thick 1:3 cement sand rough plaster including filling joints, curing etc., complete as per specification. Marble quality shall be the same as used in the flooring and shall comply with the pattern requirements as directed. | 20 | 100 Sft |
| 407 | Chemical Polishing to existing marble and Terrazzo floor including grinding, cement filling in required shade etc Complete as directed by Engineer/Incharge. | 15125 | 100 Sft |
| 408 | Providing and laying floor of 3/4 inches (19 mm) thick Granite stone Pakistani Original upto 36 Sq. inches (232 Sq.cm.) size fine dressed on the surface without winding in ground floor over 1 inch (25 mm) lime mortar 1:2 (one lime, two sand) setting the marble tiles with grey cement slurry including curing rubbing and polishing etc. complete, including the cost of mortar. | 111 | 100 Sft |
| 409 | Providing and laying terrazzo dado and floor 1-1/2 inches (38 mm) thick in overhead water tank consisting of 1/2 inch (13 mm) thick topping 1:2 (one grey cement 2 approved marble chips) over a base of 1:2:4 (one cement, two sand and four screened graded bajri) cement concrete 1 inch (25 mm) thick and cement plaster 1:3 on walls, curing, cutting, rubbing and polishing etc. complete. | 369 | 100 Sft |
| 410 | Providing and laying terrazzo floor 1-3/4 inches (44 mm) thick consisting of 3/4 inch (19 mm) thick topping 1:2 (1 white cement 2 approved marble chips No. 0 to 6) in ground floor over a base of 1:2:4 (one cement, two sand and four screened graded bajri) cement concrete 1 inch (25 mm) thick laid in panels or pattern including form work, curing, cutting, rubbing and polishing etc, complete... | 9790 | 100 Sft |
| 411 | Providing and laying 1/2" (13mm) thick marble patties (panel strips) up to 1-1/2" (38 mm) depth in floor and dado of any description in all floor. | 6168 | Rft |
| 412 | Providing and laying 1:2:4 cement concrete khuras 2" (51 mm) thick 2 ft. x 2 ft. (610 mm x 610 mm) size on roof including finishing with cement plaster 1:4, curing etc. complete. | 13 | Each |

Bill No. 1.5 Title: Plasterer & Pointer

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | 1) All non-compression joints between concrete and masonry shall be specially treated before plastering, as under:- |  |  |
|  | A 8-inch-wide heavy gauge approved quality expanded metal strip shall be tightly fixed centrally over the joint with 1 inch steel nails 12 inches center to center longitudinally on both side of joint. |  |  |
|  | Rates for all kinds of plaster include the cost of labour as well as the materials related to providing/fixing this expanded metal and no separate/additional payment shall be made for this work/item. |  |  |
| 501 | Provide, apply, finish smooth, cure, etc. ½" thick 1:5 cement sand internal plaster to wall up to any height etc. complete. |  |  |
|  | a) L.G. Floor / Ground Floor | 4366 | 100 Sft |
|  | b) First Floor | 355 | 100 Sft |
| 502 | Provide, apply, finish smooth, cure, etc. 3/8" thick 1:3 cement sand ceiling plaster, complete up to any height. |  |  |
|  | a) L.G. Floor / Ground Floor | 606 | 100 Sft |
|  | b) First Floor | 110 | 100 Sft |
| 503 | Provide, apply, finish smooth, cure, etc. ¾″ thick 1:4 cement sand external plaster, complete upto any height. |  |  |
|  | a) L.G. Floor / Ground Floor | 818 | 100 Sft |
|  | b) First Floor | 42 | 100 Sft |
| 504 | Plastering 1/2" thick in patches in cement and sand mortar 1:4 including curing and scraping old damaged portion in regular shape and removal of rubbish from the site etc. complete as per direction of Engineer-in-Charge in any floor. | 458 | 100 Sft |

Bill No. 1.6 Title: Carpenter & Joiner

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 601 | Providing and fixing best quality deodar wood frames for doors, windows, ventilator, clerestorey windows, cupboard frames and trollies work including the cost of required number of iron holdfasts (1½″ x 1/8″ x 9″ long) PCC 1:2:4 and painting the back side/embedded portions of the frame in 2 coats of hot bitumen etc. complete. | 38 | Cft |
| 602 | Providing and fixing deodar wood 1-1/2" thick shutters partly paneled partly glazed with 5 mm thick plane glass including plywood facing on back side of 1-1/2" thick strip panelled portion as per drawings, door fittings to be as per hardware schedule i/c hinges, tower bolts and handles etc. complete but excluding the cost of door lock. | 577 | Sft |
| 603 | Providing and fixing approved chromium plated brass sliding (aldrop) bolts 9″ x 5″/8″ with necessary chromium plated brass screw and galvanized iron bolts and nuts. | 34 | Each |
| 604 | Providing and fixing mortise door lock with handle and duplicate keys, made in Hungry (imported) or equivalent. | 21 | Each |
| 605 | Providing and fixing with brass screws or specially supplied screws hydraulic Door Closer imported heavy duty Ruby Make (Japan) medium size, complete as per specification. | 19 | Each |
| 606 | Providing and fixing 1″ thick best quality deodar wood planking in shelves or almirahs with deodar wood battens 1″ x 1½″ fixed in wall with screws and rawl plugs, etc. | 6 | Cft |
| 607 | P/F kitchen cabinet wal mounted of 3/4" thick laminated board with best quality deodar woood frame and beading 1/8" thick hard board backing required section in approved pattern etc. fixed on wall with reawal pugs, screws i/c cost of glue , null , piano hinges, handles and painting or ploishing etc. as drawing complete all as respect. | 22 | Sft |
| 608 | Providing and fixing on door frame etc. with sunken iron screw best quality deodar wood molding/ beading/ architrave etc. of approved design (2¼″ x 5/8″ x 3/8″ Taper), as per drawing complete. | 553 | Rft |
| 609 | Providing and fixing Gypsum board 2' x 2'x 12mm tiles ceiling including Aluminum T & L angle 1" x 1" i.c hanger clips jointing clips and G.I Wire etc complete as required in any floor. | 4967 | Sft |
| 610 | Providing and fixing windows roll up blinds sunscreen,darkout,or translucent fabric any colour/shade best quality (imported) with manual roll up arrangements complete in all respect with all other required accessories,and hardware from the approved suppliers and manufacturers,as per direction of Engineer incharge. | 937 | Sft |
|  | Note:- For all work of Aluminum doors, windows, ventilator and partitions etc. use:- i) Lock of MICO - Taiwan ii) Imported sliding bush rubber. iii) Rollers of METEX - Japan. iv) Fly screen (Jali) of OPELLETE with full patty hinge. v) Hinges of min. 3 mm thickness. vi) Imported magnetic catcher, handles & stay bars. vii) Glazing in tinted glass 5 mm thick. viii) All sections in original Aluminum/Champagne color and M/s Krudson, Lucky Pakistan Cables And A.C.P Deluxe Model,. ix) All work to be as per drawing or as directed by the Engineer, complete in all respects. x) Where fixed glazing is shown along with openable hinged or sliding door/window shutters, the fixed glazing portion shall be paid under the items for hinged or sliding shutters nearest or adjacent to the fixed portion provided that the fixed and and openable portion are constructed in. xi) Fixed glazing in separate/independent frames even adjacent to openable portions, shall be paid as partitions. |  |  |
| 611 | Providing and fixing fully glazed Silver anodized aluminium Sliding windows as per British standard manufactured by Lucky, Alcop, Krudson, Pakistan Cables and A.C.P. (fixing through their approved fabricators), Prime model box section 101mm x 30mm and 2mm thick including the cost of aluminium netting ,fittings, with all accessories cutting hole etc. and making good damages to walls etc. complete as required in any floor as per direction of engineer-in-charge. | 156 | Sq.ft |
| 612 | Providing & fixing aluminum doors having fixed glazed portion and hinged shutter with handles, hydraulic door closer (Ruby-Japan) or equivalent one on each shutter, door stopper, tower bolts and door lock (Alpha - Japan) main frame to be of 101 x 32 x 2 m.m. thick, shutter frame to be 65 m.m. x 42 m.m. x 2 m.m. thick, with bottom member 101 x 42 x 2 m.m. having corrugated kick plate 1.2 mm thick & of 1׳-6" height, sections to be of M/s Kruddson or Lucky, complete as per drawings, specification and direction of the Engineer. | 69 | Sq.ft |
| 613 | Providing and fixing aluminum windows/ventilators/partitions having fixed glazing in 5 mm clear glass, in frames including divider members of 101 x 44.55 x 2 m.m./101 x 64 x 2 m.m., sections made of M/s Kruddson or Lucky, in original aluminum colour, complete as per drawings, specification and direction of the Engineer. | 273 | Sq.ft |
| 614 | Providing and fixing with brass screws or specialty supplied screws hydraulic door closer Rayobi of approved design No. 62, Japan make, cutting wood etc., to required shape and size as per direction of engineer-in-charge. | 2 | Each. |

Bill No. 1.7 Title: Metal Worker & Glazier

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 701 | Providing and laying mild steel perforated grating over drain as per direction of the Engineer-in-Charge complete. | 71.00 | Kg |
| 702 | Providing and fixing 16 oz. (4.89 Kg/ Sqm.) aluminium sheet covering to expansion joints in walls and columns as per approved design including providing clamps and fixing with screws and Rawal plugs or wooden gutties including making slotted holes etc. complete. | 141.00 | Sft |
| 703 | Provide & fix M.S. grills in 3/8″ x 3/8″ square flat iron or M.S. Bar to windows/ventilators etc., complete, including welding, grinding and finishing as per Specs. Clause 10.5 i/c one coat of red oxide paint (Fixed at site as per Engineer Incharge instruction). | 403 | Kg |
| 704 | Providing and fixing M.S. tees, angles, channels and flats including welding all the sides of the section at the junction, gusset plates, bolts, nuts etc. complete with hoisting and erecting in position in steel trusses, frames, door, towers, stairs etc. of any design in ground floor. | 44 | Kg |
| 705 | Providing and fixing M.S. molded steel door frame of 4" x 2-1/2" (102 mm x 64 mm) manufactured from mild steel sheet of 18 gauge (1.41 mm) conforming to BSS. 1245 having a single rebate size 1-1/2" x 1/2" (38 mm x 13 mm) with provision of 3 Nos. M.S. plate, section 1-1/2" x 1/4" (38 mm x 6 mm), (2 Nos. 6 inch long welded with frame at not less than 10 points and 1 No., 12 inch long welded with frame at not less than 20 points), with holes and threads for fixing steel hinges, fitted with one locking box of same sheet (point welded inside the frame), 6 Nos. 6 inches long flat iron fixing lugs, of 1-1/4" x 3/16" section, treated with special red oxide primer coat all around including cutting holes and filling the cavity with cement concrete 1:2:4 etc. in any floor at any height complete as directed by the Engineer-in-Charge. | 87 | Rft |
| 706 | Fabrication and fixing of the Fiber shed consisting of 5-Ply fiber sheet and 2"x2" M.S. Box pipe 16 guage main frame beams and central bracing 2"x2" at 24" c/c diagonal and 2"x2" support upto 2 feet c/c over 4" dia M.S. pipe 16 guage 10-14 feet heigh i/c the cost of excavation , with pcc 1:2:4 and rcc with steel and MS steel plate 1/2" thick in foundation , size 2'x2' cutting fitting, welding paiting etc. complete as per direction of engineer incharge. | 252.00 | Sft |

Bill No. 1.8 Title: Painter & Decorator

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 801 | Provide and apply one coat of Dulux/Robbialac Primeon/Alkali Resisting Masonry Sealer and minimum 3 coats of Vinyl Emulsion/ Robbaialac/Dulux, including surface preparation, filling and rubbing etc., complete in all respects, upto any height at any floor. | 7579 | 100 Sft |
| 802 | Provide and apply enamel painting with ready mix paint of Dulux/Robbialac make, 3 coats over and i/c the coat of priming/filling/finishing, complete at any height in any floor as per specifications, on.- |  |  |
|  | i): Wood Work. | 487 | 100 Sft |
|  | ii): Steel work i/c grills and plastered surfaces | 1373 | 100 Sft |
| 803 | Providing/applying 3 coats of ICI weather Shield paint to plastered surface in approved colour/shade and according to the approved colour scheme including applying primer of same manufacturer. The surface is to be properly prepared by rubbing, sand papering, cleaning, filling and making smooth before applying the priming coats, complete as directed any height at any floor. | 3845 | 100 Sft |
| 804 | Providing/applying french or spirit polishing two coats of approved make and shade on woodwork i/c rubbing, filling for kitchen/service counter and cabinet at any height in any floor. | 902 | 100 Sft |
| 805 | Repainting two coats with vinyl emulsion ready mixed paint of approved Dulux/Robbialec/Paintex make and shade to palstered surface etc. i/c sand papering, filling (where requird) the surface as directed by the Engineer Incharge, in any floor. | 60689 | 100 Sft |
| 806 | Repainting two coats with Plastic emulsion of ready mix paint of approved Dulux/Robbialec make and shade to plaster work i/c scraping, filling (where required) and sand papering the surface as directed by the Engineer Incharge, in any floor. | 2193 | 100 Sft |
| 807 | Repainting two coat with enamel paint of approved make & shade (Robbilac) i/c cleaning, scraping the old surface. |  |  |
|  | i). Wood Work | 3660 | 100 Sft |
|  | ii). Iron make | 5657 | 100 Sft |
| 808 | Repainting two coats with weather shield paint of (ICI) Dulux make and shade on graphic surface/ plaster surface (External) i/c cleaning, fling (where required) and sand papering the surface as directed by the Engineer Incharge, in any floor. | 16456 | 100 Sft |
| 809 | Providing & Fixing Chakwal stone facing (laid flat in courses) in cement mortar 1:3 cement pointed in deep cut grooves and in any pattern in any colour/shades as per direction of the engineer-in-charge including the cost of rubbing, curing etc complete in any height and any floor. | 10 | 100 Sft |
| 810 | Providing & laying wall finish coat of any colour graffiate wall coating 2.5 mm to 3 mm thick straight and swirt for exterior surface at any hight and any floor. | 10 | 100 Sft |

Bill No. 1.9 Title: Water Supply, Sanitary, Plumbing Fixtures & Sewerage.

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 901 | P/Fixing of water closet (ICL/Forte/3Star) make orisa-900 white export quality with low down Plastic flushing cistern (Faisal/TurkPlast) 3 gallon capicity i/c syhonic fitting 1/2" dia C.P brass stop cock (Magna/Faisal/Sonex) C.P connection 1-1/2" dia G.I flush pipe with extra bend length fitting clamp P-Trap etc complete. | 10 | Each |
| 902 | Providing and fixing wash-hand Basin (ICL/Forte/3Star) make Fregate/ 60 cm (white export quality) including brackets, bracket bolts, chain plug, ½″ dia C.P. inlet connections, C.P. brass stop cock (Magna/Faisal/Sonex), 1¼″ dia C.P. brass bottle trap with union coupling and G.I/ C.P waste complete including painting pipes and brackets as specified including ½″ dia pillar cock of (Magna/Faisal/Sonex) make (brackets of 1״ x 1״ x 1/8״ of Angle Iron). | 12 | Each |
| 903 | Extra over item above for providing and fixing matching white glazed earthen ware pedestal (ICL/Forte/3Star) make, export quality. | 12 | Each |
| 904 | Extra over item above for providing single hole chromium plated mixer tap ½" dia (Magna/Faisal/Sonex), instead of the pillar cock i/c P/F one additional C.P. Brass stop cock (Magna/Faisal/Sonex) complete. | 12 | Each |
| 905 | Providing and fixing 24″ x 18″ mirror of imported Belgium glass 5mm thick or equivalent as approved, with plastic frame complete fixed to wall with spacers using C.P. screw 2″ long complete. | 12 | Each |
| 906 | Providing and fixing Plastic Rack/Shelf 24″ long of Master make with C.P. screws as approved by the Engineer Incharge. | 12 | Each |
| 907 | Providing and fixing 24″ x 3/4″ dia approved quality Chromium Plated brass towel rail complete with brackets fixed with 1″ long C.P. brass screws and rawal plug. | 6 | Each |
| 908 | Providing and fixing plastic toilet paper holder of Master make with C.P. screws. | 6 | Each |
| 909 | Providing and fixing un plasticized polyvinyl chloride pipe (U.P.V.C) “D” class (Acid Proof) of (Dadex/TurkPlast/Popular) with specials and clamps etc, including cutting and fitting complete with and including the cost of breaking through walls and roof and making good etc., after cleaning the pipe and cartage within 10 miles (16.09 km). |  |  |
|  | i) 6”dia | 87 | Rft |
|  | ii) 4”dia | 322 | Rft |
|  | iii) 3”dia | 189 | Rft |
| 910 | Providing, fixing, jointing, testing and disinfecting good quality G.I. Pipes and specials i.e. Sockets, Tees, Elbows, Bends, Cross, Reducers, Clamps, Nipples, Plugs and Unions etc. including two coats of approved bitumastic paint on concealed lines and enamel paint on surface lines, for:- Water pipelines burried in trenches, under floor or suspended from slab and duct or chased in walls i/c repair of chases and ducts where required, complete.- (IIL Medium). |  |  |
|  | i): ½″ dia (14 SWG) | 69 | Rft |
|  | ii): ¾″ dia (13 SWG) | 44 | Rft |
|  | iii): 1½″ dia (11 SWG) | 29 | Rft |
|  | iv): 2″ dia (11 SWG) | 25 | Rft |
| 911 | Providing and fixing gun metal full way heavy duty gate valves for water, (Asia/Anwar/Baig) with wheel, threaded ends, complete as specified and as approved and replacing the old one. |  |  |
|  | i): ¾″ dia | 3 | Each |
| 912 | Providing and fixing English W.C. (ICL/Forte/3Star) Export Quality, white glazed earthen ware wash down W.C. pan complete with and including the cost of white bacolite plastic seat and lid with C.P. brass hinges and buffers, 3 gallons white glazed earthen ware coupled low level flushing cistern with siphonic fittings, 1-1/2" dia white porcelain enameled flush bend, 3/4" dia G.I. warning pipes carried out side and bent vertically down wards and making requisite number of holes in walls, plinth and floor for pipe connection and making good in cement concrete 1:2:4 including 1/2" dia C.P. Brass stop cock (Magna/Faisal/Sonex), and C.P. flexible connection, S-trap, complete set, as directed and replacing the old one. | 6 | Each |
| 913 | Provide and install PVC rainwater down pipe (Dadex/TurkPlast/Popular) with mild steel clamps bolts and nuts fixed in cement concrete 1:2:4 etc complete. |  |  |
|  | a) 4" inside dia | 420 | Rft |
|  | b) 3" inside dia | 50 | Rft |
| 914 | Provide and install 4" dia PVC bend (Dadex/TurkPlast/Popular) of any degree for rain water down pipe with clamps bolts and nuts fixed in cement concrete 1:2:4 etc. complete. |  |  |
|  | a) 4" inside dia | 26 | Each |
|  | b) 3" inside dia | 6 | Each |
| 915 | Providing and fixing 1/2" dia C.P. bib cocks i/c c.p nipple (heavy duty) Magna/Faisal/Sonex, made, as approved. | 12 | Each |
| 916 | Providing and fixing 1/2" dia C.P. T. Stop cocks i/c c.p nipple (heavy duty) Magna/Faisal/Sonex made, as approved. | 14 | Each |
| 917 | Providing & fixing stainless steel floor trap grating (JALI) etc. (Magna/Faisal/Sonex) complete as par size. | 16 | Each |
| 918 | Providing and fixing 6" x 4" (150 mm x 100 mm) cement concrete gully trap with 4 inches (100 mm) outlet, complete with 4 inches (100 mm) thick 1:2:4 cement concrete for bed and kerb, 1/2 inch ("13 mm) thick cement plaster 1:3 to the kerb, C.I. grating 6 inches x 6 inches (150 mm x 150 mm) and C.I. cover and frame 12" x 12" (305 mm x 305 mm) (inside) etc., complete. | 13 | Each |
| 919 | Providing, laying, fixing, testing and disinfecting, polypropylene pipelines for cold and hot water supply (Dadex/TurkPlast/Popular) as per DIN 8077/8078 PN-20 for pipe and DIN 16962 PN-25 for fittings or equivalent BS specification, Firat (Turkey make) or formul (Turkey make) make as approved by the Engineer Incharge complete in all respects to their entire satisfaction, including specials such as tee, cross, reducer, bend, union, elbow, plug, socket, etc., supported on walls or suspended from slab or run in chases including supports, cutting and making good the same as necessary to the structure, excavation and backfilling in layers complete in all respects. |  |  |
|  | i) 25 mm o/d | 375 | Rft |
|  | ii) 32 mm o/d | 248 | Rft |
| 920 | Providing and fixing bathroom accessories set (Magna/Faisal/Sonex) smart set of 7 pieces consist of one cosmetic shelf, one towel rod with bracket, one soap dish, one toothbrush holder with glass and cover, one tissue paper holder one double hook one towel ring etc complete of approved quality as per direction of Engineer in-charge. | 2 | Each |

Bill No. 1.10 Title: Roofer & Water Proofer

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1001 | Providing and laying one coat of bitumen emulsion at the rate of one gallon per 100 Sft on wall and floor 10/20, 60% and 80/100, 40% ratio i/c surface preparation of concrete and two coats of bitumen at rate of 34 Ibs per 100 Sft total for both coats at any height. | 1586.00 | 100 Sft |
| 1002 | Provide, lay and install, including heating, curing etc. roof treatment consisting of PCC 1:2:4 (by volume) screed 4" x 1" thick (average 2½" thick) to required gradient, and in approved size panels, using Lawrancepur Sand and approved graded crushed stone (1/8" to 3/8" size) as fine and coarse aggregates respectively, one coat of approved bitumen primer at the rate of one gallon per 100 Sft i/c surface preparation of concrete screed, two coats of bitumen at the rate of 34 Lbs. per 100 Sft (total for both coats), and blinding with crush pan complete as directed. | 1481.00 | 100 Sft |

Bill No. 1.11 Title: Dismantling & Demolishing

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1101 | Removing cement plaster of any thickness from walls and roof and disposing of the rubbish etc. from site as directed . | 3037 | 100 Sft |
| 1102 | Dismantling of flooring tiles floor, Dado or skirting i/c base mortar and stacking salvaged material and disposing of surplus stuff complete. | 8916 | 100 Sft |
| 1103 | Dismantling of brick masonary in cement mortar including, stacking salvaged material serviceable) and disposal of surplus material complete. | 1741 | 100 Cft |
| 1104 | Dismantling of RCC i/c separating Reinforcement from concrete i/c stacking at site and disposing of unserviceable materail. | 118 | 100 Cft |
| 1105 | Dismantling cement concrete work i/c disposing of the material complete. | 2670 | 100 Cft |
| 1106 | Taking outdoor and window frames with or without hold fasts cutting walls, stacking salvaged material (serviceable) and disposing of unserviceable material as directed. | 25 | Each |
| 1107 | Taking out carefully shutters of doors in any floor and stacking as directed Engineer-in-Charge. | 25 | Each |
| 1108 | Taking out wooden planking / aluminum partition and stacking the material as directed Engineer-in-Charge. | 4439 | 100 Sft |

Bill No. 1.12 Title: Miscellaneous

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing/applying 2 Renewal coats of top coat on Existing graphic paint (Rockwall) (i/c patch/ repair where required) Topcoat of approved colour/shade and scheme complete at any height and any floor as directed by Engineer/ Incharge. | 110 | Sft |
| 2 | Cleaning and Refilling of existing Bath & Floor Tiles joint filling with filler and cleaning with Tile Chemical Cleaner at any height & any floor. | 4306 | Sft |
| 3 | Providing & Fixing aluminum wire Gauze in existing Windows as par instruction of Engineer In charge complete. at any height & any floor. | 272 | Sft |
| 4 | Providing & Fixing Imported frosted glass paper to aluminum window, door etc as par approved pattern complete in all respect at any height & any floor. | 560 | Sft |
| 5 | Providing and fixing 1/32" thick 4" wide and length as per door width stainless steel kick plate including stainless steel screw etc. | 168 | Rft |
| 6 | Providing and fixing Aluminum sheeted corrugated kick plate (D-61-A) to doors, 1.6 mm thick as par direction of Engineer Incharge. | 32 | Rft |
| 7 | Replacing the Aluminum Door Lock of approved Quality. | 11 | Each |
| 8 | Repair and maintenance of aluminum doors/window including replacing of hinges catchers, rubber fixing etc complete. | 464 | Sft |
| 9 | Providing and fixing flapper rubber to aluminium wire mesh shutter to stop the mosquito / fly at any height complete with all respect. | 398 | Rft |
| 10 | Providing and fixing C.P Muslim Shower Master/Faisal make complete in all respects. | 6 | Each |
| 11 | Removing defective plywood from existing shutters and fixing new Ash veneer plywood 3.5 mm thick on same door and refixing of all hardware such as door lock tower bolts and kick plates at any height. | 98 | Sft |
| 12 | Repairing of existing venetian blind i/c replacing runner hooks etc. | 577 | Sft |
| 13 | P/Replacing of Bib cock/Mixer spindle (Brass) of different brand as per site requirement. | 11 | Each |
| 14 | Providing & applying high grip water proofing on roof top i/c scraping the surface and cleaning with steel brush applying water base primer coat and applying 3 mm pg 3 membranes with gas torch etc., complete as direction and specification. | 25 | Sft |
| 15 | Removing and relaying existing choka (Insulation Tile) from roof top as specified including cleaning and scraping the roof surface with vinyl and steel brush and disposal of rubbish material from the site as directed. | 9790 | Sft |
| 16 | Proper cleaning of underground/overhead water tank including removing of scale/ fungus using proper manner/ chemical at any height and any floor complete as per the direction and satisfaction of Engineer Incharge. | 1415 | Cft |
| 17 | Providing & Fixing of Hydraulic Seat cover at existing English WC complete same as existing cover. | 3 | Each |
| 18 | Providing & Fixing of Light Plug 13 Amp and also replacing with the old one. | 36 | Each |
| 19 | Replacing of 5 or 6 mm thick plain glass of the existing window/ventilators and skylights complete in all respect.up to any height. | 279 | Sft |
| 20 | Providing and Fixing Chock for Tube light, Philips made 40 watt. | 20 | Each |
| 21 | P/Replacing of flushing tank syphon of good quality complete as per the site requirement. | 4 | Each |
| 22 | Providing & fixing curtain with lining of approved good quality cloth including railing pipe 1" dia 18 Swg i/c end cap, rings and hooks etc. of approved quality. Complete with all respect as directed by Engineer Incharge. | 413 | Sft |
| 23 | Cleaning & Re-opening of existing sewerage line of dia 6" and 9" i/c Roding pressure pump and disposing of debris, filling of concrete joints complete in all respect. | 85 | Rft |
| 24 | Providing and Fixing door & window shad using 5mm thick Fiberglass sheet over frame. a)- Frame using 1-1/2" x 1-1/2" 16 SWG M.S pipe including one coat of red oxide & thee coat of enamel paint. complete in all respects upto any height (as per drawing and instruction of site Engineer). | 147 | Sft |
| 25 | Removing existing installed electrical accessories (Tube lights/ceiling lights/Fans etc.) carefully including the cost of removing of wiring (cable and duct etc) and handing over to the directorate of works as required and directed by the engineer in-charge. | 35 | Each |
| 26 | Providing and applying termite proofing inside the building chemical component of (Agenda) drilling 18" to 24" @ 3sft . after filling the chemical caping the hole with C.S 1:2 ratio. complete and as per instructions of engineer in charge. (Area will be measured in Sft). | 19767 | Sft |

Bill No. 2 Title: Electricity

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Note:- i) All work concerning Electricity shall be executed in accordance with the contract specification and the requirement of WAPDA by an electrical licensed Subcontractor. Upon completion, all work shall be tested and a test report submitted in accordance with the requirements/specification of WAPDA. The contractor shall also be responsible for having the work cleared/approved from WAPDA. |  |  |
|  | ii) All cables must be of M/s Pakistan, Fast or GM cable. |  |  |
|  | iii) All switches, sockets, power points, light points etc. must be of Schneider / Philips / Clipsal. |  |  |
| 1 | Wiring of light/fan points wired with 3/.029 insulated cable run in 18 SWG gauge black enamelled steel conduit recessed in walls, columns, ceiling, floor etc. including all accessories such as junction boxes, outlets etc. as required and 14 SWG bare copper conductor all controlled by one 5 Amps. 250 volts on - off switch including cost of conduit and cables etc., but excluding the cost of switch. | 193 | Point |
| 2 | Same as in Item E-1, but for three pin 5 Amps. 250 V socket outlet excluding the cost of switch socket unit and not connected to power circuit. | 63 | Point |
| 3 | Wiring of power plug switch wired with 7/.036 (4mm2) PVC insulated cable run in 18 SWG black enamelled steel conduit recessed in walls, columns, ceilling, floor etc., including all accessories such as junction boxes, outlets etc., as required and 14 SWG bare copper conductor all along the length of the conduit and controlled by one 15 Amps. 250 V MCB, cost including conduit and cable etc., but excluding the switch and M C B. | 95 | Point |
| 4 | Same as Item E-1, but circuit wiring wired with (2.5 mm2) 7/.029 single core PVC cable. | 105 | Circuit |
| 5 | Supply, installation, testing & commissioning of exhaust fans with best quality approved steel shutter complete in all respects, i/c ceiling rose. Fan shall be latest model of Pak or Asia make and replacing the old one. |  |  |
|  | i): 12" sweep | 6 | Each |
|  | ii): 18" sweep | 3 | Each |
| 6 | Providing and fixing PVC cable duct (Adam Jee) |  |  |
|  | i) 16x16 | 195 | Meter |
|  | ii) 16x25 | 160 | Meter |
|  | iii) 16x38 | 105 | Meter |
| 7 | Supply, installation, testing & commissioning 56" Sweep (1422mm) ceiling fans complete with canopy, dimmer and down rod etc. fan shall be latest model of Pak or Asia branch. Down rod, if beyond the suppliers length, shall be paid as G.I. pipe of the same gauge and dimeter, and shall be duly painted. | 11 | Each |
| 8 | Supply, installation, testing & commissioning of Bracket Fans 18" Sweep with best quality approved complete in all respects, Fan shall be latest model of Pak or Asia make: | 6 | Each |
| 9 | Replacement of un-serviceable and burnt out 6 to 32 Amps single pole moulded case citcuit breaker Siemens with new one as per exising size and sample as required. | 15 | Each |
| 10 | Replacement of un-serviceable and burnt out 60 Amps (High breaking Capacity ) Siemens Triple pole moulded case circuit breaker with new one as per exising size and sample as required. | 2 | Each |
| 11 | Replacement of un-serviceable and burnt out 100 Amps (High breaking Capacity ) Siemens Triple pole moulded case citcuit breaker with new one as per exising size and sample as required. | 1 | Each |
| 12 | Replacing of un-serviceable and burnout 6 Amp. 3 pin switch socket unit (PPI) with new on as per existing size and sample as required. | 2 | Each |
| 13 | Ditto as above but 15 Amps. 3 pin plug. | 29 | Each |
| 14 | Ditto as above piano switch 6 Amps. | 178 | Each |
| 15 | Replacement of un-serviceable and burnout flush type piano fan dimmer with new one of all kind on existing board as per existing size and sample as required. | 4 | Each |
| 16 | Replacement of un-serviceable and burnt out 40/36 watt TLD lamp with new one as per existing size and sample as required. | 5 | Each |
| 17 | Replacement of u/s and w/o Fan Bearing set with new one as per existing size after the dismentalling the ceiling fan all sweep complete in all respect i/c cartage and fixing charges as required. | 4 | Each |
| 18 | Rewinding of burnt out A.C Ceiling Fan all kind sweep with suitable size of winding wire I/c varnishing /backing and testing on load complete in all respect i/c cartage and fixing charges as required. | 4 | Each |
| 19 | Providing, Installation, Testing & Commissioning of LED Tube / Batten Lights (SUNLIGHT, PIERLITE, ORIENT) including driver having warm / Neutral White / cool white colour,80-90 lm/watts efficiency, 0.9 or above power factor, 10000-15000 hours life cycle, including connection, complete in all respect as approved by the Engineer Incharge. |  |  |
|  | a) 2 Feet 18-20 Watts | 11 | Each |
|  | b) 4 Feet 36 Watts | 25 | Each |
| 20 | Providing, Installation, Testing & Commissioning of LED spot lights (PAKLITE / GALAXIE / E MAX) including driver, having warm / neutral white / cool white colour, 80-100 lm/watts efficiency, 0.5 - 0.70 power factor, 20000-25000 hours life cycle, including connection, complete in all respect as approved by the Engineer Incharge. |  |  |
|  | a) 10-12 Watts | 47 | Each |
| 21 | Providing, Installation, Testing & Commissioning of Flood Lights (IP66) including driver, having neutral white / cool white colour,110 -120 lm/watts efficiency, 0.9 or above power factor, 20000-25000 hours life cycle, including connection, complete in all respect as approved by the Engineer Incharge. |  |  |
|  | a) 180 - 210 Watts | 4 | Each |
| 22 | Dismantling of all type of fittings including finishing and making good the wall as required. | 400 | Each |
| 23 | Providing & fixing 10 Amps Polycarbonate flame retardant S.P switch with fancy required gang plate fixed on die fabricated, powder coated metal board recessed in wall or column including connection as required.(Schneider / Philips/TJ) | 220 | Each |
| 24 | Providing & fixing 13 Amps Polycarbonate flame retardant 3-Pin switch socket unit with fancy gang plate fixed on die fabricated, powder coated metal board recessed in wall or column including connection as required.(Schneider / Philips/TJ) | 105 | Each |
| 25 | Providing & fixing 15 Amps Polycarbonate flame retardant 3-Pin switch socket unit with fancy gang plate fixed on die fabricated, powder coated metal board recessed in wall or column including connection as required.(Schneider / Philips/TJ) | 68 | Each |
| 26 | Providing & installing False Ceiling fan (Box fan) with remote 24" (2'x2')size including conections,complete in all respect as approved by the Engineer incharge.(Voldam/Pak/Royal) | 55 | Each |
| 27 | Providing & fixing Cubical type metal sheet Distribution board with locking arrangement as per drawing ,vermin protected duly powder coated paint i/c all fastening material including wiring with suitable gauge PVC x PVC wire complete as required. | 12 | P/Sft |
| 28 | Providing & fixing 6 to 32 Amps, DP MCCB fixed on metal board recessed in the wall duly covered with fiber sheet, I/c inter-connection as required. | 4 | Each |
| 29 | Providing & fixing 6 to 32 Amps, SP MCCB fixed on metal board recessed in the wall duly covered with fiber sheet, I/c inter-connection as required. | 30 | Each |
| 30 | Providing & fixing rotary type, 32 Amps, 3 Phase, Phase selector switch fitted on all kind of existing board including incoming & outgoing connection as required. | 2 | Each |
| 31 | Providing & fixing pilot lamp set fitted on existing board I/c connection as required. | 3 | Set |
| 32 | Providing & fixing 4 core, PVC insulated & PVC sheathed, 600/1000 volts grade with stranded copper conductor , un armoured, L.T cable fixed on surface with heavy gauge GI flat strip saddle at suitable distance as per Pak. P.W.D specification as required. | 200 | P/Rft |
| 33 | Providing, Installation, Testing & Commissioning of recessed / surface mounted LED Panel Light 48-50 watt including driver, having warm / neutral white / cool white colour, 80-100 lm/watts efficiency, 0.9 or above power factor, 20000-25000 hours life cycle, including connection, complete in all respect as approved by the Engineer Incharge. paklite / galaxie / Emax | 105 | Each |
| 34 | Providing, Installation, Testing & Commissioning of recessed / surface mounted ceiling lights / downlights 18-20 watts including driver, having warm / neutral white / cool white colour, 80-90 lm/watts efficiency, 0.9 or above power factor, 20000-25000 hours life cycle, including connection, complete in all respect as approved by the Engineer Incharge. sunlight, pierlite, future technology, orient | 36 | Each |
| 35 | Providing & fixing 100 Amps,(High Breaking Capacity) TP MCCB fixed on metal board recessed in the wall duly covered with fiber sheet, I/c inter-connection as required. Terrasaki, LS Korea, ABB Germany, Siemens, FUJI | 2 | Each |

Bill No. 3 Title: LAN & Surveillance

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 3.1 | Passive LAN Network/Ethernet Points | 150 | Node |
|  | I/O Point: |  |  |
|  | · Cat6 UTP Tool-Less Keystone Jack, PVC Back Box, Punch free shuttered I/Os & Dual faceplate - complete Unit (3M/Schneider/ Panduit/Corning/ Vivanco/Clipsal or equivalent) |  |  |
|  | UTP Cable: |  |  |
|  | · Cat6 UTP 23 AWG PVC Solid Cable - 305m/Roll 65 Rolls (Corning/ 3M, Schneider/ Hikvision/ Dahua or equivalent) |  |  |
|  | OFC Cable: |  |  |
|  | · Optical Fiber cable (patch cords for uplink) (LC-LC single mode) |  |  |
|  | Switch Rack: |  |  |
|  | · with depth greater than 60 cm to support PoE switches, |  |  |
|  | · with front glass door and locking mechanism, |  |  |
|  | · fitted with dual cooling fans |  |  |
|  | · Supplied with a PDU (Power Distribution Unit) having two (02+) IEC-320-C13 sockets |  |  |
|  | Power Cable |  |  |
|  | · 3.29 dual core. (90 meter), fire resistant FAST/Pakistan Cable or equivalent |  |  |
|  | Duct |  |  |
|  | · PVC ducting in various sizes (40x40, 60x60, 16x25, 16x38). |  |  |
|  | · Adamjee or Equivalent – As per actual Use |  |  |
|  | Cable Management: |  |  |
|  | · Front cable managers for organized patch panel and switch cabling |  |  |
|  | Network Testing: |  |  |
|  | · Fluke end-to-end testing for all installed network links |  |  |
|  | Termination Accessories: |  |  |
|  | · Cat6 RJ-45 connectors |  |  |
|  | · Cable ties, screws, labels, and other miscellaneous accessories (e.g., flexible PVC pipes if required) |  |  |
|  | Compliance & Warranty |  |  |
|  | All materials and installation work must conform to specified industry standards and best practices. |  |  |
| 3.2 | Access Switches | 4 | Each |
|  | • Base Layer-3 PoE+ Access Switch with all accessories. |  |  |
|  | • Ports Required: 48 x 10/100/1000Mbps, 2 x 10G Fiber ports without SFPs. |  |  |
|  | • 2 X Dedicated Stack ports |  |  |
|  | • Forwarding Performance: 130Mbps or higher. |  |  |
|  | • Switching Capacity: 200 Gbps or higher. |  |  |
|  | • Virtual Chassis enabled. |  |  |
|  | • QoS with Voice VLAN enabled. |  |  |
|  | • 4K VLANs |  |  |
|  | • MAC Address entries >=30K |  |  |
|  | • ARP Entries >= 2K |  |  |
|  | • ACL Rules >= 2K |  |  |
|  | • QinQ, LACP, STP, RSTP, ERPS, MSTP/PVST+, LLDP, Jumbo Frames |  |  |
|  | • IPv4 Static Routing, RIP, RIPing, OSPF, OSPFv3 |  |  |
|  | • FIBv4 >=4k , FIBv6>=1k |  |  |
|  | • IGMPv1/v2/v3, PIM-DM, PIM-SM, MSDP, IGMP snooping |  |  |
|  | • Stacking support for upto 2 devices |  |  |
|  | • Support MAC authentication and 802.1x authentication, Dynamic VLAN, Dynamic ACL, RADIUS CoA |  |  |
|  | • CLI and Web interface (HTTPS) |  |  |
|  | • Telnet, SSH v2, RMON,SNMP v1, v2, and v3 , Telemetry |  |  |
|  | • Support Automation with SDN Controller and NETCONF/YANG |  |  |
|  | • 6 Queues per port, Support Traffic classification, Traffic behaviour, Traffic shaping, Congestion • avoidance/Management |  |  |
|  | • Support ARP security, DAI, SAI, CPU Défense, Storm Control and Strom Suppression |  |  |
|  | ""Operating Temperature -5C to +50C |  |  |
|  | • Humidity 5% to 95%RH"" |  |  |
|  | • 03 Years Comprehensive Warranty & Support |  |  |
| 3.3 | IP Surveillance Camera | 20 | Each |
|  | Type of cameras Bullet/Dome |  |  |
|  | Camera Max. Resolution 4MP or higher |  |  |
|  | IR Range Up to 40m |  |  |
|  | Camera Focal length 2.8mm or above |  |  |
|  | Image Sensor 1/3" Progressive Scan CMOS |  |  |
|  | Effective Pixels 2688(H) x 1520(V) |  |  |
|  | Minimum Illumination Color: 0.01 Lux @ (F1.2, AGC ON), 0 Lux w/IR |  |  |
|  | Shutter Speed 1/3 s to 1/100,000 s |  |  |
|  | Lens 4 mm fixed lens |  |  |
|  | Day/Night Mode IR cut filter with auto switch |  |  |
|  | Max. Resolution 2688 x 1520 (High quality imaging with 4 MP) |  |  |
|  | Frame Rate 25 fps (2688 × 1520, 1920 × 1080, 1280 × 720) |  |  |
|  | Video Compression H.265+/H.265/H.264+/H.264/MJPEG |  |  |
|  | Bit Rate 32 Kbps to 16 Mbps |  |  |
|  | Network Storage Support MicroSD/SDHC/SDXC card (up to 256 GB) |  |  |
|  | With Waterproof Power Supply Box/Power Adaptor |  |  |
| 3.4 | Network Video Recorder (NVR) – 64 Channel | 1 | Each |
|  | Number of Channels 64 Channels |  |  |
|  | Supported Cameras Up to 64 IP Cameras |  |  |
|  | Video Output HDMI, VGA |  |  |
|  | Recording Resolution Up to 12 MP |  |  |
|  | Incoming Bandwidth Up to 320 Mbps |  |  |
|  | SATA Interfaces 8 SATA interfaces for 8HDDs |  |  |
|  | Audio Output 1-ch, RCA (Linear, 1kΩ) |  |  |
|  | Network Interfaces 2, RJ-45 10/100/1000 Mbps |  |  |
|  | Playback Resolution Up to 12 MP |  |  |
|  | Synchronous Playback 16-ch |  |  |
|  | Decoding Formats H.265+/H.265/H.264/H.264+/MPEG4 |  |  |
|  | Network Protocols TCP/IP, DHCP, DNS, DDNS, NTP, SMTP, UPnP™, HTTPS |  |  |
| 3.5 | SATA Surveillance HDD | 2 | Each |
|  | Capacity 8 TB |  |  |
|  | Form Factor 3.5 inch |  |  |
|  | Interface SATA 6 Gb/s |  |  |
|  | Cache 128 MB |  |  |
|  | Rotational Speed 5400 RPM or higher |  |  |
|  | Maximum Transfer Rate Up to 194 MB/s sustained transfer rate |  |  |
|  | Mean Time Between Failures Up to 1.5 million hours |  |  |
|  | Workload Rating Up to 180 TB/year |  |  |
|  | Warranty 1 Year |  |  |

Notes for Bidders

      - Cost estimation of above Bill of Quantities has been done using rates of Scheduled Items derived by Pakistan Public Works Department under its CSR 2022, whereas Non-scheduled Items have been costed based on Market Survey.

      - The Employer has earmarked Provisional Sum(s) of Pak. Rs. 2.00 million for the execution of the assignment against unforeseen/additional work items, in addition to the cost that a bidder will offer in its Financial Proposal/Bid. Provisional Sums included and so designated in the Bill of Quantities which shall be included in the overall value of the contract, expended in whole or in part at the direction and discretion of the Employer in accordance with Sub-Clauses 13.4 and 13.5 of the General Condition of the Contract.

      - The Provisional Sum(s) shall be at sole discretion of the Employer and the Contractor shall have no legal claim of Provisional Sum(s) that shall be added in the contract’s overall value as separate line item. It may not be necessary that Provisional Sum(s) would be used at all. In case of any unutilized provision of Provisional Sum(s), the same shall be considered subtracted from the contract’s amount/provision.

      - The Employer and the Contractor shall work out the valuation of the unforeseen/additional work items based on the item rates as per CSR 2022, derived by Pakistan Public Works Department Scheduled Items, if no corresponding rate is available in the contract. Whereas for Non-scheduled Items for which there shall be no rate available in the contract, the rate shall be drawn by employing Market Survey using at least three sources and adopting most economical costs. The commercial and technical specifications of such unforeseen/additional shall invariably be defined by the Employer.

      - Soft Copy of the Bill of Quantities has been uploaded at the website of the University (www.aiou.edu.pk) which the bidders may download for their ease.

Technical Proposal

- Site Organization

- Method Statement

- Mobilization Schedule

- Construction Schedule

- Equipment

Site Organization

Method Statement

Mobilization Schedule

In accordance with the Particular Conditions, Sub-Clause 4.1, the Contractor shall not carry out mobilization to Site unless the Engineer gives consent.

Construction Schedule

Form EQU: Equipment

The Bidder shall provide adequate information to demonstrate clearly that it has the capability to meet the requirements for the key equipment listed in Section V, Evaluation and Qualification Criteria. A separate Form shall be prepared for each item of equipment listed, or for alternative equipment proposed by the Bidder.

| Item of equipment |  |  |
| --- | --- | --- |
| Equipment information | Name of manufacturer | Model and power rating |
|  | Capacity | Year of manufacture |
| Current status | Current location |  |
|  | Details of current commitments |  |
| Source | Indicate source of the equipment o Owned o Rented o Leased o Specially manufactured |  |

Omit the following information for equipment owned by the Bidder.

| Owner | Name of owner |  |
| --- | --- | --- |
|  | Address of owner |  |
|  | Telephone | Contact name and title |
|  | Fax | Telex |
| Agreements | Details of rental / lease / manufacture agreements specific to the project |  |

SECTION VII

STANDARD BIDDING FORMS

# Section VII: Standard Bidding Forms

Table of Forms

Letter of Bid – Technical Proposal	93

Letter of Bid - Financial Proposal	95

FORM ELI 1	98

Bidder Information Form	98

ELI2	99

Bidder’s JV Members Information Form	99

Financial Situation	106

EXPERIENCE	113

Form of Bid Security	118

Priced Bill of Quantities	120

Schedule of Adjustment Data	122

Form of Affidavit	123

Beneficial Ownership Disclosure Form	125

Letter of Bid – Technical Proposal

INSTRUCTIONS TO BIDDERS: DELETE THIS BOX ONCE YOU HAVE COMPLETED THE DOCUMENT Place this Letter of Bid in the first envelope “TECHNICAL PROPOSAL”. The Bidder must prepare the Letter of Bid on stationery with its letterhead clearly showing the Bidder’s complete name and business address. Note: All italicized text in black font is to help Bidders in preparing this form and Bidders shall delete it from the final document.

Date of this Bid submission: [insert date (as day, month and year) of Bid submission]

IFB No.: [insert number of Bidding process]

Alternative No.: [insert identification No if this is a Bid for an alternative]

To: [insert complete name of Employer]

We, the undersigned Bidder, hereby submit the first part of our Bid, the Technical Proposal

In submitting our Bid we make the following declarations:

- No reservations: We have examined and have no reservations to the bidding document, including addenda issued in accordance with Instructions to Bidders (ITB 9);

- Eligibility: We meet the eligibility requirements and have no conflict of interest in accordance with ITB 3;

- Bid/Proposal-Securing Declaration: We have not been suspended nor declared ineligible by the Employer based on execution of a Bid Securing Declaration or Bid Securing Declaration in the Employer’s country in accordance with ITB 3;

- Conformity: We hereby confirm that we have ensured sufficiency of our bid by employing all satisfactory measures, and we offer to execute works in conformity with the bidding document and in accordance with the works requirements: [insert a brief description of the WORKS]. We also unconditionally confirm the provisions that have been defined in respect of inclusion and usage of Provisional Sum(s);

- Bid Validity Period: Our Bid shall be valid for the period specified in BDS 18.1 (as amended, if applicable) from the date fixed for the Bid submission deadline specified in BDS 24.1 (as amended, if applicable), and it shall remain binding upon us, and may be accepted at any time before the expiration of that period;

- Performance Security: If our Bid is accepted, we commit to obtain a performance security in accordance with the bidding document;

- One Bid per Bidder: We are not submitting any other Bid(s) as an individual Bidder, and we are not participating in any other bid(s) as a Joint Venture member or as a subcontractor, and meet the requirements, other than Alternative Bids submitted in accordance with ITB 20;

- Suspension and Debarment: We, along with any of our subcontractors, suppliers, consultants, manufacturers, or service providers for any part of the contract, are not subject to, and not controlled by any entity or individual that is subject to, a temporary suspension or a debarment imposed by the Employer. Further, we are not ineligible under Pakistan laws;

- State-owned enterprise or institution: [select the appropriate option and delete the other] [We are not a state-owned enterprise or institution] / [We are a state-owned enterprise or institution ];

- Binding Contract: We understand that this Bid, together with your written acceptance thereof included in your Letter of Acceptance, shall constitute a binding contract between us, until a formal contract is prepared and executed;

- Not Bound to Accept: We understand that you are not bound to accept the Most Advantageous Bid or any other Bid that you may receive; and

- Fraud and Corruption: We hereby certify that we have taken steps to ensure that no person acting for us, or on our behalf, engages in any type of Fraud and Corruption.

Name of the Bidder: *[insert complete name of Bidder]

Country of Origin of the Bidder: [insert country of origin, in case of JV country of origin of lead member]

Name of the person duly authorized to sign the Bid on behalf of the Bidder: ** [insert complete name of person duly authorized to sign the Bid]

Title of the person signing the Bid: [insert complete title of the person signing the Bid]

Signature of the person named above: [insert signature of person whose name and capacity are shown above]

Date signed [insert date of signing] day of [insert month], [insert year]

*: In the case of the Bid submitted by a Joint Venture specify the name of the Joint Venture as Bidder.

**: Person signing the Bid shall have the power of attorney given by the Bidder. The power of attorney shall be attached with the Bid Schedules.

Letter of Bid - Financial Proposal

INSTRUCTIONS TO BIDDERS: DELETE THIS BOX ONCE YOU HAVE COMPLETED THE DOCUMENT Place this Letter of Bid - Financial Proposal in the second envelope marked “FINANCIAL PROPOSAL”. The Bidder must prepare the Letter of Bid - Financial Proposal on stationery with its letterhead clearly showing the Bidder’s complete name and business address. Note: All italicized text is to help Bidders in preparing this form.

Date of this Bid submission: [insert date (as day, month and year) of Bid submission]

Invitation for Bid No.: [insert number of bidding process]

Name of Project.: [insert identification]

Alternative No.: [insert identification No if this is a Bid for an alternative]

To: [insert complete name of Employer]

We, the undersigned Bidder, hereby submit the second part of our Bid, the Financial Proposal

In submitting our Financial Proposal we make the following additional declarations:

- Bid Validity Period: Our Bid shall be valid for the period specified in BDS 18.1 (as amended, if applicable) from the date fixed for the bid submission deadline specified in BDS 24.1 (as amended, if applicable), and it shall remain binding upon us and may be accepted at any time before the expiration of that period;

- Total Price: The total price of our Bid, inclusive of all applicable indirect taxes, is:

In case of only one lot, the total price of the Bid, inclusive of all applicable indirect taxes, is [insert the total price of the bid in words and figures, indicating the various amounts and the respective currencies];

In case of multiple lots, the total price of each lot, inclusive of all applicable indirect taxes, is [insert the total price of each lot in words and figures, indicating the various amounts and the respective currencies];

In case of multiple lots, total price of all lots (sum of all lots), inclusive of all applicable indirect taxes, is [insert the total price of all lots in words and figures, indicating the various amounts and the respective currencies];

- Commissions, gratuities and fees: We have paid, or will pay the following commissions, gratuities, or fees with respect to the bidding process or execution of the Contract: [insert complete name of each Recipient, its full address, the reason for which each commission or gratuity was paid and the amount and currency of each such commission or gratuity].

| Name of Recipient | Address | Reason | Amount |
| --- | --- | --- | --- |

(If none has been paid or is to be paid, indicate “none.”)

- Binding Contract: We understand that this Bid, together with your written acceptance thereof included in your Letter of Acceptance, shall constitute a binding contract between us, until a formal contract is prepared and executed.

Name of the Bidder:*[insert complete name of the Bidder]

Name of the person duly authorized to sign the Bid on behalf of the Bidder: ** [insert complete name of person duly authorized to sign the Bid]

Title of the person signing the Bid: [insert complete title of the person signing the Bid]

Signature of the person named above: [insert signature of person whose name and capacity are shown above]

Date signed [insert date of signing] day of [insert month], [insert year]

*: In the case of the Bid submitted by a Joint Venture specify the name of the Joint Venture as Bidder.

**: Person signing the Bid shall have the power of attorney given by the Bidder. The power of attorney shall be attached with the Bid Schedules.

Bidder’s Qualification

To establish its qualifications to perform the contract in accordance with Section V (Evaluation and Qualification Criteria) the Bidder shall provide the information requested in the corresponding Information Sheets/Forms included hereunder

The information provided in the Forms shall be substantiated with valid documentary evidence otherwise the requirement will not be considered as complied.

FORM ELI 1

Bidder Information Form

[The Bidder shall fill in this Form in accordance with the instructions indicated below. No alterations to its format shall be permitted and no substitutions shall be accepted.]

Date: [insert date (as day, month and year) of Bid submission]

IFB No.: [insert number of Bidding process]

Alternative No.: [insert identification No if this is a Bid for an alternative]

Page ________ of_ ______ pages

1. Bidder’s Name [insert Bidder’s legal name]

2. In case of JV, legal name of each member : [insert legal name of each member in JV]

3. Bidder’s actual or intended country of registration: [insert actual or intended country of registration]

4. Bidder’s year of registration: [insert Bidder’s year of registration]

5. Bidder’s Address in country of registration: [insert Bidder’s legal address in country of registration]

6. Bidder’s Authorized Representative Information Name: [insert Authorized Representative’s name] Address: [insert Authorized Representative’s Address] Telephone/Fax numbers: [insert Authorized Representative’s telephone/fax numbers] Email Address: [insert Authorized Representative’s email address]

7. Attached are copies of original documents of [check the box(es) of the attached original documents] Articles of Incorporation (or equivalent documents of constitution or association), and/or documents of registration of the legal entity named above. In case of JV, JV agreement, in accordance with ITB 3.3. Establishing that the Bidder is not under the supervision of the Employer[ in case of subsidiaries] 8. Included are the organizational chart, a list of Board of Directors, and the beneficial ownership.

ELI2

Bidder’s JV Members Information Form

[Not Applicable]

[The Bidder shall fill in this Form in accordance with the instructions indicated below. The following table shall be filled in for the Bidder and for each member of a Joint Venture].

Date: [insert date (as day, month and year) of Bid submission]

IFB No.: [insert number of IFB process]

Alternative No.: [insert identification No if this is a Bid for an alternative]

Page ________ of_ ______ pages

1. Bidder’s Name: [insert Bidder’s legal name]

2. Bidder’s JV Member’s name: [insert JV’s Member legal name]

3. Bidder’s JV Member’s country of registration: [insert JV’s Member country of registration]

4. Bidder’s JV Member’s year of registration: [insert JV’s Member year of registration]

5. Bidder’s JV Member’s legal address in country of registration: [insert JV’s Member legal address in country of registration]

6. Bidder’s JV Member’s authorized representative information Name: [insert name of JV’s Member authorized representative] Address: [insert address of JV’s Member authorized representative] Telephone/Fax numbers: [insert telephone/fax numbers of JV’s Member authorized representative] Email Address: [insert email address of JV’s Member authorized representative]

7. Attached are copies of original documents of [check the box(es) of the attached original documents] Articles of Incorporation (or equivalent documents of constitution or association), and/or registration documents of the legal entity named above. 8. Included are the organizational chart, a list of Board of Directors, and the beneficial ownership.

- Personnel

- Form PER -1

Contractor’s Representative and Key Personnel

Schedule

Bidders should provide the names and details of the suitably qualified Contractor’s Representative and Key Personnel to perform the Contract. The data on their experience should be supplied using the Form PER-2 below for each candidate.

Contractor’ Representative and Key Personnel

| 1. | Title of position: |  |
| --- | --- | --- |
|  | Name of candidate: |  |
|  | Duration of appointment: | [insert the whole period (start and end dates) for which this position will be engaged] |
|  | Time commitment: for this position: | [insert the number of days/week/months/ that has been scheduled for this position] |
|  | Expected time schedule for this position: | [insert the expected time schedule for this position (e.g. attach high level Gantt chart] |
| 2. | Title of position: |  |
|  | Name of candidate: |  |
|  | Duration of appointment: | [insert the whole period (start and end dates) for which this position will be engaged] |
|  | Time commitment: for this position: | [insert the number of days/week/months/ that has been scheduled for this position] |
|  | Expected time schedule for this position: | [insert the expected time schedule for this position (e.g. attach high level Gantt chart] |
| 3. | Title of position: |  |
|  | Name of candidate: |  |
|  | Duration of appointment: | [insert the whole period (start and end dates) for which this position will be engaged] |
|  | Time commitment: for this position: | [insert the number of days/week/months/ that has been scheduled for this position] |
|  | Expected time schedule for this position: | [insert the expected time schedule for this position (e.g. attach high level Gantt chart] |
| 4. | Title of position: |  |
|  | Name of candidate: |  |
|  | Duration of appointment: | [insert the whole period (start and end dates) for which this position will be engaged] |
|  | Time commitment: for this position: | [insert the number of days/week/months/ that has been scheduled for this position] |
|  | Expected time schedule for this position: | [insert the expected time schedule for this position (e.g. attach high level Gantt chart] |
| 5. | Title of position: |  |
|  | Name of candidate |  |
|  | Duration of appointment: | [insert the whole period (start and end dates) for which this position will be engaged] |
|  | Time commitment: for this position: | [insert the number of days/week/months/ that has been scheduled for this position] |
|  | Expected time schedule for this position: | [insert the expected time schedule for this position (e.g. attach high level Gantt chart] |
| 6. | Title of position: [insert title] |  |
|  | Name of candidate |  |
|  | Duration of appointment: | [insert the whole period (start and end dates) for which this position will be engaged] |
|  | Time commitment: for this position: | [insert the number of days/week/months/ that has been scheduled for this position] |
|  | Expected time schedule for this position: | [insert the expected time schedule for this position (e.g. attach high level Gantt chart] |

- Form PER-2:

Resume and Declaration

Contractor’s Representative and Key Personnel

Name of Bidder

| Position [#1]: [title of position from Form PER-1] |  |  |
| --- | --- | --- |
| Personnel information | Name: | Date of birth: |
|  | Address: | E-mail: |
|  | Professional qualifications: |  |
|  | Academic qualifications: |  |
|  | Language proficiency:[language and levels of speaking, reading and writing skills] |  |
| details |  |  |
|  | Address of Employer: |  |
|  | Telephone: | Contact (manager / personnel officer): |
|  | Fax: |  |
|  | Job title: | Years with present Employer: |

Summarize professional experience in reverse chronological order. Indicate particular technical and managerial experience relevant to the project.

| Project | Role | Duration of involvement | Relevant experience |
| --- | --- | --- | --- |
| [main project details] | [role and responsibilities on the project] | [time in role] | [describe the experience relevant to this position] |

Declaration

I, the undersigned [ insert either “Contractor’s Representative” or “Key Personnel” as applicable] , certify that to the best of my knowledge and belief, the information contained in this Form PER-2 correctly describes myself, my qualifications and my experience.

I confirm that I am available as certified in the following table and throughout the expected time schedule for this position as provided in the Bid:

| Commitment | Details |
| --- | --- |
| Commitment to duration of contract: | [insert period (start and end dates) for which this Contractor’s Representative or Key Personnel is available to work on this contract] |
| Time commitment: | [insert period (start and end dates) for which this Contractor’s Representative or Key Personnel is available to work on this contract] |

I understand that any misrepresentation or omission in this Form may:

- be taken into consideration during Bid evaluation;

- result in my disqualification from participating in the Bid;

- result in my dismissal from the contract.

Name of Contractor’s Representative or Key Personnel: [insert name]				Signature: __________________________________________________________

Date: (day month year): _______________________________________________

Countersignature of authorized representative of the Bidder:

Signature: ________________________________________________________

- Date: (day month year): ____________________________
Form CON – 2

- Historical Contract Non-Performance, Pending Litigation and Litigation History

Bidder’s Name: ________________
Date: ______________________
JV Member Name_________________________
IFB No. and title: ___________________________
Page _______________of ______________pages

| Non-Performed Contracts in accordance with Qualification Criteria |  |  |  |
| --- | --- | --- | --- |
| Contract non-performance did not occur January [insert year] Contract(s) not performed since [insert year] |  |  |  |
| Year | Non- performed portion of contract | Contract Identification | Total Contract Amount (current value, currency, exchange rate and PKR equivalent) |
|  |  | Contract Identification: Name of Employer(PA): Address of PA: Reason(s) for nonperformance: |  |
| Pending Litigation, in accordance with Qualification Criteria |  |  |  |
| No pending litigation |  |  |  |
| Pending litigation |  |  |  |

| Year of dispute | Amount in dispute (currency) | Contract Identification | Total Contract Amount |
| --- | --- | --- | --- |
|  |  | Contract Identification: _________ Name of PA: ____________ Address of PA: __________ Matter in dispute: ______________ Party who initiated the dispute: ____ Status of dispute: ___________ |  |
| Litigation History in accordance with Section V, Evaluation and Qualification Criteria of the Prequalification document |  |  |  |
|  No Litigation History  Litigation History |  |  |  |
| Year of award | Outcome as percentage of Net Worth | Contract Identification | Total Contract Amount (currency), PKR Equivalent (exchange rate) |
| [insert year] | [insert percentage] | Contract Identification: [indicate complete contract name, number, and any other identification] Name of PA: [insert full name] Address of PA: [insert street/city/country] Matter in dispute: [indicate main issues in dispute] Party who initiated the dispute: [indicate “Employer” or “Contractor”] Reason(s) for Litigation and award decision [indicate main reason(s)] | [insert amount] |

Financial Situation

- Form FIN – 3.1:

Financial Situation and Performance

Bidder’s Name: ________________
Date: ______________________
JV Member’s Name_________________________
IFB No. and title: ___________________________
Page _______________of ______________pages

1. Financial data

| Type of Financial information in (currency) | Historic information for previous _________years, ______________ (amount in currency, currency, exchange rate*, PKR equivalent) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Year 1 | Year 2 | Year 3 | Year4 | Year 5 |
| Statement of Financial Position (Information from Balance Sheet) |  |  |  |  |  |
| Total Assets (TA) |  |  |  |  |  |
| Total Liabilities (TL) |  |  |  |  |  |
| Total Equity/Net Worth (NW) |  |  |  |  |  |
| Current Assets (CA) |  |  |  |  |  |
| Current Liabilities (CL) |  |  |  |  |  |
| Working Capital (WC) |  |  |  |  |  |
| Information from Income Statement |  |  |  |  |  |
| Total Revenue (TR) |  |  |  |  |  |
| Profits Before Taxes (PBT) |  |  |  |  |  |
| Cash Flow Information |  |  |  |  |  |
| Cash Flow from Operating Activities |  |  |  |  |  |

*Exchange rate used for calculations should be two days prior to the bids submission.

2. Sources of Finance

Specify sources of finance to meet the cash flow requirements on works currently in progress and for future contract commitments.

| No. | Source of finance | Amount (PKR) |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

2. Financial documents

The Bidder and its parties shall provide copies of financial statements for ___________years pursuant to Evaluation and Qualifications Criteria, Sub-factor 3.1. The financial statements shall:

(a) 	reflect the financial situation of the Bidder or in case of JV member, and not an affiliated entity  (such as parent company or group member).

(b)	be independently audited or certified in accordance with local legislation.

(c)	be complete, including all notes to the financial statements.

(d)	correspond to accounting periods already completed and audited.

Attached are copies of financial statements  for the ____________years required above; and complying with the requirements

- Form FIN – 3.2:

Average Annual Construction Turnover

Bidder’s Name: ________________
Date: ______________________
JV Member’s Name_________________________
IFB No. and title: ___________________________
Page _______________of ______________pages

|  |  | Annual turnover data (construction only) |  |  |
| --- | --- | --- | --- | --- |
| Year | Amount Currency |  | Exchange rate ** | PKR equivalent |
| [indicate year] | [insert amount and indicate currency] |  |  |  |
| Average Annual Construction Turnover * |  |  |  |  |

* See Section V, Evaluation and Qualification Criteria.

* * Exchange rate used for calculations should be two days prior to the bids submission

- Form FIN – 3.3:

Financial Resources

Specify proposed sources of financing, such as liquid assets, unencumbered real assets, lines of credit, and other financial means, net of current commitments, available to meet the total construction cash flow demands of the subject contract or contracts as specified in Section – V (Evaluation and Qualification Criteria)

| Financial Resources |  |  |
| --- | --- | --- |
| No. | Source of financing | Amount |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

- Form FIN – 3.4:

Current Contract Commitments / Works in Progress

Bidders and each member to a JV should provide information on their current commitments on all contracts that have been awarded, or for which a letter of intent or acceptance has been received, or for contracts approaching completion, but for which an unqualified, full completion certificate has yet to be issued.

##### Current Contract Commitments

| No. | Name of Contract | Employer’s Contact Address, Tel, Fax | Value of Outstanding Work [PKR Equivalent] | Estimated Completion Date | Average Monthly Invoicing Over Last Six Months [PKR/month)] |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

Form FIN - 5:

Self-Assessment Tool for Bidder’s Compliance to Financial Resources (Criterion 2.1 of Section V)

This form requires the same information submitted in Forms FIN – 3.3 and FIN -3.4. All conditions of “Available Financial Resources Net of CCC ≥ Requirement for the Subject Contract” must be satisfied to qualify.

Form FIN - 5A: For Single Entities

| For Single Entities: (A) | Total Available Financial Resources from FIN – 3.3 (B) | Total Monthly Financial Requirement for Current Contract Commitments (CCC) from FIN – 3.4 (C) | Available Financial Resources Net of CCC D = (B - C) | Requirement for the Subject Contract (E) | Results: Yes or No [D must be greater than or equal to E] (F) |
| --- | --- | --- | --- | --- | --- |
| _________ (Name of Bidder) |  |  |  | . . . . . . . . . . ._ |  |

Form FIN - 5B: For Joint Ventures

| For Joint Ventures: (A) | Total Available Financial Resources from FIN – 3.3 (B) | Total Monthly Financial Requirement for Current Contract Commitments (CCC) from FIN – 3.4 (C) | Available Financial Resources Net of CCC D = (B - C) | Requirement for the Subject Contract (E) | Results: Yes or No [D must be greater than or equal to E] (F) |
| --- | --- | --- | --- | --- | --- |
| One Partner: |  |  |  |  |  |
| ____________ (Name of Partner) |  |  |  | . . . . . . . . . . . |  |
| Each Partner: |  |  |  |  |  |
| _____________ (Name of Partner 1) |  |  |  | . . . . . . . . . . . |  |
| _____________ (Name of Partner 2) |  |  |  | . . . . . . . . . . . |  |
| _____________ (Name of Partner 3) |  |  |  | . . . . . . . . . . . |  |
| All partners combined | ∑ D = Sum of available financial resources net of current contract commitments for all partners |  | ∑ D = _______ | . . . . . . . . . . . |  |

- Note -

Form FIN – 5 is made available for use by the bidder as a self-assessment tool, and by the employer as an evaluation work sheet, to determine compliance with the financial resources requirement as stated in 3.1 (ii). Failure to submit Form FIN - 5 by the Bidder shall not lead to bid rejection.

EXPERIENCE

- Form EXP - 4.1

- General Construction Experience

Bidder’s Name: ________________
Date: ______________________
JV Member’s Name_________________________
IFB No. and title: ___________________________
Page _______________of ______________pages

| Starting Year | Ending Year | Contract Identification | Role of Bidder |
| --- | --- | --- | --- |
|  |  | Contract name: ____________________ Brief Description of the Works performed by the Bidder: _____________________________ Amount of contract: ___________________ Name of PA: ____________________ Address: _____________________________ |  |
|  |  | Contract name: _________________________ Brief Description of the Works performed by the Bidder: _____________________________ Amount of contract: ___________________ Name of PA: ___________________ Address: _________________________ |  |
|  |  | Contract name: ________________________ Brief Description of the Works performed by the Bidder: __________________________ Amount of contract: ___________________ Name of PA: ___________________ Address: _________________________ |  |

- Form EXP - 4.2(a)

Specific Construction and Contract Management Experience

Bidder’s Name: ________________
Date: ______________________
JV Member’s Name_________________________
IFB No. and title: ___________________________
Page _______________of ______________pages

| Similar Contract No. | Information |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Contract Identification |  |  |  |  |  |
| Award date |  |  |  |  |  |
| Completion date |  |  |  |  |  |
| Role in Contract | Prime Contractor |  | Member in JV | Management Contractor | Sub-contractor |
| Total Contract Amount |  |  |  | PKR equivalent |  |
| If member in a JV or sub-contractor, specify participation in total Contract amount |  |  |  |  |  |
| PA’s Name: |  |  |  |  |  |
| Address: Telephone/fax number E-mail: |  |  |  |  |  |

Form EXP - 4.2(a) (cont.)

Specific Construction and Contract Management Experience (cont.)

| Similar Contract No. | Information |
| --- | --- |
| Description of the similarity in accordance with Sub-Factor 4.2(a) of Section V: |  |
| 1. Amount |  |
| 2. Physical size of required works items |  |
| 3. Complexity |  |
| 4. Methods/Technology |  |
| 5. Construction rate for key activities |  |
| 6. Other Characteristics |  |

- Form EXP - 4.2(b)

- Construction Experience in Key Activities

Bidder's Name: ________________
Date: ___________________
Bidder's JV Member Name: __________________
Sub-contractor's Name (as per ITB 34.2 and 34.3): ________________
IFB No. and title: _____________________

- Page __________________of ________________pages

All Sub-contractors for key activities must complete the information in this form as per ITB 38 and Qualification Criteria and Requirements, Sub-Factor 4.2.

1.	Key Activity No One: ________________________

|  | Information |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Contract Identification |  |  |  |  |  |
| Award date |  |  |  |  |  |
| Completion date |  |  |  |  |  |
| Role in Contract | Prime Contractor | Member in JV |  | Management Contractor | Sub-contractor |
| Total Contract Amount |  |  |  | PKR equivalent |  |
| Quantity (Volume, number or rate of production, as applicable) performed under the contract per year or part of the year | Total quantity in the contract (i) |  | Percentage participation (ii) |  | Actual Quantity Performed (i) x (ii) |
| Year 1 |  |  |  |  |  |
| Year 2 |  |  |  |  |  |
| Year 3 |  |  |  |  |  |
| Year 4 |  |  |  |  |  |
| PA’s Name: |  |  |  |  |  |  |
| Address: Telephone/fax number E-mail: |  |  |  |  |  |  |

2. Activity No. Two

3. …………………

|  | Information |
| --- | --- |
| Description of the key activities in accordance with Section V: |  |

Form of Bid Security

(Bank Guarantee)

[The bank shall fill in this Bank Guarantee Form in accordance with the instructions indicated.]

[Guarantor letterhead or SWIFT identifier code]

Beneficiary: [Purchaser to insert its name and address]

No.: [Purchaser to insert reference number for the Request for Bids]

Alternative No.: [Insert identification No if this is a Bid for an alternative]

Date: [Insert date of issue]

BID GUARANTEE No.: [Insert guarantee reference number]

Guarantor: [Insert name and address of place of issue, unless indicated in the letterhead]

We have been informed that ______ [insert name of the Bidder, which in the case of a joint venture shall be the name of the joint venture (whether legally constituted or prospective) or the names of all members thereof] (hereinafter called "the Applicant") has submitted or will submit to the Beneficiary its Bid (hereinafter called "the Bid") for the execution of ________________ under Invitation for Bids No. __________ _ (“the IFB”).

Furthermore, we understand that, according to the Beneficiary’s conditions, Bids must be supported by a Bid guarantee.

At the request of the Applicant, we, as Guarantor, hereby irrevocably undertake to pay the Beneficiary any sum or sums not exceeding in total an amount of ___________ (____________) upon receipt by us of the Beneficiary’s complying demand, supported by the Beneficiary’s statement, whether in the demand itself or a separate signed document accompanying or identifying the demand, stating that either the Applicant:

(a) 	has withdrawn its Bid during the period of Bid validity set forth in the Applicant’s Letter of Bid (“the Bid Validity Period”), or any extension thereto provided by the Applicant; or

(b) 	having been notified of the acceptance of its Bid by the Beneficiary during the Bid Validity Period or any extension thereto provided by the Applicant, (i) has failed to sign the contract agreement, or (ii) has failed to furnish the performance security, in accordance with the Instructions to Bidders (“ITB”) of the Beneficiary’s bidding document.

This guarantee will expire: (a) if the Applicant is the successful Bidder, upon our receipt of copies of the Contract agreement signed by the Applicant and the performance security issued to the Beneficiary in relation to such Contract agreement; or (b) if the Applicant is not the successful Bidder, upon the earlier of (i) our receipt of a copy of the Beneficiary’s notification to the Applicant of the results of the Bidding process; or (ii) twenty-eight days after the end of the Bid Validity Period.

Consequently, any demand for payment under this guarantee must be received by us at the office indicated above on or before that date.

_____________________________

[Signature(s)]

Note: All italicized text is for use in preparing this form and shall be deleted from the final product.

Priced Bill of Quantities

[to be printed on letterhead of the bidder]

We confirm that the rates and prices bid in the priced Bill of Quantities, except insofar as it is otherwise provided under the Contract, include all Constructional Plant, labour, supervision, materials, erection, maintenance, insurance, profit, taxes (including GST/PRA), and duties, together with all general risks, liabilities, and obligations set out or implied in the Contract.

A rate or price is entered against each item in the priced Bill of Quantities, whether quantities are stated or not.  The cost of Items against which we have failed to enter a rate, or price shall be deemed to be covered by other rates and prices entered in the Bill of Quantities. Moreover, we confirm, warrant and agree that the Employer shall be at liberty, if we have missed filling the cost/rate against any number of item or component in the bill of quantities, to adjust the overall cost of the bid based on the average price of the item or component as quoted in other substantially responsive Bids. If the price of the item or component cannot be derived from the price of other substantially responsive Bids, the Employer shall use its best estimate.

SUMMARY OF COST

| Bill # | Description | Cost (Pak. Rs.) |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
|  | Sub-Total = |  |
|  | Cost of GST/PRA = |  |
|  | Grand Total = |  |

SUMMARY OF COST

| Bill # | Description | Cost (Pak. Rs.) |
| --- | --- | --- |
| 1.1 |  |  |
| 1.2 |  |  |
| 1.3 |  |  |
| 1.4 |  |  |
| 1.5 |  |  |
|  | Total = |  |

Bill No. 1.1 Title: xx

| Item. # | Ref. | Description | Unit of Measurement | Quantity | Unit Rate (Pak. Rs.) | Total Amount (Pak. Rs.) |
| --- | --- | --- | --- | --- | --- | --- |
| Total = |  |  |  |  |  |  |

Bill No. 1.2 Title: xx

| Item. # | Ref. | Description | Unit of Measurement | Quantity | Unit Rate (Pak. Rs.) | Total Amount (Pak. Rs.) |
| --- | --- | --- | --- | --- | --- | --- |
| Total = |  |  |  |  |  |  |

Bill No. 1.3 Title: xx

| Item. # | Ref. | Description | Unit of Measurement | Quantity | Unit Rate (Pak. Rs.) | Total Amount (Pak. Rs.) |
| --- | --- | --- | --- | --- | --- | --- |
| Total = |  |  |  |  |  |  |

Schedule of Adjustment Data

(Not Applicable)

| Element Number | Element Description | Bidder’s Amount [PKR] | Bidder’s Proposed Weighting for Coefficients* |
| --- | --- | --- | --- |

| i. ii. iii. iv. v. vi. vii viii ix | Non-adjustable Cement (OPC) Steel (M.S. bars [deformed, 40/60 grade] High Speed Diesel Labour (Un-skilled/skilled) HDPE/PVC Pipe RCC Pipe Bricks Crush |  | a: (0.2) b: ( ) c: ( ) d: ( ) e: ( ) f: ( ) g: ( ) h: ( ) i: ( ) |
| --- | --- | --- | --- |
| Total |  |  | 1.00 |

* The Bidder shall enter a value within the range such that the total weighting shall be equal to 1.00.

The base and current rates of each cost element shall be taken from the Federal Bureau of Statistics (FBS), Statistical Division Government of Pakistan, Statutory notifications and official price from public sector organizations, where available, for Price Variation Adjustment for the area(s) prescribed in the list, nearest to the location of the Works. Whereas the source for prices of High Speed Diesel (HSD) shall be either Statistical Bulletins or Pakistan State Oil (PSO) or Oil and Gas Regulatory Authority (OGRA). The base values shall be 28 days prior to the submission of Bid.

Note:

- Cement (OPC) [Bags] shall represent all kinds of Cement.

- Steel (M.S. bars deformed, 40/60 grade) shall represent all kinds of steel reinforcing bars and structural steel and all kind of MS angle/sheet and sections.

- High Speed Diesel [Litre] shall represent all kinds of fuels and lubricants.

- Labour (Un-skilled/skilled) shall represent all kinds of labour.

- HDPE/PVC pipe shall represent all kind of pressure (PN)/class and size of pipes

- RCC pipes shall represent all kinds/class and size of pipes

- Bricks shall represent all kind of bricks, bricks ballast and bricks tiles

- Crush shall represent all size and grade of crush.

Form of Affidavit

[This must be printed on judicial/stamp paper having worth of at least Pak. Rs. 100/-. The judicial/stamp paper must be a registered paper with the respective vendor, having necessary entries on it and must also be Notarized. This Affidavit must be signed by the authorized representative of the bidder whose respective authorization is already confirmed as part of the Bid]

Affidavit of:	 [name of authorized representative of the bidder], having CNIC # [enter number], as [mention the designation of authorized representative] on behalf of [name of the bidder], [complete official address]

I, the above-named deponent, do hereby solemnly state on oath that I am an authorized representative of [name of the bidder] and, hereby on behalf of [name of the bidder] declare, agree, understand, warrant and undertake that:

- [name of the bidder] intends to submit a Bid for contraction works & related services, viz., [brief description of works and services] in complete understanding of the bidding document for the said procurement, issued by the Allama Iqbal Open University,

- [name of the bidder] has not been blacklisted or debarred by any federal or provincial or regional Public Procurement Regulatory Authority and/or government ministry/ department/public sector organization, international financial institutions/donors [such as World Bank, Asian Development Bank etc.],

- [name of the bidder] is not engaged in any legal proceedings with and/or against any federal or provincial or regional Public Procurement Regulatory Authority and/or government ministry/department/public sector organization including Allama Iqbal Open University, international financial institutions/donors [such as World Bank, Asian Development Bank etc.]

- [name of the bidder] has no direct and/or indirect conflict of interest with any person, employee, staff and any member of the Procurement Evaluation Committee(s) of the Allama Iqbal Open University and/or the procurement activity for which [name of the bidder] is submitting bid,

- [name of the bidder] is submitting his bid conscious of market inflation and foreign currency exchange rates risks and shall comply with the offered rates during bid validity period, and if awarded a contract shall honour the offered rates without indulging into any argument and/or asking for price adjustment, thus bearing all the related risks,

- [name of the bidder] irrevocably and unconditionally undertake and warrant that in case [name of the bidder] defaults in honouring his offered/committed rates before or after, signing of contract agreement and/or supply order, the University shall be at liberty to forfeit the earnest money and/or  performance security and/or take any punitive action including blacklisting for desired period, and [name of the bidder] shall not challenge that decision in any court of law,

- [name of the bidder] accepts full responsibility and strict liability for making any false declaration, not making full disclosure, misrepresenting facts or taking any action likely to defeat the purpose of this declaration, representation and warranty.

DEPONENT

Verification:

Verified on oath at Islamabad on this -- day of the month of -------------- of 20-- that the contents of the foregoing Affidavit are true and correct to the best of my knowledge and belief and that nothing thereof has been concealed from Allama Iqbal Open University.

DEPONENT

INSTRUCTIONS TO BIDDERS: DELETE THIS BOX ONCE YOU HAVE COMPLETED THE FORM

This Beneficial Ownership Disclosure Form (“Form”) is to be completed by the successful Bidder.  In case of joint venture, the Bidder must submit a separate Form for each member.  The beneficial ownership information to be submitted in this Form shall be current as of the date of its submission.

For the purposes of this Form, a Beneficial Owner of a Bidder is any natural person who ultimately owns or controls the Bidder by meeting one or more of the following conditions:

- directly or indirectly holding 25% or more of the shares

- directly or indirectly holding 25% or more of the voting rights

- directly or indirectly having the right to appoint a majority of the board of directors or equivalent governing body of the Bidder

Beneficial Ownership Disclosure Form

IFB No.: [insert number of IFB process]

Invitation for Bid Title: [insert identification]

To: [insert complete name of Employer]

In response to your request in the Letter of Acceptance dated [insert date of letter of Acceptance] to furnish additional information on beneficial ownership: [select one option as applicable and delete the options that are not applicable]

(i) we hereby provide the following beneficial ownership information.

Details of beneficial ownership

| Identity of Beneficial Owner | Directly or indirectly holding 25% or more of the shares (Yes / No) | Directly or indirectly holding 25 % or more of the Voting Rights (Yes / No) | Directly or indirectly having the right to appoint a majority of the board of the directors or an equivalent governing body of the Bidder (Yes / No) |
| --- | --- | --- | --- |
| [include full name (last, middle, first), nationality, country of residence] |  |  |  |

OR

(ii) We declare that there is no Beneficial Owner meeting one or more of the following conditions:

- directly or indirectly holding 25% or more of the shares

- directly or indirectly holding 25% or more of the voting rights

- directly or indirectly having the right to appoint a majority of the board of directors or equivalent governing body of the Bidder

OR

(iii) We declare that we are unable to identify any Beneficial Owner meeting one or more of the following conditions. [If this option is selected, the Bidder shall provide explanation on why it is unable to identify any Beneficial Owner]

- directly or indirectly holding 25% or more of the shares

- directly or indirectly holding 25% or more of the voting rights

- directly or indirectly having the right to appoint a majority of the board of directors or equivalent governing body of the Bidder]”

Name of the Bidder:*[insert complete name of the Bidder]_________

Name of the person duly authorized to sign the Bid on behalf of the Bidder:**[insert complete name of person duly authorized to sign the Bid]___________

Title of the person signing the Bid: [insert complete title of the person signing the Bid]______

Signature of the person named above: [insert signature of person whose name and capacity are shown above]_____

Date signed [insert date of signing] day of [insert month], [insert year]____

* In the case of the Bid submitted by a Joint Venture specify the name of the Joint Venture as Bidder. In the event that the Bidder is a joint venture, each reference to “Bidder” in the Beneficial Ownership Disclosure Form (including this Introduction thereto) shall be read to refer to the joint venture member.

** Person signing the Bid shall have the power of attorney given by the Bidder. The power of attorney shall be attached with the Bid Schedules.

If applicable