SECTION VI

WORKS REQUIREMENT, TECHNICAL SPECIFICATIONS, DRAWINGS/LAYOUTS, SUPPLEMENTARY INFORMATION AND BILL OF QUANTITIES

# Section IV. Works Requirement, Technical Provisions, Drawings/layouts, Supplementary Information & Bill of Quantities

## Scope of Works

The plot area is shown in the Drawings section. The foundation works and the structural frame up to the Ground Floor slab level have already been completed. However, the First Floor construction has not yet commenced. The site has remained abandoned for over two (2) years; therefore, certain masonry wall defects, deterioration, or remedial works may be anticipated.

The remaining scope of work includes:

        - Completion of Ground Floor grey structure (wherever incomplete)

        - Construction of First Floor on the existing Ground Floor slab, including complete grey structure and all finishing works, in all respects

        - Construction of the complete boundary wall around the plot with main gate

        - Execution of all external development works within the plot premises

In addition, all electrical, plumbing, sewerage, and networking works shall be executed.

All works shall be carried out in accordance with the drawings, technical specifications, and relevant standards.

## Technical Specifications

Technical Specifications are attached in Annex A at the end of this document.

## Drawings

Drawings are attached in Annex B at the end of this document.

# Supplementary Information

# Site Pictures are attached for reference:

Bill of Quantities

SUMMARY

| Bill # | Description |
| --- | --- |
| 1 | Main Office Building |
| 2 | RD & Staff Residence |

Bill No. 1 Title: Main Office Building

| Bill # | Description |
| --- | --- |
| 1.1 | Main Building Civil Works |
| 1.2 | External Services |
| 1.3 | Electricity |
| 1.4 | Sui Gas |

Bill No. 2 Title: RD & Staff Residence

| Bill # | Description |
| --- | --- |
| 2.1 | Main Building Civil Works |
| 2.2 | External Services |
| 2.3 | Electricity |
| 2.4 | Sui Gas |

Bill No. 1.1 Title: Main Building Civil Works

| Item. # | Description |
| --- | --- |
| 1.1.1 | Demolishing & Earthwork |
| 1.1.2 | Plain And Reinforced Concrete |
| 1.1.3 | Reinforcement Work |
| 1.1.4 | Block Masonry Work |
| 1.1.5 | Surface Rendering |
| 1.1.6 | Roofing & Waterproofing |
| 1.1.7 | Painting & Varnishing |
| 1.1.8 | Flooring |
| 1.1.9 | Carpentry And Joinery |
| 1.1.10 | Miscellaneous |
| 1.1.11 | Water Supply, Sanitary, Plumbing Fixtures & Sewerage |

Bill No. 1.2 Title: External Services

| Item. # | Description |
| --- | --- |
| 1.2.1 | Footpath, Driveway, Lawn |
| 1.2.2 | Boundary Wall |
| 1.2.3 | Guard Room |
| 1.2.4 | Parking Shed |
| 1.2.5 | External Water Supply and Sewerage |

Bill No. 1.3 Title: Electricity

| Item. # | Description |
| --- | --- |
| 1.3.1 | Main Building |
| 1.3.2 | External Electrification |
| 1.3.3 | Fire Alarm System |
| 1.3.4 | CCTV System |
| 1.3.5 | Telephone System |

Bill No. 1.4 Title: Sui Gas

| Item. # | Description |
| --- | --- |
| 1.4.1 | Sui Gas Internal |
| 1.4.2 | External Services |

Bill No. 2.1 Title: Main Building Civil Works

| Item. # | Description |
| --- | --- |
| 2.1.1 | Demolishing & Earthwork |
| 2.1.2 | Plain And Reinforced Concrete |
| 2.1.3 | Reinforcement Work |
| 2.1.4 | Block Masonry Work |
| 2.1.5 | Surface Rendering |
| 2.1.6 | Roofing & Waterproofing |
| 2.1.7 | Painting & Varnishing |
| 2.1.8 | Flooring |
| 2.1.9 | Carpentry And Joinery |
| 2.1.10 | Miscellaneous |
| 2.1.11 | Water Supply, Sanitary, Plumbing Fixtures & Sewerage |

Bill No. 2.2 Title: Electricity

| Item. # | Description |
| --- | --- |
| 2.2.1 | Wiring |
| 2.2.2 | Accessories (Clipsal & Switchkids Make) |
| 2.2.3 | Light Fittings & Fixtures (Philips, Sunlight, & Paklite Make) |
| 2.2.4 | Earthing & Lightning Protection System |
| 2.2.5 | DB Work |
| 2.2.6 | Fire Alarm System |

Bill No. 1.1.1 Title: Demolishing & Earthwork

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Dismantling of block masonry in cement mortar (including, stacking salvaged material serviceable) and disposal of surplus material complete. | 750 | 100 Cft |
|  | Dismantling cement concrete work i/c disposing of the material complete. | 530 | 100 Cft |
|  | Excavation for raft foundations, underground tanks and septic tanks (rectangular or square) in all kinds of soil (except gravelly and murum soil wet silt, clay or mud, conglomeration of gravel and boulders, soft, sandy or disintegrated and hard rock) and back filling the excavated material all round the trenches including breaking clods, watering, consolidation by ramming in layers not exceeding 9 inches (229 mm) in depth to full compaction, dressing and disposal of surplus excavated stuff as directed, lead up to one chain (30.5 R.m) and lift upto 5 feet (1.52 m). | 150 | 100 Cft |
|  | Supplying earth from approved outside sources within a radius of 5 miles (8 km) including digging, loading and unloading and filling in foundation trenches, plinth or under floor, etc. including breaking clods, dressing, watering and consolidation by ramming in layers not exceeding 9 inches (229 mm) in depth to full compaction complete within a ;lead of one chain (30.5 R.m) and lift of 5 feet (1.52 m). | 1436 | 100 Cft |

Bill No. 1.1.2 Title: Plain and Reinforced Concrete Work

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Providing and laying 1:4:8 (1 cement 4 sand and 8 coarse aggregate) cement concrete using crushed graded boulders (machine broken) in foundation including levelling, compacting and curing etc. complete. | 550 | 100 Cft |
|  | Providing and laying in situ 1:3:6 (1 cement 3 sand and 6 coarse aggregate) cement concrete using crushed graded boulders 3/4 inch (19 mm) and down gauge in foundation, basement and plinth including form work, compacting, curing and removal of form work etc. complete, foundation and basement up to 5 feet (1.52 m) depth and plinth up to 4 feet (1.2 m) height from ground level. | 1317 | 100 Cft |
|  | Providing and laying reinforced cement concrete using screened graded bajri 3/4 inch (19 mm) and down gauge having a minimum works cube crushing strength of 4500 Ibs. per sq. inch (31.04 N/mm2 at 28 days) with a mix not leaner than 1:1:2 including form work and its removal, compacting and curing etc. complete but excluding the cost of reinforcement, in foundation basement and plinth |  |  |
|  | a) Footing | 50 | 100 Cft |
|  | b) U.G.W.T |  |  |
|  | U.G.W.T Walls | 46 | 100 Cft |
|  | U.G.W.T Slab | 10 | 100 Cft |
|  | c) Plinth Beam | 610 | 100 Cft |
|  | d) Column |  |  |
|  | Short Column | 12 | 100 Cft |
|  | Ground floor |  |  |
|  | i) Column | 144 | 100 Cft |
|  | ii) Column (Decorated Column) | 192 | 100 Cft |
|  | First floor |  |  |
|  | i) Column | 711 | 100 Cft |
|  | ii) Column (Decorated Column) | 192 | 100 Cft |
|  | e) Beam |  |  |
|  | Ground floor | 4 | 100 Cft |
|  | First floor |  |  |
|  | i) Straight Beam | 653 | 100 Cft |
|  | ii) Inclined Beam | 1560 | 100 Cft |
|  | f) Lintel |  |  |
|  | Ground floor | 65 | 100 Cft |
|  | First floor | 69 | 100 Cft |
|  | Ground floor (Arch) | 19 | 100 Cft |
|  | First floor (Arch) | 28 | 100 Cft |
|  | g) Slab |  |  |
|  | Ground floor | 100 | 100 Cft |
|  | First floor |  |  |
|  | i) Ordinary Slab | 1518 | 100 Cft |
|  | ii) Ordinary Cantilevered Slab | 416 | 100 Cft |
|  | iii) Sloping Slab | 89 | 100 Cft |
|  | h) Sun Shade Or Chajja |  |  |
|  | Ground floor | 73 | 100 Cft |
|  | i) Stair |  |  |
|  | Ground floor | 44 | 100 Cft |
|  | First floor | 131 | 100 Cft |
|  | j) O.H.W.T. |  |  |
|  | i) Wall | 181 | 100 Cft |
|  | ii) Bottom Slab | 105 | 100 Cft |
|  | ii) Top Slab | 66 | 100 Cft |
|  | Extra for using sulphate resisting cement in place of ordinary Portland cement. (Up to Plinth Level) | 1328 | Cwt |
|  | Providing and laying reinforced cement concrete using screened graded bajri 3/4 inch (19 mm) and down gauge having a minimum works cube crushing strength of 2250 Ibs. per sq. inch (15.52 N/mm2) at 28 days with a mix not leaner than 1:2:4 in fencing post braces and signposts of required shape including form works and its removal, compacting and curing etc. complete but excluding the cost of reinforcement (For Rome Post) |  |  |
|  | a) First Floor | 41 | 100 Cft |
|  | Providing and laying reinforced cement concrete using screened graded bajri 3/4 inch (19 mm) and down gauge having a minimum works cube crushing strength of 2250 Ibs. per sq inch (15.52 N/mm2) at 28 days with a mix not leaner than 1:2:4 in precast spiral stairs case with railing 3 feet (0.9 m) dia with central post, nosing, rendering with cement slurry on all exposed surface including form work and its removal, compacting and curing etc. complete but excluding the cost of reinforcement, in basement plinth and ground floor. | 14 | 100 Cft |
|  | Providing and laying in situ 1:6 (1 cement 6 all in aggregate) cement concrete using screened graded bajri 3/4 inch (19 mm) and down gauge in foundation, basement and plinth including form work, compacting, curing and removal of form work etc. complete, any floor. | 211 | 100 Cft |

Bill No. 1.1.3 Title: Reinforcement Work

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Providing and laying hard grade ribbed deformed (minimum yield point 60,000 psi) reinforcement bars with & including the cost of straightening, cutting, bending, binding, wastage, and such overlaps as are not shown in the drawings, placing in position on cement concrete 1:2:4 precast or MS chairs, tying with binding wire, cost of chairs and wires etc. in all kinds of RCC work in foundation, basement, plinth and ground floor of building including septic tanks and underground tanks and in projections for future extension . |  |  |
|  | a) Ground Floor | 12054 | kg |
|  | b) First Floor | 27901 | kg |

Bill No. 1.1.4	 Title: Block Masonry Work

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Providing and laying 1:3:6 cement concrete solid block masonry 4 to 6 inches (102 mm to 152 mm) thick using graded screened bajri 3/4 inch (19 mm) and down gauge set in cement mortar 1:4 including scaffolding, raking out joints and curing etc. complete in ground floor superstructure. |  |  |
|  | a) Ground Floor | 865 | 100 Cft |
|  | b) First Floor | 1571 | 100 Cft |
|  | Providing and laying 1:3:6 cement concrete hollow block masonry of any thickness using graded screened bajri 1/2 inch (13 mm) and down gauge set in cement mortar 1:6 including scaffolding, raking, out Joints and curing etc. complete in basement and ground floor superstructure. |  |  |
|  | a) Ground Floor | 1581 | 100 Cft |
|  | b) First Floor | 1665 | 100 Cft |

Bill No. 1.1.5	 Title: Surface Rendering

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | 1/2" (13 mm) thick cement plaster 1:5 on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete. (For Internal Surface) |  |  |
|  | a) Ground Floor | 7815 | 100 Sft |
|  | b) First Floor | 8795 | 100 Sft |
|  | c) O.H.W.T. | 187 | 100 Sft |
|  | 1/2" (13 mm) thick cement plaster 1:3 in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete (For Ceiling) |  |  |
|  | a) Ground Floor | 3561 | 100 Sft |
|  | b) First Floor | 5885 | 100 Sft |
|  | Providing 3/4" (19 mm) thick ornamental plaster with 1:3 cement mortar on walls, columns and slabs finished smooth in basement, plinth mezzanine and ground floor including curing etc., complete . |  |  |
|  | a) Ground Floor | 689 | 100 Sft |
|  | b) First Floor | 644 | 100 Sft |
|  | 3/4'' (19 mm) thick cement plaster 1:4 on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete. (For External Surface) |  |  |
|  | a) Ground Floor | 2713 | 100 Sft |
|  | b) First Floor | 3179 | 100 Sft |
|  | c) O.H.W.T. | 267 | 100 Sft |
|  | Providing and applying 3/4" (19 mm) thick rough cast cement rendering consisting of 3/8" or 9.6 mm topping of 1:2 (one cement and two mixed graded bajri 3/8" or 9.5 mm and down gauge) and base of 3/8" or 9.5 mm thick rough plaster 1:4 (one cement four sand) in plinth and ground floor including finishing etc. complete .(Below Stone Facing) |  |  |
|  | a) Ground Floor | 689 | 100 Sft |
|  | b) First Floor | 644 | 100 Sft |
|  | 1-1/2" (38 mm) thick cement plaster 1:4 in two layers on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete . |  |  |
|  | a) Ground Floor | 737 | 100 Sft |
|  | b) First Floor | 682 | 100 Sft |
|  | Extra if deep cut grooves of approved sizes and design are made on plastered surface of any description in any floor | 88 | 100 Rft |

Bill No. 1.1.6 Title:	Roofing & Waterproofing

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Providing and laying 2 inches (51 mm) thick damp proof course with cement concrete 1:2:4 cast in situ using graded screened bajri of 3/4 inch (19 mm) and down gauge including compacting, curing, form work and its removal etc. complete with applying a coat of hot bitumen (maxphalt 80/100 or equivalent) using 35 Lbs. per hundred square feet (1.71 Kg/sm) on damp proof course after complete drying and cleaning the surface. | 3036 | 100 Sft |
|  | Providing damp proof course on vertical surface with 3/4 inch (19 mm) thick cement plaster 1:4 mixed with any approved water proofing agent including curing etc. complete, but excluding the cost of water proofing agent.(For O.H.W.T & U.G.W.T Internal Side) | 513 | 100 Sft |
|  | Providing a coat of bitumen emulsion at 10 Lbs. per % sft. (0.49 Kg/sm) on walls and floor in ground floor.(For Bath Floor) | 297 | 100 Sft |
|  | Providing and laying 1:2:4 cement concrete khuras 2" (51 mm) thick 2 ft. x 2 ft. (610 mm x 610 mm) size on roof including finishing with cement plaster 1:4, curing etc. complete. | 4 | Each |

Bill No. 1.1.7	 Title: Painting & Varnishing

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Painting three coats with weather shield paint deluxe (ICI) make of approved shade on plaster surface (External) and including the cost of cleaning the surface, sand papering etc. complete at any height in any floor. | 7311 | 100 Sft |
|  | Providing and Applying one Coat of Deluxe/Robbilac Primeon/Alkali Resisting Masonry Sealer and minimum three coats of Vinyl emulsion including surface preparaion,filling and rubbing etc,complete in all respect upto any height at any floor. | 24532 | 100 Sft |
|  | Applying French or spirit polishing, two coat of approved make on wood work at any height in any floor . | 1041 | 100 Sft |
|  | Provide and apply one coat of Deluxe/Robbialac Primeon/Alkali Resisting Masonry Sealer and minimum 3 coats of Deluxe Special Plastic Emulsion or Robbialac VIP including surface preparation filling and rubbing etc., as per Specs. Clause 11.4, complete in all respects, upto any height at any floor. | 1524 | 100 SFT |
|  | Provide and apply enamel painting with ready mix paint of Deluxe/Robbialac make, 3 coats over and i/c the coat of priming/filling/finishing, complete at any height in any floor as per specifications, on.- |  |  |
|  | i): Wood work. | 144 | 100 SFT |
|  | ii): Steel work i/c grills and plastered surfaces | 1765 | 100 SFT |

Bill No. 1.1.8	 Title: Flooring

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Supplying, stacking, spreading and filling stones of required size to required slope from approved quarries in plinth or under floor including filling voids with spawl and chips and consolidation by ramming layers not exceeding 9 inches (229 mm) in depth to full compaction lift upto 5 feet (1.52 m). | 1617 | 100 Cft |
|  | Providing and laying 1:9 (1 cement 9 screened bajri) cement concrete using 3/4 inch (19 mm) and down gauge screened bajri in foundation including levelling, compacting and curing etc. complete . | 550 | 100 Cft |
|  | Providing and laying floors of 2 inches (51 mm) thick 1:2:4 cement concrete using graded screened bajri 3/4 inch (19 mm) and down gauge in ground floor laid in panels including form work, consolidation, finishing, and curing etc. complete.(For Under Floor) |  |  |
|  | a) Ground Floor | 3561 | 100 Sft |
|  | b) First Floor | 4226 | 100 Sft |
|  | Ditto item 117 above, but Extra for stairs steps (in treads, risers and mid-stair landings).. |  |  |
|  | a) Ground Floor | 163 | 100 Sft |
|  | b) First Floor | 163 | 100 Sft |
|  | Provide, lay, joint, cure, etc. 1” total thick glazed (glossy) ceramic tiles dado using (Master/Shabbir/Stile) (Supreme Quality) tiles of approved colour and shade in toilets, including ¼” thick neat cement buttering on back of tiles, remaining thickness made up with 1:2 cement sand rough base, complete Size: 1.5 sft to 3 sft. |  |  |
|  | a) L.G. Floor / Ground Floor | 495 | 100 SFT |
|  | b) First Floor | 367 | 100 SFT |
|  | P/F flooring comprising 12” x 12” (Master/Shabbir/Stile) supreme quality non glazed skid free tiles of approved shade set in neat cement paste not exceeding ¼” thick and filling the joint with neat while cement slurry using matching colouring i/c ¾” thick base in 1:3 cement sand mortar laid over a bed of 1 ½” thick P.C.C 1:2:4 i/c cleaning & curing etc, complete. |  |  |
|  | a) L.G. Floor / Ground Floor | 2061 | 100 SFT |
|  | b) First Floor | 1170 | 100 SFT |
|  | Provide, lay, level, repair, cure, etc., total 7/8" thick terrazzo dado on walls and on floor in Over Head water tank with 1:2 (one part cement and two part of white Marble chips) over ½" thick cement sand (1:3) rough plaster base, including curing etc., complete, terrazzo to be 3/8" thick but without grinding polishing. | 513 | 100 SFT |
|  | Providing and Laying Flooring (Glazed) Comprising 16"x 16" 3/8" (1600 cm) available thickness Granitto tiles (Mater) fixed in neat cement buttering laid over 1-1/4" thick PCC 1:2:4 using 3/8" down course aggregates to final levels and stop including filling joints with ready made approved fillers of suitable colors complete any floor at any height as directed. | 5334 | 100 SFT |
|  | Providing and laying Khaprail tiles C.C 1:2:4: in different design laid on sloppy slab & shades laid in cement mortar 1:2 in Joints and using etc. as per directions of the Engineer incharge. | 2522 | Sq.ft. |
|  | Providing and laying 1/2" (13mm) thick marble patties (pannel strips) up to 1-1/2" (38 mm) depth in floor and dado of any description in all floor. | 234 | Rft. |

Bill No. 1.1.9 Title:	Carpentry & Joinery

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Providing and fixing best quality deodar frames for doors. windows, ventilators, clerestory windows, shelves, partitions, trellis work, etc., as required . | 30 | Cft. |
|  | Providing and fixing 1-1/2 inch (38 mm) thick best quality deodar wood shutters fully panelled with same wood, approved iron hinges and tower bolts, etc., as required.. | 40 | Sq.ft. |
|  | Providing and fixing 1-1/2 inches (38 mm) thick pressed veneered door shutters fully flushed with commercial ply wood veneering on all faces and sides fixed over deodar wood cavities core and frame work of not less than 4 inches (102 mm) wide strip around with approved brass hinges and tower bolts etc., as required . | 513 | Sq.ft. |
|  | Providing and fixing on frames etc., with sunken iron screws best quality deodar wood moulding, beading, architrave etc., of approved design over one square inch (6.45 Sqm.) and up-to 2 square inches (13 Sqm.) in cross section as required . | 356 | Rft. |
|  | Providing and fixing with brass screws door lock handle type of approved design (Hungry make), cutting wood etc., to required shape and size with two operating keys as per direction of the Engineer-in-Charge. | 18 | Each |
|  | Providing and fixing with brass screws or specially supplied screws hydraulic door closer branno of approved design No. 3 (Yugoslavia make), cutting wood etc., to required shape and size as per direction of Engineer-in-Charge. | 18 | Each |
|  | Providing and fixing double glazed Champagne anodized aluminum Sliding windows as per British standard manufactured by Luckh, Alcop, Krudson, Pakistan Cables and A.C.P. (fixing through their approved fabricators), Executive model section dubble or single glazed 101mm x 37mm and 2mm thick including the cost of aluminum netting ,fitting, with all accessories cutting hole etc. and making good damages to walls etc. complete as required in any floor as per direction of engineer-in-charge, including the cost of Tinted Glass 5mm thick (Pakistani). (For Windows & Ventilator) | 1040 | Sq.ft. |
|  | Providing and fixing 1-1/2 inches or 38 mm thick (3/4 inch or 19 mm thick battens 3/4 inch or 19 mm thick ledges and baacing) best quality deodar wood shutters battened, ledged and braced, making rebated or tongued and grooved joints with approved T-iron hinges and tower bolts as required | 208 | Sq.ft. |
|  | Providing and-fixing 1-1/2 inches (38 mm) thick best quality deodar wood planking in shelves or almirams with or without deodar wood battens 1" x 1-1/2" (25 mm x 38 mm) fixed in walls with screws and rawal plug etc., as required . | 60 | Sq.ft. |
|  | Providing and fixing approved brass drawer and cupboard lock with brass screws including two operating keys: 2-3/4" (70 mm) size (4 levers) | 12 | Each |
|  | Providing & fixing fixed steel glazing made of M.S. pipe (18 gauge) of approved section comprising of frame L-Section 1-1/2" x 1-1/2" and 1/2" x -1/2" M.S. pipe beading for fixing glass panes, clamping the screws at 4 inches center to center, rubber packing for glazing, painted two coats with red oxide paint including using iron lugs, cutting holes and making good damages to walls etc. complete as required in any floor but excluding the cost of glass panes.. (For Glass Partition) | 882 | Sq.ft. |
|  | Providing and fixing Tinted glass panes 5mm thick to M.S. Box pipe / Aluminum doors, windows and ventilators etc including the cost of labour but excluding the cost of M.S. square pipe beading, rubber packing and screw in any floor at any height. | 882 | Sq.ft. |
|  | Providing and fixing 1-1/2 inches (38 mm) thick best quality deodar wood partition fully panelled with same wood etc. as required . | 77 | Sq.ft. |

Bill No. 1.1.10 Title: Miscellaneous

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Providing and fixing plain polyvinyl chloride (PVC) water stops 8" (203 mm) wide in vertical or horizontal expansion joints including cutting and jointing etc. complete in all floors . | 3 | Rft. |
|  | Providing and fixing Gypsum board 2' x 2' x 3/4" tiles ceiling including Aluminum T & L angle 1" x 1" i.c hanger clips jointing clips and G.I Wire etc complete as required in any floor. | 1827 | Sq.ft. |
|  | Providing and fixing iron grills with required section of flat iron as per approved design including welding all sides of the section at the junction, and fixing with sunk iron screws, painting two coats of red oxide paint etc. complete as required in any floor in masonry or concrete work. | 2036 | Kg |
|  | Providing and fixing square pipe railing size 2" x 2" of 16 gauge comprising vertically posts and horizontal bracing of M.S. square pipe of same size as per design including cost of cutting fitting including embedding the braces and posts in floor and walls and making good with cement concrete 1:2:4 in any floor. | 36 | Rft. |
|  | Providing & fixing C.I Cover & Frame 24" dia (610 mm) weight 2 cwt 2 Qtr (127.00 Kg.) | 2 | Each |
|  | Providing and layaing out Termite proofing in foundations, under floors and wood work surface with a soloution using of confront TC with water and spraying the solution of the area to be treated as completed as per direction of Enmgineer Incharge. | 1139 | Sft |
|  | P/F 1-1/2 Inch(38 mm) thick best quality PVC/UPVC door shutter panelled (approved design), approved iron hings and tower bolt, etc. as required. | 250 | Sft |
|  | Providing and laying marble flooring comprising 24″ x 12″ x 7/8″ tiles resized fixed in neat cement buttering laid over 1¼″ thick P.C.C. 1:2:4 using ¼″ down course aggregates to final level/slopes, including filling joints with white cement, curing, grinding, sand papering and polishing in chemical polish complete as per specification. Marble quality to be “Badel-light/Ziarat Light Gray” and subject to prior approval by the Engineer, at any floor. | 1200 | 100 Sft |
|  | Providing and fixing marble skirting cladding etc.in 24"x4"x 3/8" pre-polished (in chemical polish) tiles with neat cement buttering over 1/2" thick 1:3 cement sand rough plaster including filling joints, curing etc., complete as per specification . Marble quality shall be the same as used in the flooring and shall comply with thepattern requirements as directed. | 45 | 100 Sft |
|  | Providing and laying floor of 3/4 inches (19 mm) thick Granite stone Pakistani Original more than One sq. ft. upto 3 Sq. ft. (0.093 Sq.m. upto 0.28 sq.m.) size fine dressed on the surface without winding in ground floor over 1 inch (25 mm) lime mortar 1:2 (one lime, two sand) setting the marble tiles with grey cement slurry including curing rubbing and polishing etc. complete, including the cost of mortar. | 105 | 100 Sft |
|  | Providing and laying floor of 3/4 inches (19 mm) thick Granite stone Pakistani Original more than 3 sq. ft. upto 5 Sq. ft. (0.28 Sq.m. upto 0.46 sq.m.) size fine dressed on the surface without winding in ground floor over 1 inch (25 mm) lime mortar 1:2 (one lime, two sand) setting the marble tiles with grey cement slurry including curing rubbing and polishing etc. complete, including the cost of mortar. | 125 | 100 Sft |
|  | Providing and laying floor of 3/4 inches (19 mm) thick Granite stone Pakistani Original more than 5 sq. ft. upto 7 Sq. ft. (0.46 Sq.m. upto 0.65 sq.m.) size fine dressed on the surface without winding in ground floor over 1 inch (25 mm) lime mortar 1:2 (one lime, two sand) setting the marble tiles with grey cement slurry including curing rubbing and polishing etc. complete, including the cost of mortar. | 150 | 100 Sft |
|  | Extra for making nosing of treads as per design and instructions of the Engineer-in-Charge. | 250 | Rft. |
|  | Extra for stairs/steps (in treads, risers and mid stair landings). | 350 | 100 Sft |
|  | Providing Anti -termite treatment by spraying /sprinkling /spreading Heptachlor 0.5% Emulsion as an overall pre -construction treatment both vertically and horizontally anti-termite shall be three time excavation base, side & plinth area etc. measur shall be only one time plinth area complete and as per instructions of engineer in charge. | 1139 | Sft |
|  | Providing & fixing of approved quality in-ward openable sound proof uPVC doors in white colour made from 60mm x 60mm frame and 60mm x 100mm sash double glazed with 5mm thick tempered glass with 8mm air space between the two glass panels vacuumed with argon gas with best quality door hinges and door lock etc complete in all respects as per site requirements, specifications, instructions of the manufacturer and as directed by the Engineer In-charge.(For Bathroom) | 396 | Sq.ft. |
|  | Providing and fixing Stainless Steel Railing 3' height, 2" dia top horizantal pipe 16 Swg and 3 no,s 5/8" dia horizantal members 18 Swg, bluster (Fancy 5mm thick Strip ) of upto 3' c/c fixed with rawal bolt , welding, polishing, buffing etc complete in all respect. | 229 | Rft |
|  | Providing & Fixing Lasbela Stone 3/4" machine cut resize as requred including sand blasting at elevation, fixing with High strength bond at any height of building etc complete, design and pattern as per approved drawings and instructions of engineer in charge. | 1666 | Sq.ft. |
|  | Making cement plaster pattas /band around straight or curved openings and around the edges of windows and opening with fine finishes as directed by engineer in charge. | 229 | Rft |
|  | Providing and laying marble flooring comprising 24″ x 12″ x 7/8″ tiles resized fixed in neat cement buttering laid over 1¼″ thick P.C.C. 1:2:4 using ¼″ down course aggregates to final level/slopes, including filling joints with white cement, curing, grinding, sand papering and polishing in chemical polish complete as per specification. Marble quality to be “Badel-light/Ziarat Light Gray” and subject to prior approval by the Engineer. The floor shall be laid in pattern using tiles of different colours/shades as directed by the Engineer, at any floor. Also using 3″ x 24″ x 7/8″ jet back marble strips as boarder as per given pattern, complete. | 1822 | Sft |
|  | Providing and fixing marble skirting/cladding etc., in 24″ x 4″ x 3/8″ pre-polished (in chemical polish) tiles with neat cement buttering over ½″ thick 1:3 cement sand rough plaster including filling joints, curing etc., complete as per specification. Marble quality shall be the Badal-Light-Shade/ Ziarat Light Gray and shall comply with the pattern requirements as directed. | 77 | Sft |
|  | Providing and laying approved Quality Marble slab polished ( Light Gray Marble Slab) Required Size Tread on laid over 7/8 inch (22 mm) thick cement mortar 1:2 setting the tiles with Bound over cement mortar, jointing the tiles with white cement slurry and nosing including curing, etc.complete (Imported/Local-origin China, Italy or equivalent) all as specified.& Instruction by engineer in charge | 241 | Sq.ft. |
|  | Providing and laying approved Marble slab) polished ( Light Gray Marble Slab) Required Size on Riser laid over 3/8 inch (10 mm) thick cement mortar 1:2 setting the tiles with Bound over cement mortar, jointing the tiles with white cement slurry and nosing including curing, etc.complete (Imported/Local-origin China, Italy or equivalent) all as specified.& Instruction by engineer in charge | 80 | Sq.ft. |
|  | Providing and laying approved Quality Granite Marble slab polished Required Size Tread on laid over 7/8 inch (22 mm) thick cement mortar 1:2 setting the tiles with Bound over cement mortar, jointing the tiles with white cement slurry and nosing including curing, etc.complete all as specified.& Instruction by engineer in charge | 33 | Sq.ft. |
|  | Providing and laying approved Granite Marble slab polished Required Size on Riser laid over 3/8 inch (10 mm) thick cement mortar 1:2 setting the tiles with Bound over cement mortar, jointing the tiles with white cement slurry and nosing including curing, etc.complete all as specified.& Instruction by engineer in charge | 22 | Sq.ft. |
|  | Providing and laying approved Quality Granite Marble tiles 24" x 24" x 7/8" on floor laid over 1 inch (25 mm) thick cement mortar 1:2 setting the tiles with Bound over cement mortar, jointing the tiles with white cement slurry and nosing including curing, etc.complete all as specified.& Instruction by engineer in charge | 112 | Sq.ft. |
|  | Providing and laying approved Marble slab Required Size on counter tops laid over 3/4 inch (20 mm) thick cement mortar 1:2 setting the tiles with Bound over cement mortar, jointing the tiles with white cement slurry and nosing including curing, etc.complete (Imported/Local-origin China, Italy or equivalent) all as specified.& Instruction by engineer in charge | 121 | Sq.ft. |
|  | Providing and laying approved Marble slab Required Size on Venity tops laid over 3/4 inch (20 mm) thick cement mortar 1:2 setting the tiles with Bound over cement mortar, jointing the tiles with white cement slurry and nosing including curing, etc.complete (Imported/Local-origin China, Italy or equivalent) all as specified.& Instruction by engineer in charge | 32 | Sq.ft. |
|  | Providing and fixing jeot cloth over jambolon sheet 1/2" thick fixed with wooden frame 2'x2' using batten partal wood 2"x2" section on polythene sheet 50 gauge on wall, fill gap with Glass wool 2" thickness cover with Soft Board 1/2" thick, on walls with screws/nail, wooden pressed sheet strip 3"x1/4" vertically on cloth, for sound proofing. Skirting of MDF lamination sheet 3/4" thick with edging around walls at floor level in 12" height. Complete as per drawing and specification. | 1526 | Sq.ft. |
|  | Providing and fixing approved quality push bar panic exit lock Latch stainless steel 5/8” throwback set 2-3/4” dogging feature strikes adjustable with slotted holes for adjustment as per instruction of engineer in charge | 2 | Each |

Bill No. 1.1.11 Title: Water Supply, Sanitary, Plumbing Fixtures & Sewerage

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Providing G.I. pipes, specials and clamps etc., including fixing cutting and fitting complete with and including the cost of breaking through walls and roof, making good etc., painting 2 coats after cleaning the pipe etc.; with white zinc paint with pigment to match the color of the building and testing with water to a pressure head of 200 feet (61 M) and cartages within 10 miles (16.09 km.) (ILLM). |  |  |
|  | a) 1-1/2 inches (40mm) dia | 337 | R.ft |
|  | b) 2 inches (50mm) dia | 269 | R.ft |
|  | c) 3 inches (75mm) dia | 6 | R.ft |
|  | Providing and fixing ball valve (with PVC Ball) made to B.S.S. 1212..... |  |  |
|  | a) 1 inches (25mm) dia | 14 | Each |
|  | b) 1-1/2 inches (40mm) dia | 12 | Each |
|  | c) 2 inches (50mm) dia | 9 | Each |
|  | d) 3 inches (75mm) dia | 1 | Each |
|  | Providing and fixing cast iron foot valve with flanged ends complete with a strainer suitable for fixing to 2 inch (50 mm) dia. suction pipe including 1/8 inches (3.2 mm) thick rubber washer… | 1 | Each |
|  | Providing and fixing gun-metal non-return valves with male and female ends as required. 2 inches (50 mm) dia.. | 1 | Each |
|  | Providing and fixing best quality squatting type white glazed earthenware W.C Pan, Pakistani (of not less than 18 Inches clear opening as measured between flushing rims) complete with and including the cost of 13.6 liter best quality low level plastic flushing cistern with internal fittings complete, P.V.C. flushing pipe suitable for squatting type with extra bends and length with fittings, C.I. trap 4 inches (100mm) dia and making requisite number of holes in walls, plinth & floor for pipe connection and making good in cement concrete 1:2:4.. | 4 | Each |
|  | Providing and fixing Pakistani best quality European style white glazed earthenware W.,C pan complete with and including the cost of plastic seat and buffers, 13.6 liters best quality low level plastic flushing cistern with internal fittings complete, P.V.C flushing pipe with fittings and making requisite number of holes in walls, plinth and floor for pipe connections and making good in cement concrete 1:2:4.. | 9 | Each |
|  | Providing and fixing 22 inches x 16 inches (560 x 406 mm) lavatory basin in white glazed earthenware (Pakistani) complete with and including the cost of Brass oxidized bolts kit built into walls 1/2 inches (15 mm) dia. Chrome plated mixer, 1-1/4 inches (32 mm) rubber plug and chrome plated brass chain 1-1/4 inches (32 mm) dia brass waste of approved pattern 1-1/4 inches (32 mm) dia chromium plated battal trap with unions and making requisite, number of holes in walls, plinth and floor for pip.e connections and making good in cement concrete 1:2:4.. | 5 | Each |
|  | Providing and fixing bath room accessories of set of 6 pieces consist of one shelf, one towel rod with bracket, one towel ring, one soap dish, one tooth Brush holder with glass, one tissue paper holder one double hook etc complete of approved quality as per direction of Engineer in-charge. | 6 | Each |
|  | Providing and fixing 20 inches x 16 inches (508 mm x 406 mm) Looking mirror of Belgium glass complete with Plastic frame and C.P. Brass screws. | 10 | Each |
|  | Fixing floor traps of sizes with gratings including cutting and making the requisite number of holes in walls, plinth and floors and making good in 1:2:4 C.C.. | 23 | Each |
|  | Providing 1/2 inch (15 mm) dia plastic connection complete with a 1/2 inch (15 mm) dia brass stop cock two brass nuts and lining jointed to plastic pipe. | 34 | Each |
|  | Providing and fixing approved quality stainless steel sink 33” x 18” Pak made (Atlas) complete with brass oxidized bolt kit/angle iron brackets built into walls ½” dia CP sink mixer 1-1/4” rubber plug and CP brass chain 1-1/4” CP brass waste 1-1/4” dia malleable iron or CP brass bottle trap with malleable iron or brass unions and making requisite number of holes in walls, plinth and floor for pipe connections and making good in cement concrete 1:2:4 | 2 | Each |
|  | Providing and fixing standing wall shower of CP brass 3 knobs of approved quality mixer unit and moveable shower head complete.. | 2 | Each |
|  | Providing and fixing 30" x 3/4"(762 mm x 20 mm) dia approved quality of chrome plated brass towel rail complete with brackets fixed with 1"(25 mm) long C.P brass screws and rawal plug. | 4 | Each |
|  | Providing and fixing M.S. clamps of the approved design to 3 inches (80mm) dia C.I. pipe sockets including the cost of cutting and making good to wall or M.S. Bolts and nuts built 3 inches (80mm) into walls including pipe distance pieces and extra painting to match the color of the building and cartage within 10 miles. (16.09 Km) | 21 | Each |
|  | Providing and fixing M.S. clamps of the approved design to 4 inches (100 mm) dia C.I. pipe spun sockets including the cost of cutting and making good to wall or M.S. bolts and nuts built. 4 inches (100 mm) in to wall including pipe distance pieces, extra painting to match the color of the building and cartage within 10 miles. (16.09 km.).. | 15 | Each |
|  | Providing and fixing 3/4 inches dia. (15 mm) brass bib cocks. | 4 | Each |
|  | Providing and fixing plastic (PVC) toilet paper holder in wall complete of any color. | 4 | Each |
|  | Providing and fixing approved quality earthen ware glazed vanity basin standard size (White/light color) and including the cost of 1-1/4” dia brass waste 1-1/4” rubber plug and chrome plated brass chain PVC waste pipe 1-1/4” dia of approved pattern etc. complete. | 4 | Each |
|  | Providing and fixing in wall fire extinguisher of approved quality 2 gallons (9.09 liters) capacity complete with brackets including cutting holes in walls, fixing wooden gutties and making good the same with cement concrete 1:2:4 etc., complete. | 4 | Each |
|  | Providing, laying, fixing, testing and disinfecting, polypropylene pipelines for cold and hot water supply as per DIN 8077/8078 PN-20 for pipe and DIN 16962 PN-25 for fittings or equivalent BS specification Dadex, Firat (Turkey make) or formul (Turkey make) make as approved by the Engineer Incharge complete in all respects to their entire satisfaction, including specials such as tee, cross, reducer, bend, union, elbow, plug, socket, etc., supported on walls or suspended from slab or run in chases including supports, cutting and making good the same as necessary to the structure, excavation and backfilling in layers complete in all respects. |  |  |
|  | i) 25 mm o/d | 29 | Rft |
|  | ii) 32 mm o/d | 377 | Rft |
|  | Providing and fixing un platisized polyvinyl chloride pipe (U.P.V.C) “D” class (Acid Proof) of (Dadex/TurkPlast/Popular) with specials and clamps etc, including cutting and fitting complete with and including the cost of breaking through walls and roof and making good etc., after cleaning the pipe and cartage within 10 miles (16.09 km). |  |  |
|  | i) 6”dia | 25 | Rft |
|  | ii) 4”dia | 80 | Rft |
|  | iii) 3”dia | 122 | Rft |
|  | P/Fixing PPR pipe of dadex i/c testing, jointing and specials inside clear dia 3/4" complete as approved by Engineer. |  |  |
|  | a)3/4 inches (20mm) dia | 29 | R.ft |
|  | b)1 inches (25mm) dia | 377 | R.ft |
|  | Providing and fixing C.P Muslim Shower master/faisal make complete in all respects. | 13 | Each |
|  | Providing and fixing upvc Soil and Waste Pipe with specials and clamps including fixing, cutting and fitting including the cost of breaking through walls and roofs etc complete and as per instructions of engineer in charge. 3 inches (75mm) dia | 222 | Rft |
|  | Providing and fixing upvc Soil and Waste Pipe with specials and clamps including fixing, cutting and fitting including the cost of breaking through walls and roofs etc complete and as per instructions of engineer in charge. 4 inches (110mm) dia | 130 | Rft |
|  | Providing and fixing uPVC Vent Pipe including cutting and fitting including the cost of painting 2 coats after cleaning the pipe with white zinc paint with pigment to match the colour of the building in any floor etc. complete and as per instructions of engineer in charge. 4 inches (110mm) dia | 25 | Rft |
|  | Providing and fixing upvc Elbow (90 degree) including Z joint and Rubber ring etc complete and as per instructions of engineer in charge. 3 inches (75mm) dia | 30 | Each |
|  | Providing and fixing upvc Elbow (90 degree) including Z joint and Rubber ring etc complete and as per instructions of engineer in charge. 4 inches (110mm) dia | 18 | Each |
|  | Providing and fixing upvc Yee Tee including Z joint and Rubber ring etc complete and as per instructions of engineer in charge. 3 inches (75mm) dia | 31 | Each |
|  | Providing and fixing upvc Yee Tee including Z joint and Rubber ring etc complete and as per instructions of engineer in charge. 4 inches (110mm) dia | 23 | Each |
|  | Providing and fixing upvc Elbow with door (90 degree) including Z joint and Rubber ring etc complete and as per instructions of engineer in charge. 3 inches (75mm) dia | 11 | Each |
|  | Providing and fixing upvc Elbow with door (90 degree) including Z joint and Rubber ring etc complete and as per instructions of engineer in charge. 4 inches (110mm) dia | 11 | Each |
|  | Providing and fixing Soap tray of approved shape pattern and size, complete with plugs, screws etc complete and as per instructions of engineer in charge. | 8 | Each |
|  | Supply and fix, Disabled bath wash basin, glazed ware, Imported (like Porta, Casamoda or equivalent) in white / Ivory colour, one hole, complete with waste pipe coupling, CP chain and plug, automatic mixer etc best quality, fixed to concrete, brick, stone or wood work as per instruction of engineer in charge | 1 | Each |
|  | Supply and fix, Disabled WC apparatus, European Pattern, complete (coupled set), comprising closet 13 lit flushing cistern glazed, in white colour, seat cover, rails ,handle etc complete set ( IFO Porta, Casamoda or equivalent,) fixed to concrete , brick ,stone or wood work, best quality, as per instruction of engineer in charge | 1 | Each |
|  | P/F gas cooking range with three burner pakistani singer model No.SG 312 with all accessories,gas inlet connection,gas cock etc. complete in all respect, as per direction of engineer incharge i/c testing/ commissioning, complete. | 1 | Each |

Bill No. 1.2.1 Title: Footpath, Driveway, Lawn

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Providing and laying 1:4:8 (1 cement 4 sand and 8 coarse aggregate) cement concrete using crushed graded boulders (machine broken) in foundation including levelling, compacting and curing etc.complete. | 231 | 100 Cft |
|  | Supplying earth from approved outside sources within a radius of 5 miles (8 km) including digging, loading and unloading and filling in foundation trenches, plinth or under floor, etc. including breaking clods, dressing, watering and consolidation by ramming in layers not exceeding 9 inches (229 mm) in depth to full compaction complete within a ;lead of one chain (30.5 R.m) and lift of 5 feet (1.52 m). | 5278 | 100 Cft |
|  | Supplying and stacking approved garden soil (sweet earth) free from salts, pebbles and grass roots etc. including all leads and lifts. | 3053 | 100 Cft |
|  | Supplying and stacking well decayed cowdung manure from approved source including all leads and lifts. | 1543 | 100 Cft |
|  | Supply live (Dacca) grass in slabs with earth intact including all lead and lifts. | 3053 | Sq.ft. |
|  | Rough dressing of the area for lawn including picking stones, pebbles, stubble, grass roots and other injurious matters and breaking of clods complete including disposal of rubbish within 3 chains (92 m). | 3053 | 100 Sft |
|  | Preparation of sub-grade over top of embankment already compacted to at least 95% modified A.A.S.H.O. maximum dry density with required dressing including cutting to required grade, camber and side slopes and disposal of surplus earth within three chains lead (92 m) complete. | 6596 | 100 Sft |
|  | Supplying, stacking, spreading and filling stones of required size to required slope from approved quarries in plinth or under floor including filling voids with spawl and chips and consolidation by ramming layers not exceeding 9 inches (229 mm) in depth to full compaction lift upto 5 feet (1.52 m). | 1649 | 100 Cft |
|  | Supplying and stacking with all lead and lift: pit sand (local) . | 1121 | 100Cft. |
|  | Providing and laying in floor C.C. 1:2:4: tuff paver 2-1/2" thick of approved design and color pattern (average strength 7000 psi) laid on sand cushion filling of joint with sand and warring etc. complete as per direction of Engineer Incharge. (the cost of sand cushion is not included) | 6596 | Sft |
|  | Providing and laying road kerb of precast cement concrete 1:2:4 (24" long 6" wide and 12" deep (610 mm long 152 mm wide and 305 mm deep) using screened graded bajri, from work and its removal compacting and curing laid in cement mortar 1:4 and cement plaster 1:4 neatly finished including cost of excavation and back filling of excavated stuff and its disposal within one chain lead as desired. | 307 | 100Rft. |
|  | Painting three coats of thermo plastic paint (for kerb stone) of approved make and shade etc complete and as per instructions of engineer in charge. | 307 | Sft |

Bill No. 1.2.2 Title:	Boundary Wall

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Dismantling stone masonry in lime or cement mortar in foundation, plinth and ground floor including stacking salvaged material (serviceable) and disposing surplus stuff as directed within three chains (91.5 m) | 1209 | 100 Cft |
|  | Cartage of stone, brickbats, ballast, shingle, boulders & C.C. block including loading, unloading and stacking etc. at site: (4.8 Km) | 1209 | 100 Cft |
|  | Excavation for foundation trenches and drains in all kinds of soil (except gravelly and murum soil, wet silt, clay or mud conglomeration of gravel and boulders, soft, sandy or disintegrated sandy and hard rock) and back filling the excavated material in foundation, plinth or under floor including breaking clods, watering, consolidation by ramming in layers not exceeding 9 inches (229 mm) in depth to full compaction, dressing and disposal of surplus excavated stuff as directed, lead upto one chain (30.5 R.m) and lift upto 5 feet (1.52 m). | 1923 | 100 Cft |
|  | Providing and laying 1:4:8 (1 cement 4 sand and 8 coarse aggregate) cement concrete using crushed graded boulders (machine broken) in foundation including levelling, compacting and curing etc.complete. | 353 | 100 Cft |
|  | Providing and laying reinforced cement concrete using screened graded bajri 3/4 inch (19 mm) and down gauge having a minimum works cube crushing strength of 4500 Ibs. per sq. inch (31.04 N/mm2 at 28 days) with a mix not leaner than 1:1:2 including form work and its removal, compacting and curing etc. complete but excluding the cost of reinforcement, in foundation basement and plinth |  |  |
|  | 1) Sub Structure |  |  |
|  | a) Footing | 317 | 100 Cft |
|  | b) Short Column | 178 | 100 Cft |
|  | c) Plinth Beam | 422 | 100 Cft |
|  | 2) Super Structure |  |  |
|  | a) Column | 260 | 100 Cft |
|  | b) Beam | 80 | 100 Cft |
|  | Providing and laying in situ 1:3:6 (1 cement 3 sand and 6 coarse aggregate) cement concrete using crushed graded boulders 3/4 inch (19 mm) and down gauge in foundation, basement and plinth including form work, compacting, curing and removal of form work etc. complete, foundation and basement up to 5 feet (1.52 m) depth and plinth up to 4 feet (1.2 m) height from ground level . | 496 | 100 Cft |
|  | Providing and laying in situ 1:2:4 cement concrete using screened graded bajri 3/4 inch (19 mm) and down gauge in architraves, coping and cornice etc. of required shape in superstructure including compacting, curing, cost of form work and its removal etc., complete in ground floor .. | 19 | 100 Cft |
|  | d)Extra for using sulphate resisting cement in place of ordinary portland cement ..(Up to Plinth Level) | 377 | Cwt |
|  | Providing and laying hard grade ribbed deformed (minimum yield point 60,000 psi) reinforcement bars with & including the cost of straightening, cutting, bending, binding, wastage, and such overlaps as are not shown in the drawings, placing in position on cement concrete 1:2:4 precast or m.s. chairs, tying with binding wire, cost of chairs and wires etc. in all kinds of RCC work in foundation, basement, plinth and ground floor of building including septic tanks and under ground tanks and in projections for future extension . | 5702 | kg |
|  | Providing and laying 1:3:6 cement concrete solid block masonry 4 to 6 inches (102 mm to 152 mm) thick using graded screened bajri 3/4 inch (19 mm) and down gauge set in cement mortar 1:4 including scaffolding, raking out joints and curing etc. complete in ground floor superstructure. | 1588 | 100 Cft |
|  | Providing and fixing angle iron fencing posts embedded in concrete and painting exposed portion with aluminum paint 3 coats including necessary holes but excluding the cost of excavation and concrete, complete . | 4 | Cwt. |
|  | Providing and fixing galvanized iron barbed wire fencing (with four points) to the posts of any description complete with hooks and binding wire etc- if required (cost of posts not included). | 2 | Cwt. |
|  | 3/4'' (19 mm) thick cement plaster 1:4 on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete. (For External Surface) | 9260 | 100 Sft |
|  | Providing and applying 3/4" (19 mm) thick rough cast cement rendering consisting of 3/8" or 9.6 mm topping of 1:2 (one cement and two mixed graded bajri 3/8" or 9.5 mm and down gauge) and base of 3/8" or 9.5 mm thick rough plaster 1:4 (one cement four sand) in plinth and ground floor including finishing etc. complete .(Below Stone Facing) | 76 | 100 Sft |
|  | 1-1/2" (38 mm) thick cement plaster 1:4 in two layers on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete . | 105 | 100 Sft |
|  | Painting three coats with weather shield paint deluxe (ICI) make of approved shade on plaster surface (External) and including the cost of cleaning the surface, sand papering etc. complete at any height in any floor. | 9260 | 100 Sft. |
|  | Providing and fixing Galvanized Razor wire (Local made) having double sharp 4 Nos. pointers @ making 18" dia, Ring 8" c/c fixed on angle iron post 1-1/2" x 1-1/2" x 3/16" @ 7'-0" c/c Y shape, 3 Nos. of tension wire 10 SWG horizontaly. Complete in all respect. | 600 | Rft |
|  | Providing and fixing heavy type steel gates with teak wood planks of 1" thick made to crittal or equivalent standard and of approved sections painted with coats of red oxide paint including using approved brass section oxidized fittings, iron lugs, including cutting holes and making good damages to walls etc. complete as required and as per instructions of engineer in charge. | 1 | Job |

Bill No. 1.2.3 Title:	Guard Room

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Excavation for foundation trenches and drains in all kinds of murum soil and back filling the excavated material in foundation trenches plinth or under floor including breaking clods, watering, consolidation by ramming in layers not exceeding 9 inches (229 mm) in depth to full compaction, dressing and disposal of surplus excavated stuff as directed lead up to one chain (23.5 R.m) and lift 5 feet (1.52 m). | 163 | 100 Cft |
|  | Supplying earth from approved outside sources within a radius of 5 miles (8 km) including digging, loading and unloading and filling in foundation trenches, plinth or under floor, etc. including breaking clods, dressing, watering and consolidation by ramming in layers not exceeding 9 inches (229 mm) in depth to full compaction complete within a ;lead of one chain (30.5 R.m) and lift of 5 feet (1.52 m). | 66 | 100 Cft |
|  | Providing and laying 1:4:8 (1 cement 4 sand and 8 coarse aggregate) cement concrete using crushed graded boulders (machine broken) in foundation including levelling, compacting and curing etc. complete. | 25 | 100 Cft |
|  | Providing and laying in situ 1:3:6 (1 cement 3 sand and 6 coarse aggregate) cement concrete using crushed graded boulders 3/4 inch (19 mm) and down gauge in foundation, basement and plinth including form work, compacting, curing and removal of form work etc. complete, foundation and basement up to 5 feet (1.52 m) depth and plinth up to 4 feet (1.2 m) height from ground level . | 25 | 100 Cft |
|  | Providing and laying reinforced cement concrete using screened graded bajri 3/4 inch (19 mm) and down gauge having a minimum works cube crushing strength of 4500 Ibs. per sq. inch (31.04 N/mm2 at 28 days) with a mix not leaner than 1:1:2 including form work and its removal, compacting and curing etc. complete but excluding the cost of reinforcement, in foundation basement and plinth |  |  |
|  | 1) Foundation |  |  |
|  | a) Footing | 27 | 100 Cft |
|  | b) Short Column | 8 | 100 Cft |
|  | c) Plinth Beam | 24 | 100 Cft |
|  | 2) Ground Floor |  |  |
|  | a) Column | 78 | 100 Cft |
|  | b) Lintel | 5 | 100 Cft |
|  | c) Inclined Beam | 99 | 100 Cft |
|  | d) Sloping Slab | 19 | 100 Cft |
|  | Extra for using sulphate resisting cement in place of ordinary Portland cement. (Up to Plinth Level) | 23 | Cwt |
|  | Providing and laying hard grade ribbed deformed (minimum yield point 60,000 psi) reinforcement bars with & including the cost of straightening, cutting, bending, binding, wastage, and such overlaps as are not shown in the drawings, placing in position on cement concrete 1:2:4 precast or m.s. chairs, tying with binding wire, cost of chairs and wires etc. in all kinds of RCC work in foundation, basement, plinth and ground floor of building including septic tanks and under ground tanks and in projections for future extension . | 1173 | kg |
|  | Providing and laying 1:3:6 cement concrete solid block masonry 4 to 6 inches (102 mm to 152 mm) thick using graded screened bajri 3/4 inch (19 mm) and down gauge set in cement mortar 1:4 including scaffolding, raking out joints and curing etc. complete in ground floor superstructure. | 83 | 100 Cft |
|  | 1/2" (13 mm) thick cement plaster 1:5 on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete .(For Internal Surface) | 224 | 100 Sft |
|  | 1/2" (13 mm) thick cement plaster 1:3 on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete .(For Ceiling ) | 156 | 100 Sft |
|  | 3/4'' (19 mm) thick cement plaster 1:4 on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete. (For External Surface) | 240 | 100 Sft |
|  | Providing and applying 3/4" (19 mm) thick rough cast cement rendering consisting of 3/8" or 9.6 mm topping of 1:2 (one cement and two mixed graded bajri 3/8" or 9.5 mm and down gauge) and base of 3/8" or 9.5 mm thick rough plaster 1:4 (one cement four sand) in plinth and ground floor including finishing etc. complete .(Below Stone Facing) | 14 | 100 Sft |
|  | Providing/applying 3 coats of ICI weather Shield paint to plastered surface in approved color/shade and according to the approved color scheme including applying primer of same manufacturer. The surface is to be properly prepared by rubbing, sand papering, cleaning, filling and making smooth before applying the priming coats, complete as directed any height at any floor. | 226 | 100 Sft |
|  | .Providing and Applying one Coat of Deluxe/Robbilac Primeon/Alkali Resisting Masonry Sealer and minimum three coats of Vinyl emulsion including surface preparaion,filling and rubbing etc,as per specification clause11.4,complete in all respect upto any height at any floor. | 380 | 100 Sft |
|  | Supplying, stacking, spreading and filling stones of required size to required slope from approved quarries in plinth or under floor including filling voids with spawl and chips and consolidation by ramming layers not exceeding 9 inches (229 mm) in depth to full compaction lift upto 5 feet (1.52 m). | 19 | 100 Cft |
|  | Providing and laying 1:9 (1 cement 9 screened bajri) cement concrete using 3/4 inch (19 mm) and down gauge screened bajri in foundation including levelling, compacting and curing etc. complete . | 10 | 100 Cft |
|  | Providing and laying floors of 2 inches (51 mm) thick 1:2:4 cement concrete using graded screened bajri 3/4 inch (19 mm) and down gauge in ground floor laid in panels including form work, consolidation, finishing, and curing etc. complete.(For Under Floor) | 56 | 100 Sft |
|  | Providing and laying Khaprail tiles C.C 1:2:4: in different design laid on sloppy slab & shades laid in cement mortar 1:2 in Joints and using etc. as per directions of the Engineer incharge. | 39 | Sq.ft. |
|  | Providing and fixing best quality deodar frames for doors. windows, ventilators, clerestory windows, shelves, partitions, trellis work, etc., as required . | 2 | Cft. |
|  | Providing and fixing 1-1/2 inches (38 mm) thick pressed veneered door shutters fully flushed with commercial ply wood veneering on all faces and sides fixed over deodar wood cavitied core and frame work of not less than 4 inches (102 mm) wide strip alround with approved brass hinges and tower bolts etc., as required . | 18 | Sq.ft. |
|  | Applying French or spirit polishing, two coat of approved make on wood work at any height in any floor . | 35 | 100 Sft |
|  | Providing and fixing on frames etc., with sunken iron screws best quality deodar wood moulding, beading, architrave etc., of approved design over one square inch (6.45 Sqm.) and up-to 2 square inches (13 Sqm.) in cross section as required . | 17 | Rft. |
|  | Providing and fixing with brass screws door lock handle type of approved design (Hungry make), cutting wood etc., to required shape and size with two operating keys as per direction of the Engineer-in-Charge. | 1 | Each |
|  | Providing and fixing with brass screws or specially supplied screws hydraulic door closer branno of approved design No. 3 (Yugoslavia make), cutting wood etc., to required shape and size as per direction of Engineer-in-Charge. | 1 | Each |
|  | Providing and fixing double glazed Champagne anodized aluminum Sliding windows as per British standard manufactured by Luckh, Alcop, Krudson, Pakistan Cables and A.C.P. (fixing through their approved fabricators), Executive model section dubble or single glazed 101mm x 37mm and 2mm thick including the cost of aluminum netting ,fitting, with all accessories cutting hole etc. and making good damages to walls etc. complete as required in any floor as per direction of engineer-in-charge, including the cost of Tinted Glass 5mm thick (Pakistani). (For Windows & Ventilator) | 32 | Sq.ft. |
|  | Providing and Laying Flooring (Glazed) Comprising 16"x 16" 3/8" (1600 cm) available thickness Granitto tiles (Mater) fixed in neat cement buttering laid over 1-1/4" thick PCC 1:2:4 using 3/8" down course aggregates to final levels and stop including filling joints with ready made approved fillers of suitable colors complete any floor at any height as directed. | 71 | 100 Sft |
|  | Providing and fixing iron grills with required section of flat iron as per approved design including welding all sides of the section at the junction, and fixing with sunk iron screws, painting two coats of red oxide paint etc. complete as required in any floor in masonry or concrete work. | 74 | Kg |
|  | Providing Anti -termite treatment by spraying /sprinkling /spreading Heptachlor 0.5% Emulsion as an overall pre -construction treatment both vertically and horizontally etc. complete and as per instructions of engineer in charge. | 72 | Sq.ft. |
|  | Making cement plaster pattas /band around straight or curved openings and around the edges of windows and opening with fine finishes as directed by engineer in charge. | 150 | Rft |
|  | Providing & Fixing Lasbela Stone 1/2” machine cut resize as requred including sand blasting at elevation, fixing with High strength bond at any height of building etc complete, design and pattern as per approved drawings and instructions of engineer in charge. | 14 | Sq.ft. |

Bill No. 1.2.4 Title:	Parking Shed

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Excavation for foundation trenches and drains in all kinds of murum soil and back filling the excavated material in foundation trenches plinth or under floor including breaking clods, watering, consolidation by ramming in layers not exceeding 9 inches (229 mm) in depth to full compaction, dressing and disposal of surplus excavated stuff as directed lead upto one chain (23.5 R.m) and lift 5 feet (1.52 m).. | 527 | 100 Cft |
| 2 | Providing and laying 1:4:8 (1 cement 4 sand and 8 coarse aggregate) cement concrete using crushed graded boulders (machine broken) in foundation including levelling, compacting and curing etc.complete. | 53 | 100 Cft |
| 3 | Providing and laying reinforced cement concrete using screened graded bajri 3/4 inch (19 mm) and down gauge having a minimum works cube crushing strength of 4500 Ibs. per sq. inch (31.04 N/mm2 at 28 days) with a mix not leaner than 1:1:2 including form work and its removal, compacting and curing etc. complete but excluding the cost of reinforcement, in foundation basement and plinth |  |  |
|  | 1)Foundation |  |  |
|  | a) Footing | 144 | 100 Cft |
|  | b) Short Column | 15 | 100 Cft |
|  | 2) Ground Floor |  |  |
|  | a) Column | 48 | 100 Cft |
|  | b) Inclined Beam | 208 | 100 Cft |
|  | c) Sloping Slab | 290 | 100 Cft |
| 5 | d)Extra for using sulphate resisting cement in place of ordinary portland cement ..(Up to Plinth Level) | 54 | Cwt |
| 6 | Providing and laying hard grade ribbed deformed (minimum yield point 60,000 psi) reinforcement bars with & including the cost of straightening, cutting, bending, binding, wastage, and such overlaps as are not shown in the drawings, placing in position on cement concrete 1:2:4 precast or m.s. chairs, tying with binding wire, cost of chairs and wires etc. in all kinds of RCC work in foundation, basement, plinth and ground floor of building including septic tanks and under ground tanks and in projections for future extension . | 3202 | kg |
| 7 | 3/4'' (19 mm) thick cement plaster 1:4 on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete. (For External Suface) | 879 | 100 Sft |
| 8 | Painting three coats with weather shield paint deluxe (ICI) make of approved shade on plaster surface (External) and including the cost of cleaning the surface, sand papering etc. complete at any height in any floor. | 128 | 100 Sft |
| 9 | Providing and Applying one Coat of Deluxe/Robbilac Primeon/Alkali Resisting Masonry Sealer and minimum three coats of Vinyl emulsion including surface preparaion,filling and rubbing etc,complete in all respect upto any height at any floor. | 879 | 100 Sft |
| 10 | Providing and laying Khaprail tiles C.C 1:2:4: in different design laid on sloppy slab & shades laid in cement mortar 1:2 in Joints and using etc. as per directions of the Engineer incharge. | 879 | Sq.ft. |
| 11 | Preparation of sub-grade over top of embankment already compacted to at least 95% modified A.A.S.H.O. maximum dry density with required dressing including cutting to required grade, camber and side slopes and disposal of surplus earth within three chains lead (92 m) complete. | 879 | 100 Sft |
| 12 | Supplying, stacking, spreading and filling stones of required size to required slope from approved quarries in plinth or under floor including filling voids with spawl and chips and consolidation by ramming layers not exceeding 9 inches (229 mm) in depth to full compaction lift upto 5 feet (1.52 m). | 440 | 100 Cft |
| 13 | Supplying and stacking with all lead and lift: pit sand (local) . | 149 | 100Cft. |
| 14 | Providing and laying in floor C.C. 1:2:4: tuff paver 2-1/2" thick of approved design and colour pattren (average strength 7000 psi) laid on sand cushion filling of joint with sand and warring etc. complete as per direction of Engineer Incharge. (the cost of sand cushion is not included) | 879 | Sft |

Bill No. 1.2.5	 Title: External Water Supply and Sewerage

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing G.I. pipes and specials, etc.; including fixing, cutting and fitting, complete with and including the cost of cutting trench upto 2-1/2 feet (762 mm) deep, refilling, watering, ramming and disposal of surplus earth within one chain and painting 2 coats of bitumen paint to pipes and specials after cleaning and hessian cloth soaked in maxphalt composition wrapped tightly round the pipes and testing to a pressure head of 200 feet (61 m) and cartage within 10 miles. (16.09km). (ILLM)...1-1/2 inches (40 mm) dia pipe | 364 | R.ft |
| 2 | Providing and fixing 1-1/2 inches (40 mm) dia dia ball valve (with PVC Ball) made to B.S.S. 1212..... | 3 | Each |
| 3 | Making connection with the existing G.I. pipe of size larger than 2 inches to 6 inches (50 mm to 150 mm) dia. including cutting the pipe and providing and fixing necessary fittings. | 1 | Each |
| 4 | Providing R.C.C. pipes and collars of class 'C' and fixing in trench including cutting, fitting and jointing with maxphalt composition and cement mortar (1:1) and back filling the excavated material in trench including testing with water to a pressure head of 80 feet (25 m) and cartage within 10 miles (16.09 km.) |  |  |
|  | a) 6 inches (150 mm) dia. pipe.. | 190 | R.ft. |
|  | b) 8 inches (200 mm) dia. pipe.. | 260 | R.ft. |
| 5 | Providing manhole type 'A' size 2 feet x 1-1/2 feet or (610 mm x 457 mm) (inside dimensions) x 2 feet (610 mm) deep as per approved design and specifications complete for 4 inches to 12 inches (100 to 305 mm) dia pipes up to 3 feet-11 inches (1.19 m) depth with cast iron cover and frame 16 inches (406 mm) dia, inside (weight=1 Cwt.1 Qtr.) or 63.5 kg. fixed in 4 inches (100 mm) thick RCC 1:2:4 slab, 8 inches (203 mm) thick c.c. 1:3:6 block masonry walls set in 1:3 c.m; 6 inches (150 mm) thick 1:3:6 c.c. in foundation 1:2:4 c.c. in benching, 1/2 inches (13 mm) thick cement plaster in 1:3 C.M. to all inside wall surface, channels and benching etc. and to top including making requisite umber of main and branch channels but excluding the cost of excavation back filling and disposal of excavated stuff, manhole cover and frame. | 12 | Each |
| 6 | Providing manhole type 'B' size 3 feet x 2-1/2 feet or 914 mm x 762 mm (inside dimensions) x 4 feet (1.22m) deep as per approved design and specifications complete 4 inches to 12 inches (100 mm to 305 mm) dia. pipes 4 feet to 7 feet-5 inches (1.22 m to 2.26 m) depth with cast iron cover and frame 18 inches (460 mm) dia. (inside) weight -1 Cwt. 3 Qtrs. (88.90 kg,) in 6 inches (150 mm) thick RCC 1:2:4 slab, 8 inches (203 mm) thick c.c. 1:3:6 block masonry walls set in 1:3 c.m. 6 inches (150mm) thick, 1:3:6, c.c in foundation 1:2:4 c.c in benching, 1/2 inch (13mm) thick cement plaster in 1:4 c.m. to all inside wall surfaces, channels and benching etc. and to top including providing and fixing cast iron foot rest at every foot of depth and making requisite number of main and branch channels complete but excluding the cost of excavation, back filing, disposal of excavated stuff manhole cover and frame. | 10 | Each |

Bill No. 1.3.1 Title:	Main Building

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Wiring for light, fan or call bell point with 3/.029, PVC insulated wire in 20mm dia PVC conduit recessed in the wall, column roof etc. including 1mm² single core PVC insulated wire as ECC as required. |  |  |
|  | Ground Floor | 118 | Point |
|  | First Floor | 145 | Point |
|  | Roof | 12 | Point |
| 2 | Wiring for plug point with 1.5mm² single core PVC insulated wire copper conductor in 20mm dia PVC conduit recessed in wall, column or roof etc including 1mm² single core PVC insulated wire as ECC as required. |  |  |
|  | Ground Floor | 50 | Point |
|  | First Floor | 47 | Point |
| 3 | Wiring for sub main with 2x2.5mm² PVC insulated wire 300/500 volts grade single core stranded copper conductor wire in 20mm dia PVC conduit recessed in the wall, column or roof etc including 1.5mm² PVC insulated wire single core wire as ECC as required. For power plug & Circuit wiring. |  |  |
|  | Ground Floor | 360 | Meter |
|  | First Floor | 392 | Meter |
|  | Roof | 5 | Meter |
| 4 | Wiring for sub main with 2x4mm² PVC insulated wire 300/500 volts grade single core stranded copper conductor wire in 20mm dia PVC conduit recessed in the wall, column or roof etc including 1.5mm² PVC insulated wire single core wire as ECC as required. For AC. |  |  |
|  | Ground Floor | 76 | Meter |
|  | First Floor | 282 | Meter |
| 5 | Wiring of power plug switch wired with 7/.036 PVC Insulated cabel run in 18 SWG black enamelled steel conduit recessed in walls, columns, ceilling, floor etc., including all accessories such as junction boxes, outets etc., as required and 14 SWG bare copper conductor all along the length of the conduit and controlled by one 15 Amps. 250 V MCB, cost including conduit and cable etc., but excluding the switch and MCB. | 1 | Point |
| 6 | Providing & fixing 10 Amps Polycarbonate flame retardant S.P switch with fancy "Ivory" gang plate fixed on die fabricated, powder coated, metal board recessed in the wall or column including connection as required. (MATRIX / SOK / TJ SERIES/TRUST make) |  |  |
|  | Ground Floor | 128 | Nos |
|  | First Floor | 155 | Nos |
|  | Roof | 11 | Nos |
| 7 | Providing & fixing 10 Amps Polycarbonate flame retardant Bell Push with fancy "Ivory" gang plate fixed on die fabricated, powder coated, metal board recessed in the wall or column including connection as required.(MATRIX / SOK / TJ SERIES/TRUST make) |  |  |
|  | Ground Floor | 1 | Nos |
| 8 | Providing & fixing 10 Amps Polycarbonate flame retardant 2-Pin socket with fancy "Ivory" gang plate fixed on die fabricated, powder coated, metal board recessed in the wall or column including connection as required.(MATRIX / SOK / TJ SERIES/TRUST make) |  |  |
|  | Ground Floor | 11 | Nos |
|  | First Floor | 12 | Nos |
| 9 | Providing & fixing 10 Amper Polycarbonate flame retardant universal/schuko socket with fancy "Ivory" gang plate fixed on die fabricated, powder coated, metal board recessed in the wall or column including connection as required. (MATRIX / SOK / TJ SERIES/TRUST make) |  |  |
|  | Ground Floor | 39 | Nos |
|  | First Floor | 35 | Nos |
| 10 | Providing & fixing 15 Amps Polycarbonate flame retardant 3-Pin switch socket unit with fancy gang plate fixed on die fabricated, powder coated metal board recessed in wall or column including connection as required. (Schneider / Philips) |  |  |
|  | Ground Floor | 2 | Nos |
|  | First Floor | 1 | Nos |
| 11 | Providing & fixing Polycarbonate flame retardant Click type Fan dimmer of all any sweep with fancy "Ivory" gang plate fixed on die fabricated, powder coated, metal board recessed in the wall or column including connection as required. (MATRIX / SOK / TJ SERIES/TRUST make) |  |  |
|  | Ground Floor | 15 | Nos |
|  | First Floor | 15 | Nos |
| 12 | Providing & fixing Polycarbonate flame retardant Data socket unit 8 contact cat-5 with fancy gang plate fixed on die fabricated, powder coated metal board recessed in wall or column including connection as required. (Schneider / Philips) |  |  |
|  | Ground Floor | 1 | Nos |
| 13 | Providing & fixing Polycarbonate flame retardant Telephone socket 4 contact single 2 line with fancy gang plate fixed on die fabricated, powder coated metal board recessed in wall or column including connection as required. (Schneider / Philips) |  |  |
|  | Ground Floor | 5 | Nos |
| 14 | Providing & fixing Polycarbonate flame retardant, double T.V Socket with plate fixed on die fabricated, powder coated, metal board recessed in the wall or column including connection as required. (Schneider / Philips) |  |  |
|  | Ground Floor | 1 | Nos |
| 15 | Providing & fixing 6 Amps plastic batten/Angle holder on PVC Round block/Round Cover fitted on surface including connection as required. |  |  |
|  | Ground Floor | 42 | Nos |
|  | First Floor | 63 | Nos |
|  | Roof | 12 | Nos |
| 16 | Providing & fixing 6 Amps plastic ceiling rose on PVC Round block/Round Cover fitted on surface including connection as required. |  |  |
|  | Ground Floor | 15 | Nos |
|  | First Floor | 15 | Nos |
| 17 | Providing and fixing plastic body electric buzzer on wooden block fitted on surface including connection. |  |  |
|  | Ground Floor | 2 | Nos |
| 18 | Providing & fixing 6 to 32 Amps, DP MCCB fixed on metal board recessed in the wall duly covered with fiber sheet, I/c inter-connection as required. |  |  |
|  | Ground Floor | 5 | Nos |
|  | First Floor | 16 | Nos |
| 19 | Providing & fixing Three phase 15 to 90 Amps, 400/440 volts energy meter fitted on existing board I/c connection as required. (Syed Brother, PEL) |  |  |
|  | Ground Floor | 1 | Nos |
| 20 | Providing, Installation, Testing & Commissioning of LED Tube / Batten Lights including driver having warm / Neutral White / cool white colour,80-90 lm/watts efficiency, 0.9 or above power factor, 10000-15000 hours life cycle, including connection, complete in all respect as approved by the Engineer Incharge. 4 Feet 36 Watts |  |  |
|  | Ground Floor | 34 | Nos |
|  | First Floor | 26 | Nos |
| 21 | Providing, Installation, Testing & Commissioning of LED Tube / Batten Lights including driver having warm / Neutral White / cool white colour,80-90 lm/watts efficiency, 0.9 or above power factor, 10000-15000 hours life cycle, including connection, complete in all respect as approved by the Engineer Incharge. 4 Feet 18 Watts |  |  |
|  | Ground Floor | 9 | Nos |
|  | First Floor | 21 | Nos |
| 22 | Providing, Installation, Testing & Commissioning of Flood Lights (IP66) including driver, having neutral white / cool white color, 90 - 110 lm/watts efficiency, 0.9 or above power factor, 20000-25000 hours life cycle, including connection, complete in all respect as approved by the Engineer Incharge. 110-120 Watts |  |  |
|  | Ground Floor | 14 | Nos |
| 23 | Providing, Installation, Testing & Commissioning of Flood Lights (IP66) including driver, having neutral white / cool white colour,90 - 110 lm/watts efficiency, 0.9 or above power factor, 20000-25000 hours life cycle, including connection, complete in all respect as approved by the Engineer Incharge. 90-100 Watts |  |  |
|  | Ground Floor | 3 | Nos |
| 24 | Providing & installing 140 Cm (56") sweep ceiling fan with blades, canopy, standard length of down rod including connection with 14.0076" flexible wire complete as required. (without regulator) Pak, Royal, GFC, Younas, Millat. |  |  |
|  | Ground Floor | 15 | Nos |
|  | First Floor | 15 | Nos |
| 25 | Providing & installing 40 Cm (16") sweep wall bracket fan complete with blades, regulator & pulling cord switch, fitted on wall or column with GI clip including connection as required. Pak, Royal, GFC, Younas, Millat. |  |  |
|  | Ground Floor | 3 | Nos |
|  | First Floor | 3 | Nos |
| 26 | Providing & installing 30 Cm (12") sweep, metallic body Exhaust fan complete with blades, motor, etc fitted in existing hole including connection with 14/.0076" flexible wire complete as required Pak, Royal, GFC, Younas, Millat. |  |  |
|  | Ground Floor | 8 | Nos |
|  | First Floor | 4 | Nos |
| 27 | Making hole in the wall for 12" to 16" sweep metallic exhaust fan and fixing of holefast, duly pilaster and making good the damage as required. |  |  |
|  | Ground Floor | 8 | Nos |
|  | First Floor | 4 | Nos |
| 28 | Providing & fixing 4½" X 4½" size MS die casted, Powder coated recessed type fan clamp box with 1/2" dia MS bar fan clamp fixed in roof at casting time as required. |  |  |
|  | Ground Floor | 15 | Nos |
|  | First Floor | 15 | Nos |
| 29 | Providing & fixing heavy gauge metallic shutter, suitable for 12" to 14" sweep exhaust fan's fitted on existing hole as required. |  |  |
|  | Ground Floor | 8 | Nos |
|  | First Floor | 4 | Nos |
| 30 | Providing and fixing battery operated Exit light good quality as approved by the Engineer Incharge as required. |  |  |
|  | Ground Floor | 2 | Nos |
|  | First Floor |  | Nos |
| 31 | Providing and fixing led bulb 12 w good quality on existing baten holder as required. |  |  |
|  | Ground Floor | 54 | Nos |
|  | First Floor | 3 | Nos |
|  | Roof | 12 | Nos |
| 32 | Providing & Fixing energy saving false ceiling type ceiling fan 18" (2' x 2') with plastic body good quality including connection complete as required. |  |  |
|  | Ground Floor | 2 | Nos |
|  | First Floor | 18 | Nos |

Bill No. 1.3.2 Title:	External Electrification

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing, Installation, Testing & Commissioning of Flood Lights (IP66) including driver, having neutral white / cool white colour,90 - 110 lm/watts efficiency, 0.9 or above power factor, 20000-25000 hours life cycle, including connection, complete in all respect as approved by the Engineer Incharge. 110-120 Watts |  |  |
|  | Ground Floor | 15 | Nos |
| 2 | Wiring for sub-main with 3x 50 mm² & 1x 35 mm², PVC insulated wire 450/750 volts grade, single core, stranded copper conductor wire in 40x40 mm uPVC duct fitted on surface including 6 mm² PVC insulated wire single core wire as ECC as required. |  |  |
|  | From GF MDB to FF DB | 5 | Metre |
| 3 | Providing & laying 3½ core70 mm², , PVC insulated & PVC sheathed, 600/1000 volts grade with stranded copper conductor , armoured underground, L.T cable buried in the ground dully covered with sand and warning cable tiles as per Pak. P.W.D specification as required. |  |  |
|  | From Energy Meter to GF Main DB. | 77 | Metre |
| 4 | Wiring for sub-main with 2 core, 4 mm², stranded copper conductor, PVC insulated & PVC sheathed cable i/c 1.5mm single core PVC insulated as ECC wire in 25 mm (1") dia high pressure ' D' class uPVC conduit buried in ordinary soil at the depth of 0.5 meter as per Pak. P.W.D Specifications as required. |  |  |
|  | Ground Floor | 14 | Nos |
| 5 | Providing and fixing earthing set with 2' x 2' x 1/8" copper plate burried in the ground at a depth of 12 feet or less if water comes out from the ground level (with salt and charcoal or earthing chemical powder) etc. making the pit 12 feet deep by excavation of all type of soyal (except soft or hard rock) including 8 swg copper wire in 1/2" GI conduit coplete in all respect as required. |  |  |
|  | Ground Floor | 2 | JOB |
| 6 | Providing and fixing 1" dia 3feet long lightning copper rod with 5 spikes and balls on its head as required. |  |  |
|  | Top of the Roof | 1 | Nos |
| 7 | Providing and fixing 1½" x 1/3" thick copper strip with suitable saddle at 12" intervals complete as required. |  |  |
|  | Top of the Roof & down to ECP | 12 | Metre |
| 8 | Providing & fixing centrifugal pumping set complete with AC 1 HP 2850/2900 RPM 220V 50 cycle motor with centrifugal pump having suction & discharge 1” x 1 ” coupled on a common base plate as approved by the engineer incharge as required. | 1 | Each |
| 9 | Providing & fixing Cubical type metal sheet Distribution board with locking arrangement as per drawing ,vermin protected duly powder coated paint i/c all fastening material including wiring with suitable gauge PVC x PVC wire complete as required. | 6 | Sft |
| 10 | Providing and fixing volt meter along with selector switch on existing all kinds of board with new one as per existing size and sample as required. | 1 | Nos |
| 11 | Providing and fixing Ampere meter CT operated 150-1000 A 96mm x 96mm (Cost 2249.28 + Labour charges 170 = 2759.28) | 1 | Nos |
| 12 | Providing and fixing rotery type 63/50A 3 phase phase selector switch fitted on all kind of existing board including incoming & out going connection as required. | 2 | Nos |
| 13 | Providing and fixing 60 A 500 volts 3 phase 4 pole change over switch fitted on all kind of existing board including incomming & out going connection as required. | 1 | Nos |
| 14 | Providing and fixing pilot lamp set fitted on existing board including connecting as required. | 3 | Set |
| 15 | Providing and fixing 6-32 A SP MCCB fixed on existing board including inter connection as required. | 42 | Nos |
| 16 | Providing and fixing 10-60 A (high breaking capacity) TP MCCB fixed on existing board including inter connection as required. | 1 | Nos |
| 17 | Providing and fixing 160-250 A adjustable (high breaking capacity) 4 pole circuit breaker fixed on existing board including inter connection as required. | 1 | Nos |
| 18 | DB First Floor Providing and fixing cubical type metal sheet distribution board with locking arrangement as per drawing vermin protected duely powder coated paint including all fastening material including wiring with suitable gauge PVC x PVC wire complete as required. | 4 | Sft |
| 19 | Providing and fixing pilot lamp set fitted on existing board including connecting as required. | 3 | Nos |
| 20 | Providing and fixing 6-32 A SP MCCB fixed on existing board including inter connection as required. | 47 | Nos |
| 21 | Providing and fixing 80-125 A adjustable (high breaking capacity) 4 pole circuit breaker fixed on existing board including inter connection as required. | 1 | Nos |

Bill No. 1.3.3	 Title:	Fire Alarm System

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing, fixing, testing and commissioning of microprocessor based 04 loop addressable fire control panel surface /recessed mounting type with full switching programming capabilities ,system suitable for operation at 230 volts 50hz with full functionality as per specification, drawing, etc. complete in all respect as approved by the Engineer Incharge |  |  |
|  | Ground Floor | 1 | Job |
| 2 | Providing, fixing, testing and commissioning of addressable optical smoke detector with addressable base with full functioning switching programming capabilities ,system as per specification & recommended of drawing / manufacturer with allied components, complete in all respect as approved by the Engineer Incharge |  |  |
|  | Ground Floor | 17 | Job |
|  | First Floor | 16 | Job |
| 3 | Providing and fixing testing and commissioning of analogue addressable ionization heat detector with addressable base ith full functioning switching programming capabilities system as per specification and recommended of drawing / manufacturer with allied components with T&P etc imported approve make / model or as approved by the Engineer Incharge as required. |  |  |
|  | Ground Floor | 1 | Job |
|  | First Floor | 1 | Job |
| 4 | Providing, fixing, testing and commissioning of manual call point break glass type with spare glass with allied components with full functionality as per specification of manufacturer/ drawing etc approved make or as approved by the Engineer in-charge as required. |  |  |
|  | Ground Floor | 2 | Job |
|  | First Floor | 2 | Job |
| 5 | Providing, fixing, testing and commissioning of addressable Wall Sounder with Flasher performing full functioning, switching programming capabilities ,system as per specification & recommended of drawing / manufacturer, complete in all respect as approved by the Engineer Incharge |  |  |
|  | Ground Floor | 2 | Job |
|  | First Floor | 2 | Job |

Bill No. 1.3.4	 Title:	CCTV System

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing DAHUA 16CH DVR with 2HD Supported H.265/H.264 codec decoding max 200Mbps incoming band width up to 8MP Resolution for Preview and play back complete or as approved by the Engineer Incharge as required. |  |  |
|  | Ground Floor | 1 | Job |
| 2 | Providing DHAU 2MP Camera max 30fps@1080p HD and SD output switchable 3.6mm fixied lens (6mm optional) max IR length 20m, smart IR IP67, DC 12V complete as required. |  |  |
|  | Ground Floor | 2 | Job |
| 3 | Providing LCD monitor 19" good quality. |  |  |
|  | Ground Floor | 1 | Nos |
| 4 | Providing Power Supply Unit. |  |  |
|  | Ground Floor | 2 | Nos |
| 5 | Providing BNC & DC connectors. |  |  |
|  | Ground Floor | 2 | Nos |
| 6 | Providing Hard Disk 1TB | 2 | Nos |
| 7 | RJ 6 video cable |  |  |
|  | Ground Floor | 20 | Meter |
| 8 | Installation charges / Camera. |  |  |
|  | Ground Floor | 2 | Nos |

Bill No. 1.3.5	 Title: Telephone System

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing and fixing 3-8 line Panasonic PABX System complete as required. |  |  |
|  | Ground Floor | 1 | Nos |
| 2 | Providing and fixing operator cansol set complete as required. |  |  |
|  | Ground Floor | 1 | Nos |
| 3 | Providing and fixing Panasonic telephone set with cli 7705. |  |  |
|  | Ground Floor | 2 | Nos |
| 4 | Providing and fixing Junction box with face plate. |  |  |
|  | Ground Floor | 1 | Nos |
| 5 | Wiring for Telephone system with 5 pair PVC copper conductor in 1" dia PVC conduit recessed in the wall, column or roof as required. |  |  |
|  | Ground Floor | 20 | Meter |

Bill No. 1.4.1	 Title: Sui Gas Internal

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Note:- All work concerning Sui Gass shall be executed in accordance with contract specification and requirements of the Sui Northern Gas Company by a Sub-contractor duly approved/licensed by the Sui Northern Gas Company. All work shall be tested and a test report submitted in accordance with the requirements of Sui Company. The contractor shall also be responsible for having the work cleared/approved from the Gas Company as and where necessary. |  |  |
| 1 | Providing, laying, fixing, jointing, testing commissioning Sui Gas G.I. (Good Quality) socketed pipe and specials, on surface including all cost of supports hangers, jointing material and painting with two coats of enamel paint complete in all respect to the requirement of S.N.G.P.L. |  |  |
|  | Note:- One extra union will be fixed along with every gas cock. |  |  |
|  | i): 2" dia ( 9 SWG) 50 mm | 210 | RFT |
|  | ii): 1½" dia (10 SWG) 40 mm | 50 | RFT |
|  | iii) 1" dia (10 SWG) 20 mm | 50 | RFT |
|  | iv) ¾" dia (12 SWG) 20 mm | 220 | RFT |
|  | v) ½" dia (12 SWG) 13 mm | 850 | RFT |
| 2 | Providing and fixing gun metal full way heavy quality gas cocks, Asia/Anwar/Baig with handle, complete as specified and approved. |  |  |
|  | i) ½" dia | 18 | No. |
|  | i) ¾" dia | 1 | No. |
|  | ii) 1" dia | 1 | No. |
| 3 | Providing, Installing, Testing & Commissioning of Gas Geysers 35 Gallons, complete in all respect, as approved by the Engineer Incharge | 2 | Each |
| 4 | Providing & fixing gas burner (Chullah) Double Burner (Stainless Steel) superior quality with rail cock i/c making gas connection as approved by the Engineer in-charge as required. | 2 | Each |

Bill No. 1.4.2	 Title: Sui Gas External Services

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing and laying commissioning MS Pipe 20mm dia (3/4”)high pressure including cleaning applying primer coating with BBES (bitumen binding enamel Surface) Compound with wrapping with Fiber Glass complete as lead in trance(excluding Excavation) | 380 | Rft |
| 2 | Providing and fixing gas regulator(043) as required | 1 | Each |
| 3 | Providing and fixing Audco Valve 20mm(3/4”) as required | 1 | Each |
| 4 | Providing and fixing gas Muller tee 20mm(3/4”) as required | 1 | Each |

Bill No. 2.1.1 Title: Demolishing & Earthwork

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Dismantling of block masonry in cement mortar (including, stacking salvaged material serviceable) and disposal of surplus material complete. | 90 | 100 Cft |
| 2 | Dismantling cement concrete work i/c disposing of the material complete. | 75 | 100 Cft |
| 3 | Excavation for raft foundations, underground tanks and septic tanks (rectangular or square) in all kinds of soil (except gravelly and murum soil wet silt, clay or mud, conglomeration of gravel and boulders, soft, sandy or disintegrated and hard rock) and back filling the excavated material all round the trenches including breaking clods, watering, consolidation by ramming in layers not exceeding 9 inches (229 mm) in depth to full compaction, dressing and disposal of surplus excavated stuff as directed, lead up to one chain (30.5 R.m) and lift upto 5 feet (1.52 m). | 1370 | 100 Cft |
| 4 | Supplying earth from approved outside sources within a radius of 5 miles (8 km) including digging, loading and unloading and filling in foundation trenches, plinth or under floor, etc. including breaking clods, dressing, watering and consolidation by ramming in layers not exceeding 9 inches (229 mm) in depth to full compaction complete within a ;lead of one chain (30.5 R.m) and lift of 5 feet (1.52 m). | 2651 | 100 Cft |

Bill No. 2.1.2 Title: Plain and Reinforced Concrete Work

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing and laying 1:4:8 (1 cement 4 sand and 8 coarse aggregate) cement concrete using crushed graded boulders (machine broken) in foundation including levelling, compacting and curing etc.complete. | 206 | 100 Cft |
| 2 | Providing and laying in situ 1:3:6 (1 cement 3 sand and 6 coarse aggregate) cement concrete using crushed graded boulders 3/4 inch (19 mm) and down gauge in foundation, basement and plinth including form work, compacting, curing and removal of form work etc. complete, foundation and basement up to 5 feet (1.52 m) depth and plinth up to 4 feet (1.2 m) height from ground level . | 281 | 100 Cft |
| 3 | Providing and laying reinforced cement concrete using screened graded bajri 3/4 inch (19 mm) and down gauge having a minimum works cube crushing strength of 4500 Ibs. per sq. inch (31.04 N/mm2 at 28 days) with a mix not leaner than 1:1:2 including form work and its removal, compacting and curing etc. complete but excluding the cost of reinforcement, in foundation basement and plinth |  |  |
|  | a)Footing | 167 | 100 Cft |
|  | b) U.G.W.T |  |  |
|  | U.G.W.T Walls | 36 | 100 Cft |
|  | U.G.W.T Slab | 7 | 100 Cft |
|  | c) Plinth Beam | 44 | 100 Cft |
|  | d) Column |  |  |
|  | Short Column | 50 | 100 Cft |
|  | Ground floor | 41 | 100 Cft |
|  | First Floor | 30 | 100 Cft |
|  | e) Beam |  |  |
|  | Ground floor | 5 | 100 Cft |
|  | First floor | 30 | 100 Cft |
|  | f) Lintel |  |  |
|  | Ground floor | 13 | 100 Cft |
|  | First floor | 10 | 100 Cft |
|  | Ground floor (Arch) | 19 | 100 Cft |
|  | G) Slab |  |  |
|  | Ground floor | 129 | 100 Cft |
|  | First floor |  |  |
|  | i) OrdinarySlab | 10 | 100 Cft |
|  | ii) Ordinary Cantilevered Slab | 10 | 100 Cft |
|  | iii) Sloping Slab | 10 | 100 Cft |
|  | h) Sun Shade Or Chajja |  |  |
|  | Ground floor | 10 | 100 Cft |
|  | First Floor | 21 | 100 Cft |
|  | i) Stair | 13 | 100 Cft |
|  | j) O.H.W.T |  |  |
|  | i) Wall | 121 | 100 Cft |
|  | ii) Bottom Slab | 69 | 100 Cft |
|  | iii) Top Slab | 44 | 100 Cft |
| 4 | Extra for using sulphate resisting cement in place of ordinary portland cement ..(Up to Plinth Level) | 583 | Cwt |
| 5 | Providing and laying reinforced cement concrete using screened graded bajri 3/4 inch (19 mm) and down gauge having a minimum works cube crushing strength of 2250 Ibs. per sq inch (15.52 N/mm2) at 28 days with a mix not leaner than 1:2:4 in precast spiral stairs case with railing 3 feet (0.9 m) dia with central post, nosing, rendering with cement slurry on all exposed surface including form work and its removal, compacting and curing etc. complete but excluding the cost of reinforcement, in basement plinth and ground floor. | 27 | 100Cft. |
| 6 | Providing and laying in situ 1:6 (1 cement 6 all in aggregate) cement concrete using screened graded bajri 3/4 inch (19 mm) and down gauge in foundation, basement and plinth including form work, compacting, curing and removal of form work etc. complete, till any floor. | 60 | 100Cft. |

Bill No. 2.1.3 Title: Reinforcement Work

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing and laying hard grade ribbed deformed (minimum yield point 60,000 psi) reinforcement bars with & including the cost of straightening, cutting, bending, binding, wastage, and such overlaps as are not shown in the drawings, placing in position on cement concrete 1:2:4 precast or MS chairs, tying with binding wire, cost of chairs and wires etc. in all kinds of RCC work in foundation, basement, plinth and ground floor of building including septic tanks and underground tanks and in projections for future extension |  |  |
|  | a) Ground Floor | 5280 | kg |
|  | b) First Floor | 500 | kg |

Bill No. 2.1.4	 Title: Block Masonry Work

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing and laying 1:3:6 cement concrete solid block masonry 4 to 6 inches (102 mm to 152 mm) thick using graded screened bajri 3/4 inch (19 mm) and down gauge set in cement mortar 1:4 including scaffolding, raking out joints and curing etc. complete in ground floor superstructure. |  |  |
|  | a) Ground Floor | 197 | 100 Cft |
|  | b) First Floor | 10 | 100 Cft |
| 2 | Providing and laying 1:3:6 cement concrete hollow block masonry of any thickness using graded screened bajri 1/2 inch (13 mm) and down gauge set in cement mortar 1:6 including scaffolding, raking, out Joints and curing etc. complete in basement and ground floor superstructure. |  |  |
|  | a) Ground Floor | 584 | 100 Cft |
|  | b) First Floor | 10 | 100 Cft |

Bill No. 2.1.5	 Title: Surface Rendering

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | 1/2" (13 mm) thick cement plaster 1:5 on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete. (For Internal Surface) |  |  |
|  | a) Ground Floor | 3410 | 100Sft |
|  | b) First Floor | 300 | 100Sft |
|  | c) O.H.W.T. | 233 | 100Sft |
| 2 | 1/2" (13 mm) thick cement plaster 1:3 on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete. (For Ceiling) |  |  |
|  | a) Ground Floor | 957 | 100Sft |
|  | b) First Floor | 50 | 100Sft |
| 3 | Providing 3/4" (19 mm) thick ornamental plaster with 1:3 cement mortar on walls, columns and slabs finished smooth in basement, plinth mezzanine and ground floor including curing etc., complete. |  |  |
|  | a) Ground Floor | 388 | 100Sft |
|  | b) First Floor | 50 | 100Sft |
| 4 | 3/4'' (19 mm) thick cement plaster 1:4 on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete. (For External Suface) |  |  |
|  | a) Ground Floor | 579 | 100Sft |
|  | b) First Floor | 200 | 100Sft |
|  | c) O.H.W.T. | 212 | 100Sft |
| 5 | Providing and applying 3/4" (19 mm) thick rough cast cement rendering consisting of 3/8" or 9.6 mm topping of 1:2 (one cement and two mixed graded bajri 3/8" or 9.5 mm and down gauge) and base of 3/8" or 9.5 mm thick rough plaster 1:4 (one cement four sand) in plinth and ground floor including finishing etc. complete. (Below Stone Facing) |  |  |
|  | a) Ground Floor | 463 | 100Sft |
|  | b) First Floor | 20 | 100Sft |
| 6 | 1-1/2" (38 mm) thick cement plaster 1:4 in two layers on walls and columns etc. in basement, plinth, mezzanine and ground floor including making edges, corners, and curing etc., complete. |  |  |
|  | a) Ground Floor | 745 | 100Sft |
|  | b) First Floor | 200 | 100Sft |
| 7 | Extra if deep cut grooves of approved sizes and design are made on plastered surface of any description in any floor | 288 | 100Rft |

Bill No. 2.1.6 Title:	Roofing & Waterproofing

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing and laying 2 inches (51 mm) thick damp proof course with cement concrete 1:2:4 cast in situ using graded screened bajri of 3/4 inch (19 mm) and down gauge including compacting, curing, form work and its removal etc. complete with applying a coat of hot bitumen (maxphalt 80/100 or equivalent) using 35 Lbs. per hundred square feet (1.71 Kg/sm) on damp proof course after complete drying and cleaning the surface . | 1062 | 100 Sft |
| 2 | Providing damp proof course on vertical surface with 3/4 inch (19 mm) thick cement plaster 1:4 mixed with any approved water proofing agent including curing etc. complete, but excluding the cost of water proofing agent.(ForO.H.W.T & U.G.W.T Internal Side) | 486 | 100 Sft |
| 3 | Providing a coat of bitumen emulsion at 10 Lbs. per % sft. (0.49 Kg/sm) on walls and floor in ground floor.(For Bath Floor) | 72 | 100 Sft |
| 4 | Providing and laying 1:2:4 cement concrete khuras 2" (51 mm) thick 2 ft. x 2 ft. (610 mm x 610 mm) size on roof including finishing with cement plaster 1:4, curing etc. complete. | 5 | Each |

Bill No. 2.1.7	 Title: Painting & Varnishing

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Painting three coats with weather shield paint deluxe (ICI) make of approved shade on plaster surface (External) and including the cost of cleaning the surface, sand papering etc. complete at any height in any floor. | 2200 | 100 Sft |
| 2 | Providing and Applying one Coat of Deluxe/Robbilac Primeon/Alkali Resisting Masonry Sealer and minimum three coats of Vinyl emulsion including surface preparaion,filling and rubbing etc,as per specification clause11.4,complete in all respect upto any height at any floor. | 4650 | 100 Sft |
| 3 | Applying French or spirit polishing, two coat of approved make on wood work at any height in any floor . | 350 | 100 Sft |

Bill No. 2.1.8	 Title: Flooring

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Supplying, stacking, spreading and filling stones of required size to required slope from approved quarries in plinth or under floor including filling voids with spawl and chips and consolidation by ramming layers not exceeding 9 inches (229 mm) in depth to full compaction lift upto 5 feet (1.52 m). | 390 | 100 Cft |
|  | Providing and laying 1:9 (1 cement 9 screened bajri) cement concrete using 3/4 inch (19 mm) and down gauge screened bajri in foundation including levelling, compacting and curing etc. complete . | 120 | 100 Cft |
|  | Providing and laying floors of 2 inches (51 mm) thick 1:2:4 cement concrete using graded screened bajri 3/4 inch (19 mm) and down gauge in ground floor laid in panels including form work, consolidation, finishing, and curing etc. complete.(For Under Floor) |  |  |
|  | a) Ground Floor | 859 | 100 Sft |
|  | b) First Floor | 50 | 100 Sft |
|  | Ditto item 117 above, but Extra for stairs steps (in treads, risers and mid-stair landings).. |  |  |
|  | a) Ground Floor | 92 | 100 Sft |
|  | b) First Floor | 10 | 100 Sft |
|  | Provide, lay, joint, cure, etc. 1” total thick glazed (glossy) ceramic tiles dado using “Master” (Supreme Quality) tiles of approved colour and shade in toilets, including ¼” thick neat cement buttering on back of tiles, remaining thickness made up with 1:2 cement sand rough base, complete “Size: 12”x18" |  |  |
|  | Ground Floor | 165 | 100 SFT |
|  | First Floor | 10 | 100 SFT |
|  | P/F flooring comprising 12” x 12” “Master” supreme quality non glazed skid free tiles of approved shade set in neat cement paste not exceeding ¼” thick and filling the joint with neat while cement slurry using matching colouring i/c ¾” thick base in 1:3 cement sand mortar laid over a bed of 1 ½” thick P.C.C 1:2:4 i/c cleaning & curing etc, complete. |  |  |
|  | Ground Floor | 970 | 100 SFT |
|  | First Floor | 10 | 100 SFT |
|  | Provide, lay, level, repair, cure, etc., total 7/8" thick terrazzo dado on walls and on floor in Over Head water tank with 1:2 (one part cement and two part of white Marble chips) over ½" thick cement sand (1:3) rough plaster base, including curing etc., complete, terrazzo to be 3/8" thick but without grinding polishing. | 550 | 100 SFT |
|  | Providing and Laying Flooring (Glazed) Comprising 16"x 16" 3/8" (1600 cm) available thickness Granitto tiles (Mater) fixed in neat cement buttering laid over 1-1/4" thick PCC 1:2:4 using 3/8" down course aggregates to final levels and stop including filling joints with ready made approved fillers of suitable colours complete any floor at any height as directed. | 860 | 100 SFT |
|  | Providing and laying Khaprail tiles C.C 1:2:4: in different design laid on sloppy slab & shades laid in cement mortar 1:2 in Joints and using etc. as per directions of the Engineer incharge. | 620 | Sq.ft. |
|  | Providing and laying 1/2" (13mm) thick marble patties (panel strips) up to 1-1/2" (38 mm) depth in floor and dado of any description in all floor. | 320 | Rft. |

Bill No. 2.1.9 Title:	Carpentry & Joinery

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Providing and fixing best quality deodar frames for doors. windows, ventilators, clerestory windows, shelves, partitions, trellis work, etc., as required . | 20 | Cft. |
|  | Providing and fixing 1-1/2 inch (38 mm) thick best quality deodar wood shutters fully paneled with same wood, approved iron hinges and tower bolts, etc., as required.. | 27 | Sq.ft. |
|  | Providing and fixing 1-1/2 inches (38 mm) thick pressed veneered door shutters fully flushed with commercial ply wood veneering on all faces and sides fixed over deodar wood cavitied core and frame work of not less than 4 inches (102 mm) wide strip alround with approved brass hinges and tower bolts etc., as required . | 135 | Sq.ft. |
|  | Providing and fixing on frames etc., with sunken iron screws best quality deodar wood moulding, beading, architrave etc., of approved design over one square inch (6.45 Sqm.) and up-to 2 square inches (13 Sqm.) in cross section as required . | 120 | Rft. |
|  | Providing and fixing with brass screws door lock handle type of approved design (Hungry make), cutting wood etc., to required shape and size with two operating keys as per direction of the Engineer-in-Charge. | 8 | Each |
|  | Providing and fixing with brass screws or specially supplied screws hydraulic door closer branno of approved design No. 3 (Yugoslavia make), cutting wood etc., to required shape and size as per direction of Engineer-in-Charge. | 8 | Each |
|  | Providing and fixing double glazed Champagne anodized aluminum Sliding windows as per British standard manufactured by Luckh, Alcop, Krudson, Pakistan Cables and A.C.P. (fixing through their approved fabricators), Executive model section dubble or single glazed 101mm x 37mm and 2mm thick including the cost of aluminum netting ,fitting, with all accessories cutting hole etc. and making good damages to walls etc. complete as required in any floor as per direction of engineer-in-charge, including the cost of Tinted Glass 5mm thick (Pakistani). (For Windows & Ventilator) | 132 | Sq.ft. |
|  | Providing and fixing 1-1/2 inches or 38 mm thick (3/4 inch or 19 mm thick battens 3/4 inch or 19 mm thick ledges and baacing) best quality deodar wood shutters battened, ledged and braced, making rebated or tongued and grooved joints with approved T-iron hinges and tower bolts as required | 48 | Sq.ft. |
|  | Providing and fixing approved brass drawer and cupboard lock with brass screws including two operating keys: 2-3/4" (70 mm) size (4 levers) | 3 | Each |

Bill No. 2.1.10 Title: Miscellaneous

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Providing and fixing plain polyvinyl chloride (PVC) water stops 8" (203 mm) wide in vertical or horizontal expansion joints including cutting and jointing etc. complete in all floors. | 5 | Rft. |
|  | Providing and fixing Gypsum board 2' x 2' x 3/4" tiles ceiling including Aluminum T & L angle 1" x 1" i.c hanger clips jointing clips and G.I Wire etc complete as required in any floor. | 95 | Sq.ft. |
|  | Providing and fixing iron grills with required section of flat iron as per approved design including welding all sides of the section at the junction, and fixing with sunk iron screws, painting two coats of red oxide paint etc. complete as required in any floor in masonry or concrete work. | 280 | Kg |
|  | Providing & fixing C.I Cover & Frame 24" dia (610 mm) weight 2 cwt 2 Qtr (127.00 Kg.) | 2 | Each |
|  | Providing & fixing C.I Cover & Frame 24" dia (610 mm) weight 2 cwt 2 Qtr (127.00 Kg.) | 2 | Each |
|  | Providing Anti -termite treatment by spraying /sprinkling /spreading Heptachlor 0.5% Emulsion as an overall pre -construction treatment both vertically and horizontally anti-termite shall be three time excavation base, side & plinth area etc. measur shall be only one time plinth area complete and as per instructions of engineer in charge. | 304 | Sq.ft. |
|  | Providing & fixing of approved quality in-ward openable sound proof uPVC doors in white colour made from 60mm x 60mm frame and 60mm x 100mm sash double glazed with 5mm thick tempered glass with 8mm air space between the two glass panels vacuumed with argon gas with best quality door hinges and door lock etc complete in all respects as per site requirements, specifications, instructions of the manufacturer and as directed by the Engineer In-charge.(For Bathroom) | 60 | Sq.ft. |
|  | Providing and fixing S.S. pipe railing, comprising, vertical posts of 1-1/2" x 1-1/2'' S.S Square tube and horizontal bracing of 3/4'' x 3/4'' S.S. Square tube 2'' at top as per design including cost of specials, bends, threading, cutting and making good damages on the floor or wall of any kind etc. at any floor. complete as per instruction of the Engineer-in-Charge | 77 | Rft |
|  | Preparing the surface and painting with matt finish including rubbing the surface with bathy (Silicon carbide rubbing brick) filling the voids with zink/chalk/plaster of paris mixture, applying first coat premix making the surface smooth and then painting 3 coats with matt finish of approved make etc: complete | 4882 | Sq.ft. |
|  | Providing & Fixing Lasbela Stone 3/4" machine cut resize as requred including sand blasting at elevation, fixing with High strength bond at any height of building etc complete, design and pattern as per approved drawings and instructions of engineer in charge. | 628 | Sq.ft. |
|  | Making cement plaster pattas /band around straight or curved openings and around the edges of windows and opening with fine finishes as directed by engineer in charge. | 67 | Rft |
|  | Providing and laying approved Quality Marble slab polished ( Light Gray Marble Slab) Required Size Tread on laid over 7/8 inch (22 mm) thick cement mortar 1:2 setting the tiles with Bound over cement mortar, jointing the tiles with white cement slurry and nosing including curing, etc.complete (Imported/Local-origin China, Italy or equivalent) all as specified.& Instruction by engineer in charge | 85 | Sq.ft. |
|  | Providing and laying approved Marble slab) polished ( Light Gray Marble Slab) Required Size on Riser laid over 3/8 inch (10 mm) thick cement mortar 1:2 setting the tiles with Bound over cement mortar, jointing the tiles with white cement slurry and nosing including curing, etc.complete (Imported/Local-origin China, Italy or equivalent) all as specified.& Instruction by engineer in charge | 36 | Sq.ft. |
|  | Providing and laying approved Marble slab Required Size on counter tops laid over 3/4 inch (20 mm) thick cement mortar 1:2 setting the tiles with Bound over cement mortar, jointing the tiles with white cement slurry and nosing including curing, etc.complete (Imported/Local-origin China, Italy or equivalent) all as specified.& Instruction by engineer in charge | 42 | Sq.ft. |
|  | Providing and laying approved Marble slab Required Size on Venity tops laid over 3/4 inch (20 mm) thick cement mortar 1:2 setting the tiles with Bound over cement mortar, jointing the tiles with white cement slurry and nosing including curing, etc.complete (Imported/Local-origin China, Italy or equivalent) all as specified.& Instruction by engineer in charge | 16 | Sq.ft. |

Bill No. 2.1.11 Title: Water Supply, Sanitary, Plumbing Fixtures & Sewerage

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Providing G.I. pipes, specials and clamps etc., including fixing cutting and fitting complete with and including the cost of breaking through walls and roof, making good etc., painting 2 coats after cleaning the pipe etc.; with white zinc paint with pigment to match the colour of the building and testing with water to a pressure head of 200 feet (61 M) and cartages within 10 miles (16.09 km.) (ILLM). |  |  |
|  | a)1-1/2 inches (40mm) dia | 140 | R.ft |
|  | b)2 inches (50mm) dia | 50 | R.ft |
|  | c) 3 inches (75mm) dia | 12 | R.ft |
|  | Providing and fixing ball valve (with PVC Ball) made to B.S.S. 1212..... |  |  |
|  | a)1 inches (25mm) dia | 7 | Each |
|  | b)1-1/2 inches (40mm) dia | 1 | Each |
|  | c)2 inches (50mm) dia | 2 | Each |
|  | d)3 inches (75mm) dia | 1 | Each |
|  | Providing and fixing cast iron foot valve with flanged ends complete with a strainer suitable for fixing to 2 inch (50 mm) dia. suction pipe including 1/8 inches (3.2 mm) thick rubber washer… | 1 | Each |
|  | Providing and fixing gun-metal non-return valves with male and female ends as required. 2 inches (50 mm) dia.. | 1 | Each |
|  | Providing and fixing Pakistani best quality European style white glazed earthenware W.,C pan complete with and including the cost of plastic seat and buffers, 13.6 liters best quality low level plastic flushing cistern with internal fittings complete, P.V.C flushing pipe with fittings and making requisite number of holes in walls, plinth and floor for pipe connections and making good in cement concrete 1:2:4. | 2 | Each |
|  | Providing and fixing bath room accessories of set of 6 pieces consist of one shelf, one towel rod with bracket, one towel ring, one soap dish, one tooth Brush holder with glass, one tissue paper holder one double hook etc complete of approved quality as per direction of Engineer in-charge. | 2 | Each |
|  | Providing and fixing 20 inches x 16 inches (508 mm x 406 mm) Looking mirror of Belgium glass complete with Plastic frame and C.P. Brass screws. | 2 | Each |
|  | Fixing floor traps of sizes with gratings including cutting and making the requisite number of holes in walls, plinth and floors and making good in 1:2:4 C.C.. | 8 | Each |
|  | Providing 1/2 inch (15 mm) dia plastic connection complete with a 1/2 inch (15 mm) dia brass stop cock two brass nuts and lining jointed to plastic pipe. | 8 | Each |
|  | Providing and fixing approved quality stainless steel sink 33” x 18” Pak made (Atlas) complete with brass oxidized bolt kit/angle iron brackets built into walls ½” dia CP sink mixer 1-1/4” rubber plug and CP brass chain 1-1/4” CP brass waste 1-1/4” dia malleable iron or CP brass bottle trap with malleable iron or brass unions and making requisite number of holes in walls, plinth and floor for pipe connections and making good in cement concrete 1:2:4 | 2 | Each |
|  | Providing and fixing standing wall shower of CP brass 3 knobs of approved quality mixer unit and moveable shower head complete.. | 2 | Each |
|  | Providing and fixing M.S. clamps of the approved design to 3 inches (80mm) dia C.I. pipe sockets including the cost of cutting and making good to wall or M.S. Bolts and nuts built 3 inches (80mm) into walls including pipe distance pieces and extra painting to match the colour of the building and cartage within 10 miles. (16.09 Km) | 4 | Each |
|  | Providing and fixing M.S. clamps of the approved design to 4 inches (100 mm) dia C.I. pipe spun sockets including the cost of cutting and making good to wall or M.S. bolts and nuts built. 4 inches (100 mm) in to wall including pipe distance pieces, extra painting to match the colour of the building and cartage within 10 miles. (16.09 km.).. | 8 | Each |
|  | Providing and fixing 3/4 inches dia. (15 mm) brass bib cocks. | 2 | Each |
|  | Providing and fixing approved quality earthen ware glazed vanity basin standard size (White/light colour) and including the cost of 1-1/4” dia brass waste 1-1/4” rubber plug ,chromium plated mixer tap 1/2 inch (15 mm) dia and chrome plated brass chain PVC waste pipe 1-1/4” dia of approved pattern etc. complete. | 2 | Each |
|  | P/Fixing PPR pipe of dadex i/c testing, jointing and specials inside clear dia 3/4" complete as approved by Engineer. |  |  |
|  | a)3/4 inches (20mm) dia | 11 | R.ft |
|  | b)1 inches (25mm) dia | 159 | R.ft |
|  | Supplying & fixing Muslim Shower with double Bib cock & ring Pipe etc complete Quality like, (Master, Sonex or equivalent) | 3 | Each |
|  | Providing and fixing upvc Soil and Waste Pipe with specials and clamps including fixing, cutting and fitting including the cost of breaking through walls and roofs etc complete and as per instructions of engineer in charge. 3 inches (75mm) dia | 82 | Rft |
|  | Providing and fixing upvc Soil and Waste Pipe with specials and clamps including fixing, cutting and fitting including the cost of breaking through walls and roofs etc complete and as per instructions of engineer in charge. 4 inches (110mm) dia | 50 | Rft |
|  | Providing and fixing uPVC Vent Pipe including cutting and fitting including the cost of painting 2 coats after cleaning the pipe with white zinc paint with pigment to match the colour of the building in any floor etc. complete and as per instructions of engineer in charge. 4 inches (110mm) dia | 15 | Rft |
|  | Providing and fixing upvc Elbow (90 degree) including Z joint and Rubber ring etc complete and as per instructions of engineer in charge. 3 inches (75mm) dia | 8 | Each |
|  | Providing and fixing upvc Elbow (90 degree) including Z joint and Rubber ring etc complete and as per instructions of engineer in charge. 4 inches (110mm) dia | 4 | Each |
|  | Providing and fixing upvc Yee Tee including Z joint and Rubber ring etc complete and as per instructions of engineer in charge. 3 inches (75mm) dia | 10 | Each |
|  | Providing and fixing upvc Yee Tee including Z joint and Rubber ring etc complete and as per instructions of engineer in charge. 4 inches (110mm) dia | 4 | Each |
|  | Providing and fixing upvc Elbow with door (90 degree) including Z joint and Rubber ring etc complete and as per instructions of engineer in charge. 3 inches (75mm) dia | 6 | Each |
|  | Providing and fixing upvc Elbow with door (90 degree) including Z joint and Rubber ring etc complete and as per instructions of engineer in charge. 4 inches (110mm) dia | 5 | Each |

Bill No. 2.2.1 Title: Wiring

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1. | Wiring for light, fan or call bell point with 3/.029, PVC insulated wire in 20mm dia PVC conduit recessed in the wall, column roof etc. including 1mm² single core PVC insulated wire as ECC as required. |  |  |
|  | Ground Floor | 30 | Point |
|  | First Floor | 2 | Point |
| 2. | Wiring for plug point with 1.5mm² single core PVC insulated wire copper conductor in 20mm dia PVC conduit recessed in wall, column or roof etc including 1mm² single core PVC insulated wire as ECC as required. |  |  |
|  | a) Ground Floor | 14 | Point |
|  | b) First Floor | 1 | Point |
| 3. | Wiring for sub main with 2x2.5mm² PVC insulated wire 300/500 volts grade single core stranded copper conductor wire in 20mm dia PVC conduit recessed in the wall, column or roof etc including 1.5mm² PVC insulated wire single core wire as ECC as required. For power plug & Circuit wiring. |  |  |
|  | a) Ground Floor | 90 | Meter |
|  | b) First Floor | 2 | Meter |
| 4. | Wiring for sub main with 2x4mm² PVC insulated wire 300/500 volts grade single core stranded copper conductor wire in 20mm dia PVC conduit recessed in the wall, column or roof etc including 1.5mm² PVC insulated wire single core wire as ECC as required. For AC. |  |  |
|  | a) Ground Floor | 38 | Meter |
| 5. | Wiring of power plug switch wired with 7/.036 PVC Insulated cabel run in 18 SWG black enamelled steel conduit recessed in walls, columns, ceilling, floor etc., including all accessories such as junction boxes, outets etc., as required and 14 SWG bare copper conductor all along the length of the conduit and controlled by one 15 Amps. 250 V MCB, cost including conduit and cable etc., but excluding the switch and MCB. | 1 | Point |

Bill No. 2.2.2 Title: Accessories (Clipsal & Switchkids Make)

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing and fixing 10 A poly corbonate flame retardent SP switch with fancy required gang plate fixed on difabricated powder coated metal board recessed in the wall or column including connection as required.(E31-36/1/2a). |  |  |
|  | Ground Floor | 32 | Nos |
|  | First Floor | 2 | Nos |
| 2 | Providing and fixing 10 A poly corbonate flame retardent bell push with fancy gang plate fixed on difabricated powder coated metal board recessed in the wall or column including connection as required.(E31MBPR). |  |  |
|  | Ground Floor | 1 | Nos |
| 3 | Providing and fixing 10 A poly corbonate flame retardent tow pin switch socket unit (universal) with fancy gang plate fixed on difabricated powder coated metal board recessed in the wall or column including connection as required.(E15u). |  |  |
|  | Ground Floor | 4 | Nos |
|  | First Floor | 1 | Nos |
| 4 | Providing and fixing 13 A poly corbonate flame retardent three pin switch socket unit with fancy gang plate fixed on difabricated powder coated metal board recessed in the wall or column including connection as required.(E15). |  |  |
|  | Ground Floor | 10 | Nos |
|  | First Floor | 1 | Nos |
| 5 | Providing and fixing 15 A poly corbonate flame retardent three pin switch socket unit with fancy gang plate fixed on difabricated powder coated metal board recessed in the wall or column including connection as required. |  |  |
|  | Ground Floor | 3 | Nos |
| 6 | Providing and fixing 500w light dimmer polly corbonate flame retardent with fancy gang plate fixed on difabricated powder coated metal board recessed in wall, column including connection as required. |  |  |
|  | Ground Floor | 4 | Nos |
|  | First Floor | 1 | Nos |
| 7 | Providing and fixing polly corbonate flame retardent data socket unit 8 contect cat 5 with fancy gang plate fixed on difabricated powder coated metal board recessed in wall, column including connection as required. |  |  |
|  | Ground Floor | 1 | Nos |
| 8 | Providing and fixing polly corbonate flame retardent telephone socket unit 4 contect single two lines with fancy gang plate fixed on difabricated powder coated metal board recessed in wall, column including connection as required.(E31 RJ 64SM). |  |  |
|  | Ground Floor | 1 | Nos |
|  | First Floor | 1 | Nos |
| 9 | Providing and fixing polly corbonate flame retardent double TV socket with plate fixed on difabricated powder coated metal board recessed in wall, column including connection as required. |  |  |
|  | Ground Floor | 2 | Nos |
| 10 | Providing and fixing 6 a plastic batten / angle holder on pvc round block / cover fitted on surface including connection. |  |  |
|  | Ground Floor | 11 | Nos |
|  | First Floor | 1 | Nos |
| 11 | Providing and fixing 6 a plastic ceiling rose on pvc round block / cover fitted on surface including connection. |  |  |
|  | Ground Floor | 4 | Nos |
|  | First Floor | 1 | Nos |
| 12 | Providing and fixing plastic body electric buzzer on wooden block fitted on surface including connection. |  |  |
|  | Ground Floor | 1 | Nos |
| 13 | Providing and fixing 6 - 32 A DP circuit breaker fixed on surface on wooden board duely covered with fiber sheet including inter connection as required. (For AC). |  |  |
|  | Ground Floor | 4 | Nos |
|  | First Floor | 1 | Nos |
| 14 | Providing and fixing three phase 15 to 90 A 400/440 volts energy meter fitte on existing board including connection as required. (Syed Brothers, PEL). |  |  |
|  | Ground Floor | 1 | Nos |

Bill No. 2.2.3 Title: Light Fittings & Fixtures (Philips, Sunlight, & Paklite Make)

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing and fixing basic batten surface type fitting complete with prismatic defuser 1x36 w white day light lamp and electromagnatic blast as philips cat no GMP 015/136. |  |  |
|  | Ground Floor | 4 | Nos |
| 2 | Providing and fixing surface mounting luminair with m1 type metalic louver TLD 4 x 18w white day light lamp electronic blast complete as philips cat no TCS418 M1 |  |  |
|  | Ground Floor | 1 | Nos |
| 3 | Providing and fixing well finished aluminum casted base glass cover water tide bulk head fitting complete eith brass holder and 60w clear lamp including connection as required. |  |  |
|  | a) Ground Floor | 7 | Nos |
|  | b) First Floor | 2 | Nos |
| 4 | Providing and fixing aluminum casted stove enamilled painted gate light fitting complete with glass brass holder and 100 w clear bulb (as approved by the Engineer Incharge) including connection as required. |  |  |
|  | Ground Floor | 2 | Nos |
| 5 | Providing and installing 140 cm (56") sweep ceiling fan with blades canopy standard length of down rod including connection with 14/.0076" flexible wire complete as required ( with out regulator) Millat / Pak / Asia. |  |  |
|  | a) Ground Floor | 4 | Nos |
|  | b) First Floor | 1 | Nos |
| 6 | Providing and installing 40 cm (16") sweep wall bracket fan complete with blades regulator and pulling cord fitted on wall or column with GI clip including connection as required Millat / Pak / Asia. |  |  |
|  | a) Ground Floor | 1 | Nos |
|  | b) First Floor | 1 | Nos |
| 7 | Providing and installing 30 cm (12") sweep metalic body exhaust fan complete with blades motor etc fitted in existing hole including connection with 14/.0076" flexible wire complete as required Millat / Pak / Asia. |  |  |
|  | a) Ground Floor | 3 | Nos |
|  | b) First Floor | 1 | Nos |
| 8 | Making hole in the wall for 12"-16" exhaust fan and fixing of hole fast duely plastered amd making good the damages as required. |  |  |
|  | a) Ground Floor | 3 | Nos |
|  | b) First Floor | 1 | Nos |
| 9 | Providing and fixing 4.5" x 4.5" size ms dicasted power coated recessed fan clamp box with ½" dia ms bar fan clamp fixed in roof at casting time as required. |  |  |
|  | a) Ground Floor | 4 | Nos |
|  | b) First Floor | 1 | Nos |
| 10 | Providing and fixing heavy gauge metallic shutter suitable for 12"-14" sweep exhaust fan fitted on existing hole as required. |  |  |
|  | a) Ground Floor | 3 | Nos |
|  | b) First Floor | 1 | Nos |
| 11 | Providing and fixing led bulb 12 w good quality on existing baten holder as required. |  |  |
|  | a) Ground Floor | 11 | Nos |
|  | b) First Floor | 1 | Nos |

Bill No. 2.2.4 Title: Earthing & Lightning Protection System

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing and fixing earthing set with 2' x 2' x 1/8" copper plate burried in the ground at a depth of 12 feet or less if water comes out from the ground level (with salt and charcoal or earthing chemical powder) etc. making the pit 12 feet deep by excavation of all type of soyal (except soft or hard rock) including 8 swg copper wire in 1/2" GI conduit complete in all respect as required. |  |  |
|  | Ground Floor | 1 | JOB |
| 2 | Providing and fixing 1" dia 3feet long lightning copper rod with 5 spikes and balls on its head as required. |  |  |
|  | Top of the Roof | 1 | Nos |
| 3 | Providing and fixing 1½" x 1/3" thick copper strip with suitable saddle at 12" intervals complete as required. |  |  |
|  | Top of the Roof & down to ECP | 12 | Meter |
| 4 | Providing and fixing centrifugal pumping set complete with AC 1 HP 2900 RPM 220v 50Cycle motor with centrifugal pump JD/B model having section & discharge 1”x1” coupled on as common base plate as approved | 1 | Each |

Bill No. 2.2.5 Title: DB Work

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Main DB + Ground Floor DB |  |  |
|  | Providing and fixing cubical type metal sheet distribution board with locking arrangement as per drawing vermin protected duely powder coated paint including all fastening material including wiring with suitable gauge PVC x PVC wire complete as required. Main Distribution Board + DB Ground Floor. | 3 | Sft |
|  | Providing and fixing Volt meter 0-500 volts 96mm x 96mm (Cost 506.25 + Labour charges 17 = 523.25) | 1 | Nos |
|  | Providing and fixing Ampere meter CT operated 150-1000 A 96mm x 96mm (Cost 506.25 + Labour charges 17 = 523.25) | 1 | Nos |
|  | Providing and fixing rotery type 63/50A 3 phase phase selector switch fitted on all kind of existing board including incoming & out going connection as required. | 1 | Nos |
|  | Providing and fixing 10-60 A (high breaking capacity) TP MCCB fixed on existing board including interconnection as required. | 2 | Nos |
|  | DB First Floor |  | Sft |
|  | Providing and fixing cubical type metal sheet distribution board with locking arrangement as per drawing vermin protected duely powder coated paint including all fastening material including wiring with suitable gauge PVC x PVC wire complete as required. |  |  |

Bill No. 2.2.6 Title: Fire Alarm System

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing and fixing testing and commissioning of micro processeor base 04 loop analogue addressable and 2 wire control panel surface / Recessed mounting type with full switching programming capabilities system suitable operation of 230 volts 50 HZ with full functionality as per specification of manufacturer allied components. with T&P etc and as per specific drawing imported approve make/ Model or as approved by the Engineer Incharge as required. |  |  |
|  | Ground Floor | 1 | Job |
| 2 | Providing and fixing testing and commissioning of analogue addressable ionization smoke detector with addressable base ith full functioning switching programming capabilities system as per specification and recommended of drawing / manufacturer with allied components with T&P etc imported approve make / model or as approved by the Engineer Incharge as required. |  |  |
|  | Ground Floor | 4 | Job |
| 3 | Providing and fixing testing and commissioning of analogue addressable ionization heat detector with addressable base ith full functioning switching programming capabilities system as per specification and recommended of drawing / manufacturer with allied components with T&P etc imported approve make / model or as approved by the Engineer Incharge as required. |  |  |
|  | Ground Floor | 1 | Job |
| 4 | Providing and fixing testing and commissioning addressable manual break glass unit with addressable base with full functioning switching programming capabilities system as per specification and recommended of drawing/ manufacturer with allied component with T&P etc imported approve make / model or as approved by the Engineer Incharge as required. |  |  |
|  | Ground Floor | 1 | Job |
| 5 | Providing and fixing testing and commissioning addressable electronic sounder unit with addressable base with full functioning switching programming capabilities system as per specification and recommended of drawing/ manufacturer with allied component with T&P etc imported approve make / model or as approved by the Engineer Incharge as required. |  |  |
|  | Ground Floor | 1 | Job |

Bill No. 2.2.7 Title: CCTV System

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing DAHUA 16CH DVR with 2HD Supported H.265/H.264 codec decoding max 200Mbps incoming band width up to 8MP Resolution for Preview and play back complete or as approved by the Engineer Incharge as required. |  |  |
|  | Ground Floor | 1 | Job |
| 2 | Providing DHAU 2MP Camera max 30fps@1080p HD and SD output switchable 3.6mm fixied lens (6mm optional) max IR length 20m, smart IR IP67, DC 12V complete as required. |  |  |
|  | Ground Floor | 2 | Job |
| 3 | Providing LCD monitor 19" good quality. |  |  |
|  | Ground Floor | 1 | Nos |
| 4 | Providing Power Supply Unit. |  |  |
|  | Ground Floor | 2 | Nos |
| 5 | Providing BNC & DC connectors. |  |  |
|  | Ground Floor | 2 | Nos |
| 6 | Providing Hard Disk 1TB | 2 | Nos |
| 7 | RJ 6 video cable |  |  |
|  | Ground Floor | 15 | Metre |
| 8 | Installation charges / Camera. |  |  |
|  | Ground Floor | 2 | Nos |

Bill No. 2.2.8 Title: Telephone System

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
| 1 | Providing and fixing 3-8 line Panasonic PABX System complete as required. |  |  |
|  | Ground Floor | 1 | Nos |
| 2 | Providing and fixing operator cansol set. |  |  |
|  | Ground Floor | 1 | Nos |
| 3 | Providing and fixing Panasonic telephone set with cli 7705. |  |  |
|  | Ground Floor | 1 | Nos |
| 4 | Providing and fixing Junction box with face plate. |  |  |
|  | Ground Floor | 1 | Nos |
| 5 | Wiring for Telephone system with 5 pair PVC copper conductor in 1" dia PVC conduit recessed in the wall, column or roof as required. |  |  |
|  | Ground Floor | 12 | Metre |

Bill No. 2.3 Title: Sui Gas

| Item. # | Description | Quantity | Unit of Measurement |
| --- | --- | --- | --- |
|  | Providing & fixing low pressure gas G.I pipe 13mm(1/2”) dia (medium)with special such as bend tee, union, and socket etc. as required | 143 | Rft |
|  | Providing and fixing brass valve ball type screw both ends complete with steel handle washer/nut 15mm(1/2”)dia as required | 5 | Each |
|  | Providing and installation gas geyser 30 gallons capacity as approved by the engineer incharge lao consumption fully automatic thermostat control (foreign make) having on pilot holder and one large burner both operated on gas i/c making connection | 1 | Each |
|  | Providing and fixing double gas burner (Chullah) Superior Quality made of stainless steel with rail cocks i/c making gas connection as approved by the engineer in charge as required | 2 | Each |
|  | Providing & fixing Gas Room Heater with single Burner (Nas Gas (DG-2001) with all accessories, gas inlet connection, gas cock etc., and fixing G.I. sheet 26 Gauge flue of appropriate size and design complete in all respect as per direction of the Engineer Incharge including testing/commissioning, complete. | 5 | No. |

Notes for Bidders

      - Cost estimation of above Bill of Quantities has been done using rates of Scheduled Items derived by Pakistan Public Works Department under its CSR 2022, whereas Non-scheduled Items have been costed based on Market Survey.

      - The Employer has earmarked Provisional Sum(s) of Pak. Rs. 4.00 million for the execution of the assignment against unforeseen/additional work items, in addition to the cost that a bidder will offer in its Financial Proposal/Bid. Provisional Sums included and so designated in the Bill of Quantities which shall be included in the overall value of the contract, expended in whole or in part at the direction and discretion of the Employer in accordance with Sub-Clauses 13.4 and 13.5 of the General Condition of the Contract.

      - The Provisional Sum(s) shall be at sole discretion of the Employer and the Contractor shall have no legal claim of Provisional Sum(s) that shall be added in the contract’s overall value as separate line item. It may not be necessary that Provisional Sum(s) would be used at all. In case of any unutilized provision of Provisional Sum(s), the same shall be considered subtracted from the contract’s amount/provision.

      - The Employer and the Contractor shall work out the valuation of the unforeseen/additional work items based on the item rates as per CSR 2022, derived by Pakistan Public Works Department Scheduled Items, if no corresponding rate is available in the contract. Whereas for Non-scheduled Items for which there shall be no rate available in the contract, the rate shall be drawn by employing Market Survey using at least three sources and adopting most economical costs. The commercial and technical specifications of such unforeseen/additional shall invariably be defined by the Employer.

      - Soft Copy of the Bill of Quantities has been uploaded at the website of the University (www.aiou.edu.pk) which the bidders may download for their ease.

Technical Proposal

- Site Organization

- Method Statement

- Mobilization Schedule

- Construction Schedule

- Equipment

Site Organization

Method Statement

Mobilization Schedule

In accordance with the Particular Conditions, Sub-Clause 4.1, the Contractor shall not carry out mobilization to Site unless the Engineer gives consent.

Construction Schedule

Form EQU: Equipment

The Bidder shall provide adequate information to demonstrate clearly that it has the capability to meet the requirements for the key equipment listed in Section V, Evaluation and Qualification Criteria. A separate Form shall be prepared for each item of equipment listed, or for alternative equipment proposed by the Bidder.

| Item of equipment |  |  |
| --- | --- | --- |
| Equipment information | Name of manufacturer | Model and power rating |
|  | Capacity | Year of manufacture |
| Current status | Current location |  |
|  | Details of current commitments |  |
| Source | Indicate source of the equipment o Owned o Rented o Leased o Specially manufactured |  |

Omit the following information for equipment owned by the Bidder.

| Owner | Name of owner |  |
| --- | --- | --- |
|  | Address of owner |  |
|  | Telephone | Contact name and title |
|  | Fax | Telex |
| Agreements | Details of rental / lease / manufacture agreements specific to the project |  |

SECTION VII

STANDARD BIDDING FORMS

# Section VII: Standard Bidding Forms

Table of Forms

Letter of Bid – Technical Proposal	122

Letter of Bid - Financial Proposal	124

FORM ELI 1	127

Bidder Information Form	127

ELI2	128

Bidder’s JV Members Information Form	128

Financial Situation	135

EXPERIENCE	142

Form of Bid Security	147

Priced Bill of Quantities	149

Schedule of Adjustment Data	151

Form of Affidavit	152

Beneficial Ownership Disclosure Form	154

Letter of Bid – Technical Proposal

INSTRUCTIONS TO BIDDERS: DELETE THIS BOX ONCE YOU HAVE COMPLETED THE DOCUMENT Place this Letter of Bid in the first envelope “TECHNICAL PROPOSAL”. The Bidder must prepare the Letter of Bid on stationery with its letterhead clearly showing the Bidder’s complete name and business address. Note: All italicized text in black font is to help Bidders in preparing this form and Bidders shall delete it from the final document.

Date of this Bid submission: [insert date (as day, month and year) of Bid submission]

IFB No.: [insert number of Bidding process]

Alternative No.: [insert identification No if this is a Bid for an alternative]

To: [insert complete name of Employer]

We, the undersigned Bidder, hereby submit the first part of our Bid, the Technical Proposal

In submitting our Bid we make the following declarations:

- No reservations: We have examined and have no reservations to the bidding document, including addenda issued in accordance with Instructions to Bidders (ITB 9);

- Eligibility: We meet the eligibility requirements and have no conflict of interest in accordance with ITB 3;

- Bid/Proposal-Securing Declaration: We have not been suspended nor declared ineligible by the Employer based on execution of a Bid Securing Declaration or Bid Securing Declaration in the Employer’s country in accordance with ITB 3;

- Conformity: We hereby confirm that we have ensured sufficiency of our bid by employing all satisfactory measures, and we offer to execute works in conformity with the bidding document and in accordance with the works requirements: [insert a brief description of the WORKS]. We also unconditionally confirm the provisions that have been defined in respect of inclusion and usage of Provisional Sum(s);

- Bid Validity Period: Our Bid shall be valid for the period specified in BDS 18.1 (as amended, if applicable) from the date fixed for the Bid submission deadline specified in BDS 24.1 (as amended, if applicable), and it shall remain binding upon us, and may be accepted at any time before the expiration of that period;

- Performance Security: If our Bid is accepted, we commit to obtain a performance security in accordance with the bidding document;

- One Bid per Bidder: We are not submitting any other Bid(s) as an individual Bidder, and we are not participating in any other bid(s) as a Joint Venture member or as a subcontractor, and meet the requirements, other than Alternative Bids submitted in accordance with ITB 20;

- Suspension and Debarment: We, along with any of our subcontractors, suppliers, consultants, manufacturers, or service providers for any part of the contract, are not subject to, and not controlled by any entity or individual that is subject to, a temporary suspension or a debarment imposed by the Employer. Further, we are not ineligible under Pakistan laws;

- State-owned enterprise or institution: [select the appropriate option and delete the other] [We are not a state-owned enterprise or institution] / [We are a state-owned enterprise or institution ];

- Binding Contract: We understand that this Bid, together with your written acceptance thereof included in your Letter of Acceptance, shall constitute a binding contract between us, until a formal contract is prepared and executed;

- Not Bound to Accept: We understand that you are not bound to accept the Most Advantageous Bid or any other Bid that you may receive; and

- Fraud and Corruption: We hereby certify that we have taken steps to ensure that no person acting for us, or on our behalf, engages in any type of Fraud and Corruption.

Name of the Bidder: *[insert complete name of Bidder]

Country of Origin of the Bidder: [insert country of origin, in case of JV country of origin of lead member]

Name of the person duly authorized to sign the Bid on behalf of the Bidder: ** [insert complete name of person duly authorized to sign the Bid]

Title of the person signing the Bid: [insert complete title of the person signing the Bid]

Signature of the person named above: [insert signature of person whose name and capacity are shown above]

Date signed [insert date of signing] day of [insert month], [insert year]

*: In the case of the Bid submitted by a Joint Venture specify the name of the Joint Venture as Bidder.

**: Person signing the Bid shall have the power of attorney given by the Bidder. The power of attorney shall be attached with the Bid Schedules.

Letter of Bid - Financial Proposal

INSTRUCTIONS TO BIDDERS: DELETE THIS BOX ONCE YOU HAVE COMPLETED THE DOCUMENT Place this Letter of Bid - Financial Proposal in the second envelope marked “FINANCIAL PROPOSAL”. The Bidder must prepare the Letter of Bid - Financial Proposal on stationery with its letterhead clearly showing the Bidder’s complete name and business address. Note: All italicized text is to help Bidders in preparing this form.

Date of this Bid submission: [insert date (as day, month and year) of Bid submission]

Invitation for Bid No.: [insert number of bidding process]

Name of Project.: [insert identification]

Alternative No.: [insert identification No if this is a Bid for an alternative]

To: [insert complete name of Employer]

We, the undersigned Bidder, hereby submit the second part of our Bid, the Financial Proposal

In submitting our Financial Proposal we make the following additional declarations:

- Bid Validity Period: Our Bid shall be valid for the period specified in BDS 18.1 (as amended, if applicable) from the date fixed for the bid submission deadline specified in BDS 24.1 (as amended, if applicable), and it shall remain binding upon us and may be accepted at any time before the expiration of that period;

- Total Price: The total price of our Bid, inclusive of all applicable indirect taxes, is:

In case of only one lot, the total price of the Bid, inclusive of all applicable indirect taxes, is [insert the total price of the bid in words and figures, indicating the various amounts and the respective currencies];

In case of multiple lots, the total price of each lot, inclusive of all applicable indirect taxes, is [insert the total price of each lot in words and figures, indicating the various amounts and the respective currencies];

In case of multiple lots, total price of all lots (sum of all lots), inclusive of all applicable indirect taxes, is [insert the total price of all lots in words and figures, indicating the various amounts and the respective currencies];

- Commissions, gratuities and fees: We have paid, or will pay the following commissions, gratuities, or fees with respect to the bidding process or execution of the Contract: [insert complete name of each Recipient, its full address, the reason for which each commission or gratuity was paid and the amount and currency of each such commission or gratuity].

| Name of Recipient | Address | Reason | Amount |
| --- | --- | --- | --- |

(If none has been paid or is to be paid, indicate “none.”)

- Binding Contract: We understand that this Bid, together with your written acceptance thereof included in your Letter of Acceptance, shall constitute a binding contract between us, until a formal contract is prepared and executed.

Name of the Bidder:*[insert complete name of the Bidder]

Name of the person duly authorized to sign the Bid on behalf of the Bidder: ** [insert complete name of person duly authorized to sign the Bid]

Title of the person signing the Bid: [insert complete title of the person signing the Bid]

Signature of the person named above: [insert signature of person whose name and capacity are shown above]

Date signed [insert date of signing] day of [insert month], [insert year]

*: In the case of the Bid submitted by a Joint Venture specify the name of the Joint Venture as Bidder.

**: Person signing the Bid shall have the power of attorney given by the Bidder. The power of attorney shall be attached with the Bid Schedules.

Bidder’s Qualification

To establish its qualifications to perform the contract in accordance with Section V (Evaluation and Qualification Criteria) the Bidder shall provide the information requested in the corresponding Information Sheets/Forms included hereunder

The information provided in the Forms shall be substantiated with valid documentary evidence otherwise the requirement will not be considered as complied.

FORM ELI 1

Bidder Information Form

[The Bidder shall fill in this Form in accordance with the instructions indicated below. No alterations to its format shall be permitted and no substitutions shall be accepted.]

Date: [insert date (as day, month and year) of Bid submission]

IFB No.: [insert number of Bidding process]

Alternative No.: [insert identification No if this is a Bid for an alternative]

Page ________ of_ ______ pages

1. Bidder’s Name [insert Bidder’s legal name]

2. In case of JV, legal name of each member : [insert legal name of each member in JV]

3. Bidder’s actual or intended country of registration: [insert actual or intended country of registration]

4. Bidder’s year of registration: [insert Bidder’s year of registration]

5. Bidder’s Address in country of registration: [insert Bidder’s legal address in country of registration]

6. Bidder’s Authorized Representative Information Name: [insert Authorized Representative’s name] Address: [insert Authorized Representative’s Address] Telephone/Fax numbers: [insert Authorized Representative’s telephone/fax numbers] Email Address: [insert Authorized Representative’s email address]

7. Attached are copies of original documents of [check the box(es) of the attached original documents] Articles of Incorporation (or equivalent documents of constitution or association), and/or documents of registration of the legal entity named above. In case of JV, JV agreement, in accordance with ITB 3.3. Establishing that the Bidder is not under the supervision of the Employer[ in case of subsidiaries] 8. Included are the organizational chart, a list of Board of Directors, and the beneficial ownership.

ELI2

Bidder’s JV Members Information Form

[The Bidder shall fill in this Form in accordance with the instructions indicated below. The following table shall be filled in for the Bidder and for each member of a Joint Venture].

Date: [insert date (as day, month and year) of Bid submission]

IFB No.: [insert number of IFB process]

Alternative No.: [insert identification No if this is a Bid for an alternative]

Page ________ of_ ______ pages

1. Bidder’s Name: [insert Bidder’s legal name]

2. Bidder’s JV Member’s name: [insert JV’s Member legal name]

3. Bidder’s JV Member’s country of registration: [insert JV’s Member country of registration]

4. Bidder’s JV Member’s year of registration: [insert JV’s Member year of registration]

5. Bidder’s JV Member’s legal address in country of registration: [insert JV’s Member legal address in country of registration]

6. Bidder’s JV Member’s authorized representative information Name: [insert name of JV’s Member authorized representative] Address: [insert address of JV’s Member authorized representative] Telephone/Fax numbers: [insert telephone/fax numbers of JV’s Member authorized representative] Email Address: [insert email address of JV’s Member authorized representative]

7. Attached are copies of original documents of [check the box(es) of the attached original documents] Articles of Incorporation (or equivalent documents of constitution or association), and/or registration documents of the legal entity named above. 8. Included are the organizational chart, a list of Board of Directors, and the beneficial ownership.

Personnel

Form PER -1

Contractor’s Representative and Key Personnel

Schedule

Bidders should provide the names and details of the suitably qualified Contractor’s Representative and Key Personnel to perform the Contract. The data on their experience should be supplied using the Form PER-2 below for each candidate.

Contractor’ Representative and Key Personnel

| 1. | Title of position: |  |
| --- | --- | --- |
|  | Name of candidate: |  |
|  | Duration of appointment: | [insert the whole period (start and end dates) for which this position will be engaged] |
|  | Time commitment: for this position: | [insert the number of days/week/months/ that has been scheduled for this position] |
|  | Expected time schedule for this position: | [insert the expected time schedule for this position (e.g. attach high level Gantt chart] |
| 2. | Title of position: |  |
|  | Name of candidate: |  |
|  | Duration of appointment: | [insert the whole period (start and end dates) for which this position will be engaged] |
|  | Time commitment: for this position: | [insert the number of days/week/months/ that has been scheduled for this position] |
|  | Expected time schedule for this position: | [insert the expected time schedule for this position (e.g. attach high level Gantt chart] |
| 3. | Title of position: |  |
|  | Name of candidate: |  |
|  | Duration of appointment: | [insert the whole period (start and end dates) for which this position will be engaged] |
|  | Time commitment: for this position: | [insert the number of days/week/months/ that has been scheduled for this position] |
|  | Expected time schedule for this position: | [insert the expected time schedule for this position (e.g. attach high level Gantt chart] |
| 4. | Title of position: |  |
|  | Name of candidate: |  |
|  | Duration of appointment: | [insert the whole period (start and end dates) for which this position will be engaged] |
|  | Time commitment: for this position: | [insert the number of days/week/months/ that has been scheduled for this position] |
|  | Expected time schedule for this position: | [insert the expected time schedule for this position (e.g. attach high level Gantt chart] |
| 5. | Title of position: |  |
|  | Name of candidate |  |
|  | Duration of appointment: | [insert the whole period (start and end dates) for which this position will be engaged] |
|  | Time commitment: for this position: | [insert the number of days/week/months/ that has been scheduled for this position] |
|  | Expected time schedule for this position: | [insert the expected time schedule for this position (e.g. attach high level Gantt chart] |
| 6. | Title of position: [insert title] |  |
|  | Name of candidate |  |
|  | Duration of appointment: | [insert the whole period (start and end dates) for which this position will be engaged] |
|  | Time commitment: for this position: | [insert the number of days/week/months/ that has been scheduled for this position] |
|  | Expected time schedule for this position: | [insert the expected time schedule for this position (e.g. attach high level Gantt chart] |

Form PER-2:

Resume and Declaration

Contractor’s Representative and Key Personnel

Name of Bidder

| Position [#1]: [title of position from Form PER-1] |  |  |
| --- | --- | --- |
| Personnel information | Name: | Date of birth: |
|  | Address: | E-mail: |
|  | Professional qualifications: |  |
|  | Academic qualifications: |  |
|  | Language proficiency:[language and levels of speaking, reading and writing skills] |  |
| details |  |  |
|  | Address of Employer: |  |
|  | Telephone: | Contact (manager / personnel officer): |
|  | Fax: |  |
|  | Job title: | Years with present Employer: |

Summarize professional experience in reverse chronological order. Indicate particular technical and managerial experience relevant to the project.

| Project | Role | Duration of involvement | Relevant experience |
| --- | --- | --- | --- |
| [main project details] | [role and responsibilities on the project] | [time in role] | [describe the experience relevant to this position] |

Declaration

I, the undersigned [ insert either “Contractor’s Representative” or “Key Personnel” as applicable] , certify that to the best of my knowledge and belief, the information contained in this Form PER-2 correctly describes myself, my qualifications and my experience.

I confirm that I am available as certified in the following table and throughout the expected time schedule for this position as provided in the Bid:

| Commitment | Details |
| --- | --- |
| Commitment to duration of contract: | [insert period (start and end dates) for which this Contractor’s Representative or Key Personnel is available to work on this contract] |
| Time commitment: | [insert period (start and end dates) for which this Contractor’s Representative or Key Personnel is available to work on this contract] |

I understand that any misrepresentation or omission in this Form may:

- be taken into consideration during Bid evaluation;

- result in my disqualification from participating in the Bid;

- result in my dismissal from the contract.

Name of Contractor’s Representative or Key Personnel: [insert name]				Signature: __________________________________________________________

Date: (day month year): _______________________________________________

Countersignature of authorized representative of the Bidder:

Signature: ________________________________________________________

Date: (day month year): ____________________________
Form CON – 2

Historical Contract Non-Performance, Pending Litigation and Litigation History

Bidder’s Name: ________________
Date: ______________________
JV Member Name_________________________
IFB No. and title: ___________________________
Page _______________of ______________pages

| Non-Performed Contracts in accordance with Qualification Criteria |  |  |  |
| --- | --- | --- | --- |
| Contract non-performance did not occur January [insert year] Contract(s) not performed since [insert year] |  |  |  |
| Year | Non- performed portion of contract | Contract Identification | Total Contract Amount (current value, currency, exchange rate and PKR equivalent) |
|  |  | Contract Identification: Name of Employer(PA): Address of PA: Reason(s) for nonperformance: |  |
| Pending Litigation, in accordance with Qualification Criteria |  |  |  |
| No pending litigation |  |  |  |
| Pending litigation |  |  |  |

| Year of dispute | Amount in dispute (currency) | Contract Identification | Total Contract Amount |
| --- | --- | --- | --- |
|  |  | Contract Identification: _________ Name of PA: ____________ Address of PA: __________ Matter in dispute: ______________ Party who initiated the dispute: ____ Status of dispute: ___________ |  |
| Litigation History in accordance with Section V, Evaluation and Qualification Criteria of the Prequalification document |  |  |  |
|  No Litigation History  Litigation History |  |  |  |
| Year of award | Outcome as percentage of Net Worth | Contract Identification | Total Contract Amount (currency), PKR Equivalent (exchange rate) |
| [insert year] | [insert percentage] | Contract Identification: [indicate complete contract name, number, and any other identification] Name of PA: [insert full name] Address of PA: [insert street/city/country] Matter in dispute: [indicate main issues in dispute] Party who initiated the dispute: [indicate “Employer” or “Contractor”] Reason(s) for Litigation and award decision [indicate main reason(s)] | [insert amount] |

Financial Situation

Form FIN – 3.1:

Financial Situation and Performance

Bidder’s Name: ________________
Date: ______________________
JV Member’s Name_________________________
IFB No. and title: ___________________________
Page _______________of ______________pages

1. Financial data

| Type of Financial information in (currency) | Historic information for previous _________years, ______________ (amount in currency, currency, exchange rate*, PKR equivalent) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Year 1 | Year 2 | Year 3 | Year4 | Year 5 |
| Statement of Financial Position (Information from Balance Sheet) |  |  |  |  |  |
| Total Assets (TA) |  |  |  |  |  |
| Total Liabilities (TL) |  |  |  |  |  |
| Total Equity/Net Worth (NW) |  |  |  |  |  |
| Current Assets (CA) |  |  |  |  |  |
| Current Liabilities (CL) |  |  |  |  |  |
| Working Capital (WC) |  |  |  |  |  |
| Information from Income Statement |  |  |  |  |  |
| Total Revenue (TR) |  |  |  |  |  |
| Profits Before Taxes (PBT) |  |  |  |  |  |
| Cash Flow Information |  |  |  |  |  |
| Cash Flow from Operating Activities |  |  |  |  |  |

*Exchange rate used for calculations should be two days prior to the bids submission.

2. Sources of Finance

Specify sources of finance to meet the cash flow requirements on works currently in progress and for future contract commitments.

| No. | Source of finance | Amount (PKR) |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

2. Financial documents

The Bidder and its parties shall provide copies of financial statements for ___________years pursuant to Evaluation and Qualifications Criteria, Sub-factor 3.1. The financial statements shall:

(a) 	reflect the financial situation of the Bidder or in case of JV member, and not an affiliated entity  (such as parent company or group member).

(b)	be independently audited or certified in accordance with local legislation.

(c)	be complete, including all notes to the financial statements.

(d)	correspond to accounting periods already completed and audited.

Attached are copies of financial statements  for the ____________years required above; and complying with the requirements

Form FIN – 3.2:

Average Annual Construction Turnover

Bidder’s Name: ________________
Date: ______________________
JV Member’s Name_________________________
IFB No. and title: ___________________________
Page _______________of ______________pages

|  |  | Annual turnover data (construction only) |  |  |
| --- | --- | --- | --- | --- |
| Year | Amount Currency |  | Exchange rate ** | PKR equivalent |
| [indicate year] | [insert amount and indicate currency] |  |  |  |
| Average Annual Construction Turnover * |  |  |  |  |

* See Section V, Evaluation and Qualification Criteria.

* * Exchange rate used for calculations should be two days prior to the bids submission

Form FIN – 3.3:

Financial Resources

Specify proposed sources of financing, such as liquid assets, unencumbered real assets, lines of credit, and other financial means, net of current commitments, available to meet the total construction cash flow demands of the subject contract or contracts as specified in Section – V (Evaluation and Qualification Criteria)

| Financial Resources |  |  |
| --- | --- | --- |
| No. | Source of financing | Amount |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Form FIN – 3.4:

Current Contract Commitments / Works in Progress

Bidders and each member to a JV should provide information on their current commitments on all contracts that have been awarded, or for which a letter of intent or acceptance has been received, or for contracts approaching completion, but for which an unqualified, full completion certificate has yet to be issued.

##### Current Contract Commitments

| No. | Name of Contract | Employer’s Contact Address, Tel, Fax | Value of Outstanding Work [PKR Equivalent] | Estimated Completion Date | Average Monthly Invoicing Over Last Six Months [PKR/month)] |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

Form FIN - 5:

Self-Assessment Tool for Bidder’s Compliance to Financial Resources (Criterion 2.1 of Section V)

This form requires the same information submitted in Forms FIN – 3.3 and FIN -3.4. All conditions of “Available Financial Resources Net of CCC ≥ Requirement for the Subject Contract” must be satisfied to qualify.

Form FIN - 5A: For Single Entities

| For Single Entities: (A) | Total Available Financial Resources from FIN – 3.3 (B) | Total Monthly Financial Requirement for Current Contract Commitments (CCC) from FIN – 3.4 (C) | Available Financial Resources Net of CCC D = (B - C) | Requirement for the Subject Contract (E) | Results: Yes or No [D must be greater than or equal to E] (F) |
| --- | --- | --- | --- | --- | --- |
| _________ (Name of Bidder) |  |  |  | . . . . . . . . . . ._ |  |

Form FIN - 5B: For Joint Ventures

| For Joint Ventures: (A) | Total Available Financial Resources from FIN – 3.3 (B) | Total Monthly Financial Requirement for Current Contract Commitments (CCC) from FIN – 3.4 (C) | Available Financial Resources Net of CCC D = (B - C) | Requirement for the Subject Contract (E) | Results: Yes or No [D must be greater than or equal to E] (F) |
| --- | --- | --- | --- | --- | --- |
| One Partner: |  |  |  |  |  |
| ____________ (Name of Partner) |  |  |  | . . . . . . . . . . . |  |
| Each Partner: |  |  |  |  |  |
| _____________ (Name of Partner 1) |  |  |  | . . . . . . . . . . . |  |
| _____________ (Name of Partner 2) |  |  |  | . . . . . . . . . . . |  |
| _____________ (Name of Partner 3) |  |  |  | . . . . . . . . . . . |  |
| All partners combined | ∑ D = Sum of available financial resources net of current contract commitments for all partners |  | ∑ D = _______ | . . . . . . . . . . . |  |

- Note -

Form FIN – 5 is made available for use by the bidder as a self-assessment tool, and by the employer as an evaluation work sheet, to determine compliance with the financial resources requirement as stated in 3.1 (ii). Failure to submit Form FIN - 5 by the Bidder shall not lead to bid rejection.

EXPERIENCE

Form EXP - 4.1

General Construction Experience

Bidder’s Name: ________________
Date: ______________________
JV Member’s Name_________________________
IFB No. and title: ___________________________
Page _______________of ______________pages

| Starting Year | Ending Year | Contract Identification | Role of Bidder |
| --- | --- | --- | --- |
|  |  | Contract name: ____________________ Brief Description of the Works performed by the Bidder: _____________________________ Amount of contract: ___________________ Name of PA: ____________________ Address: _____________________________ |  |
|  |  | Contract name: _________________________ Brief Description of the Works performed by the Bidder: _____________________________ Amount of contract: ___________________ Name of PA: ___________________ Address: _________________________ |  |
|  |  | Contract name: ________________________ Brief Description of the Works performed by the Bidder: __________________________ Amount of contract: ___________________ Name of PA: ___________________ Address: _________________________ |  |

Form EXP - 4.2(a)

Specific Construction and Contract Management Experience

Bidder’s Name: ________________
Date: ______________________
JV Member’s Name_________________________
IFB No. and title: ___________________________
Page _______________of ______________pages

| Similar Contract No. | Information |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Contract Identification |  |  |  |  |  |
| Award date |  |  |  |  |  |
| Completion date |  |  |  |  |  |
| Role in Contract | Prime Contractor |  | Member in JV | Management Contractor | Sub-contractor |
| Total Contract Amount |  |  |  | PKR equivalent |  |
| If member in a JV or sub-contractor, specify participation in total Contract amount |  |  |  |  |  |
| PA’s Name: |  |  |  |  |  |
| Address: Telephone/fax number E-mail: |  |  |  |  |  |

Form EXP - 4.2(a) (cont.)

Specific Construction and Contract Management Experience (cont.)

| Similar Contract No. | Information |
| --- | --- |
| Description of the similarity in accordance with Sub-Factor 4.2(a) of Section V: |  |
| 1. Amount |  |
| 2. Physical size of required works items |  |
| 3. Complexity |  |
| 4. Methods/Technology |  |
| 5. Construction rate for key activities |  |
| 6. Other Characteristics |  |

Form EXP - 4.2(b)

Construction Experience in Key Activities

Bidder's Name: ________________
Date: ___________________
Bidder's JV Member Name: __________________
Sub-contractor's Name (as per ITB 34.2 and 34.3): ________________
IFB No. and title: _____________________

Page __________________of ________________pages

All Sub-contractors for key activities must complete the information in this form as per ITB 38 and Qualification Criteria and Requirements, Sub-Factor 4.2.

1.	Key Activity No One: ________________________

|  | Information |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Contract Identification |  |  |  |  |  |
| Award date |  |  |  |  |  |
| Completion date |  |  |  |  |  |
| Role in Contract | Prime Contractor | Member in JV |  | Management Contractor | Sub-contractor |
| Total Contract Amount |  |  |  | PKR equivalent |  |
| Quantity (Volume, number or rate of production, as applicable) performed under the contract per year or part of the year | Total quantity in the contract (i) |  | Percentage participation (ii) |  | Actual Quantity Performed (i) x (ii) |
| Year 1 |  |  |  |  |  |
| Year 2 |  |  |  |  |  |
| Year 3 |  |  |  |  |  |
| Year 4 |  |  |  |  |  |
| PA’s Name: |  |  |  |  |  |  |
| Address: Telephone/fax number E-mail: |  |  |  |  |  |  |

2. Activity No. Two

3. …………………

|  | Information |
| --- | --- |
| Description of the key activities in accordance with Section V: |  |

Form of Bid Security

(Bank Guarantee)

[The bank shall fill in this Bank Guarantee Form in accordance with the instructions indicated.]

[Guarantor letterhead or SWIFT identifier code]

Beneficiary: [Purchaser to insert its name and address]

No.: [Purchaser to insert reference number for the Request for Bids]

Alternative No.: [Insert identification No if this is a Bid for an alternative]

Date: [Insert date of issue]

BID GUARANTEE No.: [Insert guarantee reference number]

Guarantor: [Insert name and address of place of issue, unless indicated in the letterhead]

We have been informed that ______ [insert name of the Bidder, which in the case of a joint venture shall be the name of the joint venture (whether legally constituted or prospective) or the names of all members thereof] (hereinafter called "the Applicant") has submitted or will submit to the Beneficiary its Bid (hereinafter called "the Bid") for the execution of ________________ under Invitation for Bids No. __________ _ (“the IFB”).

Furthermore, we understand that, according to the Beneficiary’s conditions, Bids must be supported by a Bid guarantee.

At the request of the Applicant, we, as Guarantor, hereby irrevocably undertake to pay the Beneficiary any sum or sums not exceeding in total an amount of ___________ (____________) upon receipt by us of the Beneficiary’s complying demand, supported by the Beneficiary’s statement, whether in the demand itself or a separate signed document accompanying or identifying the demand, stating that either the Applicant:

(a) 	has withdrawn its Bid during the period of Bid validity set forth in the Applicant’s Letter of Bid (“the Bid Validity Period”), or any extension thereto provided by the Applicant; or

(b) 	having been notified of the acceptance of its Bid by the Beneficiary during the Bid Validity Period or any extension thereto provided by the Applicant, (i) has failed to sign the contract agreement, or (ii) has failed to furnish the performance security, in accordance with the Instructions to Bidders (“ITB”) of the Beneficiary’s bidding document.

This guarantee will expire: (a) if the Applicant is the successful Bidder, upon our receipt of copies of the Contract agreement signed by the Applicant and the performance security issued to the Beneficiary in relation to such Contract agreement; or (b) if the Applicant is not the successful Bidder, upon the earlier of (i) our receipt of a copy of the Beneficiary’s notification to the Applicant of the results of the Bidding process; or (ii) twenty-eight days after the end of the Bid Validity Period.

Consequently, any demand for payment under this guarantee must be received by us at the office indicated above on or before that date.

_____________________________

[Signature(s)]

Note: All italicized text is for use in preparing this form and shall be deleted from the final product.

Priced Bill of Quantities

[to be printed on letterhead of the bidder]

We confirm that the rates and prices bid in the priced Bill of Quantities, except insofar as it is otherwise provided under the Contract, include all Constructional Plant, labour, supervision, materials, erection, maintenance, insurance, profit, taxes (including GST/PRA), and duties, together with all general risks, liabilities, and obligations set out or implied in the Contract.

A rate or price is entered against each item in the priced Bill of Quantities, whether quantities are stated or not.  The cost of Items against which we have failed to enter a rate, or price shall be deemed to be covered by other rates and prices entered in the Bill of Quantities. Moreover, we confirm, warrant and agree that the Employer shall be at liberty, if we have missed filling the cost/rate against any number of item or component in the bill of quantities, to adjust the overall cost of the bid based on the average price of the item or component as quoted in other substantially responsive Bids. If the price of the item or component cannot be derived from the price of other substantially responsive Bids, the Employer shall use its best estimate.

SUMMARY OF COST

| Bill # | Description | Cost (Pak. Rs.) |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
|  | Sub-Total = |  |
|  | Cost of GST/PRA = |  |
|  | Grand Total = |  |

SUMMARY OF COST

| Bill # | Description | Cost (Pak. Rs.) |
| --- | --- | --- |
| 1.1 |  |  |
| 1.2 |  |  |
| 1.3 |  |  |
| 1.4 |  |  |
| 1.5 |  |  |
|  | Total = |  |

Bill No. 1.1 Title: xx

| Item. # | Ref. | Description | Unit of Measurement | Quantity | Unit Rate (Pak. Rs.) | Total Amount (Pak. Rs.) |
| --- | --- | --- | --- | --- | --- | --- |
| Total = |  |  |  |  |  |  |

Bill No. 1.2 Title: xx

| Item. # | Ref. | Description | Unit of Measurement | Quantity | Unit Rate (Pak. Rs.) | Total Amount (Pak. Rs.) |
| --- | --- | --- | --- | --- | --- | --- |
| Total = |  |  |  |  |  |  |

Bill No. 1.3 Title: xx

| Item. # | Ref. | Description | Unit of Measurement | Quantity | Unit Rate (Pak. Rs.) | Total Amount (Pak. Rs.) |
| --- | --- | --- | --- | --- | --- | --- |
| Total = |  |  |  |  |  |  |

Schedule of Adjustment Data

| Element Number | Element Description | Bidder’s Amount [PKR] | Bidder’s Proposed Weighting for Coefficients* |
| --- | --- | --- | --- |

| i. ii. iii. iv. v. | Non-adjustable Cement (OPC) Steel (M.S. bars [deformed, 40/60 grade] High Speed Diesel Labour (Un-skilled/skilled) |  | a: (0.54) b: (0.10) c: (0.18) d: (0.07) e: (0.11) |
| --- | --- | --- | --- |
| Total |  |  | 1.00 |

* The Bidder shall enter a value within the range such that the total weighting shall be equal to 1.00.

The base and current rates of each cost element shall be taken from the Federal Bureau of Statistics (FBS), Statistical Division Government of Pakistan, Statutory notifications and official price from public sector organizations, where available, for Price Variation Adjustment for the area(s) prescribed in the list, nearest to the location of the Works. Whereas the source for prices of High Speed Diesel (HSD) shall be either Statistical Bulletins or Pakistan State Oil (PSO) or Oil and Gas Regulatory Authority (OGRA). The base values shall be 28 days prior to the submission of Bid.

Note:

- Cement (OPC) [Bags] shall represent all kinds of Cement.

- Steel (M.S. bars deformed, 40/60 grade) shall represent all kinds of steel reinforcing bars and structural steel and all kind of MS angle/sheet and sections.

- High Speed Diesel [Litre] shall represent all kinds of fuels and lubricants.

- Labour (Un-skilled/skilled) shall represent all kinds of labour.

- HDPE/PVC pipe shall represent all kind of pressure (PN)/class and size of pipes

- RCC pipes shall represent all kinds/class and size of pipes

- Bricks shall represent all kind of bricks, bricks ballast and bricks tiles

- Crush shall represent all size and grade of crush.

Form of Affidavit

[This must be printed on judicial/stamp paper having worth of at least Pak. Rs. 100/-. The judicial/stamp paper must be a registered paper with the respective vendor, having necessary entries on it and must also be Notarized. This Affidavit must be signed by the authorized representative of the bidder whose respective authorization is already confirmed as part of the Bid]

Affidavit of:	 [name of authorized representative of the bidder], having CNIC # [enter number], as [mention the designation of authorized representative] on behalf of [name of the bidder], [complete official address]

I, the above-named deponent, do hereby solemnly state on oath that I am an authorized representative of [name of the bidder] and, hereby on behalf of [name of the bidder] declare, agree, understand, warrant and undertake that:

- [name of the bidder] intends to submit a Bid for contraction works & related services, viz., [brief description of works and services] in complete understanding of the bidding document for the said procurement, issued by the Allama Iqbal Open University,

- [name of the bidder] has not been blacklisted or debarred by any federal or provincial or regional Public Procurement Regulatory Authority and/or government ministry/ department/public sector organization, international financial institutions/donors [such as World Bank, Asian Development Bank etc.],

- [name of the bidder] is not engaged in any legal proceedings with and/or against any federal or provincial or regional Public Procurement Regulatory Authority and/or government ministry/department/public sector organization including Allama Iqbal Open University, international financial institutions/donors [such as World Bank, Asian Development Bank etc.]

- [name of the bidder] has no direct and/or indirect conflict of interest with any person, employee, staff and any member of the Procurement Evaluation Committee(s) of the Allama Iqbal Open University and/or the procurement activity for which [name of the bidder] is submitting bid,

- [name of the bidder] is submitting his bid conscious of market inflation and foreign currency exchange rates risks and shall comply with the offered rates during bid validity period, and if awarded a contract shall honour the offered rates without indulging into any argument and/or asking for price adjustment, thus bearing all the related risks,

- [name of the bidder] irrevocably and unconditionally undertake and warrant that in case [name of the bidder] defaults in honouring his offered/committed rates before or after, signing of contract agreement and/or supply order, the University shall be at liberty to forfeit the earnest money and/or  performance security and/or take any punitive action including blacklisting for desired period, and [name of the bidder] shall not challenge that decision in any court of law,

- [name of the bidder] accepts full responsibility and strict liability for making any false declaration, not making full disclosure, misrepresenting facts or taking any action likely to defeat the purpose of this declaration, representation and warranty.

DEPONENT

Verification:

Verified on oath at Islamabad on this -- day of the month of -------------- of 20-- that the contents of the foregoing Affidavit are true and correct to the best of my knowledge and belief and that nothing thereof has been concealed from Allama Iqbal Open University.

DEPONENT

INSTRUCTIONS TO BIDDERS: DELETE THIS BOX ONCE YOU HAVE COMPLETED THE FORM

This Beneficial Ownership Disclosure Form (“Form”) is to be completed by the successful Bidder.  In case of joint venture, the Bidder must submit a separate Form for each member.  The beneficial ownership information to be submitted in this Form shall be current as of the date of its submission.

For the purposes of this Form, a Beneficial Owner of a Bidder is any natural person who ultimately owns or controls the Bidder by meeting one or more of the following conditions:

- directly or indirectly holding 25% or more of the shares

- directly or indirectly holding 25% or more of the voting rights

- directly or indirectly having the right to appoint a majority of the board of directors or equivalent governing body of the Bidder

Beneficial Ownership Disclosure Form

IFB No.: [insert number of IFB process]

Invitation for Bid Title: [insert identification]

To: [insert complete name of Employer]

In response to your request in the Letter of Acceptance dated [insert date of letter of Acceptance] to furnish additional information on beneficial ownership: [select one option as applicable and delete the options that are not applicable]

(i) we hereby provide the following beneficial ownership information.

Details of beneficial ownership

| Identity of Beneficial Owner | Directly or indirectly holding 25% or more of the shares (Yes / No) | Directly or indirectly holding 25 % or more of the Voting Rights (Yes / No) | Directly or indirectly having the right to appoint a majority of the board of the directors or an equivalent governing body of the Bidder (Yes / No) |
| --- | --- | --- | --- |
| [include full name (last, middle, first), nationality, country of residence] |  |  |  |

OR

(ii) We declare that there is no Beneficial Owner meeting one or more of the following conditions:

- directly or indirectly holding 25% or more of the shares

- directly or indirectly holding 25% or more of the voting rights

- directly or indirectly having the right to appoint a majority of the board of directors or equivalent governing body of the Bidder

OR

(iii) We declare that we are unable to identify any Beneficial Owner meeting one or more of the following conditions. [If this option is selected, the Bidder shall provide explanation on why it is unable to identify any Beneficial Owner]

- directly or indirectly holding 25% or more of the shares

- directly or indirectly holding 25% or more of the voting rights

- directly or indirectly having the right to appoint a majority of the board of directors or equivalent governing body of the Bidder]”

Name of the Bidder:*[insert complete name of the Bidder]_________

Name of the person duly authorized to sign the Bid on behalf of the Bidder:**[insert complete name of person duly authorized to sign the Bid]___________

Title of the person signing the Bid: [insert complete title of the person signing the Bid]______

Signature of the person named above: [insert signature of person whose name and capacity are shown above]_____

Date signed [insert date of signing] day of [insert month], [insert year]____

* In the case of the Bid submitted by a Joint Venture specify the name of the Joint Venture as Bidder. In the event that the Bidder is a joint venture, each reference to “Bidder” in the Beneficial Ownership Disclosure Form (including this Introduction thereto) shall be read to refer to the joint venture member.

** Person signing the Bid shall have the power of attorney given by the Bidder. The power of attorney shall be attached with the Bid Schedules.

If applicable